3GPP TSG-RAN WG2 Meeting #102	R2-1806822

# Busan, Republic of Korea, 21st – 25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0102 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on BFR for SCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It has been agreed that BFR on SCell should be supported, by using a BFR MAC CE to report BFR on SCell that also indicates the selected candidate beam. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following sections have been changed: - Section 5.17: Beam failure recovery for an SCell is reported using a BFR MAC CE. MAC considers a BFR procedure for SCell successful if it receives a PDCCH transmission on the SCell addressed to its C-RNTI. - Section 6.1.3: description of the BFR MAC CE. BFR MAC CE is 1 octet composed of 4 bits that indicate the selected beam ID and 4 reserved bits - Section 6.2.1 – added new LCID value for BFR reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | BFR reporting for SCell is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17, 6.1.3, 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modification 1>

## 5.17	Beam Failure Detection and Recovery procedure

The MAC entity may be configured by RRC with a beam failure recovery procedure which is used for indicating to the serving gNB of a new SSB or CSI-RS when beam failure is detected on the serving SSB(s)/CSI-RS(s). Beam failure is detected by counting beam failure instance indication from the lower layers to the MAC entity.

RRC configures the following parameters in the BeamFailureRecoveryConfig for the Beam Failure Detection and Recovery procedure:

-	beamFailureInstanceMaxCount for the beam failure detection;

-	beamFailureDetectionTimer for the beam failure detection;

-	beamFailureCandidateBeamThreshold: an RSRP threshold for the beam failure recovery;

-	preamblePowerRampingStep: preamblePowerRampingStep for the beam failure recovery;

-	preambleReceivedTargetPower: preambleReceivedTargetPower for the beam failure recovery;

-	preambleTxMax: preambleTxMax for the beam failure recovery;

-	ra-ResponseWindow: the time window to monitor response(s) for the beam failure recovery using contention-free Random Access Preamble.

The following UE variables are used for the beam failure detection procedure:

-	BFI_COUNTER: counter for beam failure instance indication which is initially set to 0.

The MAC entity shall for each cell configured with beam failure recovery:

1>	if beam failure instance indication has been received from lower layers:

2>	start or restart the beamFailureDetectionTimer;

2>	increment BFI_COUNTER by 1;

2>	if BFI_COUNTER = beamFailureInstanceMaxCount + 1:

3>	if this cell is the SpCell:

4> initiate a Random Access procedure (see subclause 5.1) on the SpCell by applying the parameters configured in BeamFailureRecoveryConfig.

3> else:

4> instruct the Multiplexing and Assembly procedure to generate the BFR MAC CE;

1>	if the beamFailureDetectionTimer expires:

2>	set BFI_COUNTER to 0.

1>	if the cell on which beam failure is detected is the SpCell:

2> if Random Access procedure is successfully completed (see subclause 5.1):

3> 	consider the Beam Failure Recovery procedure successfully completed.

1>	else:

2>	if a PDCCH transmission on this SCell is addressed to the C-RNTI:

3>	consider the Beam Failure Recovery procedure successfully completed.

<End of modification 1>

<Start of modification 2>

### 6.1.3	MAC Control Elements (CEs)

#### 6.1.3.xx	BFR MAC CE

The MAC entity reports beam failure for an SCell using a BFR MAC CE.

The BFR MAC control element is identified by MAC PDU subheader with LCID as specified in table 6.2.1-1.  This control element has a fixed size of a single octet defined as follows (figure 6.1.3.xx-1):

-	R: reserved bit, set to "0";

-	Selected Candidate Beam Identity  (SCBI): this field indicate the selected beam identity from candidateBeamRSList of the corresponding SCell.

Figure 6.1.3.xx-1: BFR MAC CE

### 6.2.1	MAC header for DL-SCH, UL-SCH and MCH

Table 6.2.1-2 Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 | Identity of the logical channel |
| 01011 | CCCH |
| 01100 | CCCH |
| 01101-10010 | Reserved |
| 10011 | BFR |
| 10100 | Recommended bit rate query |
| 10101 | SPS confirmation |
| 10110 | Truncated Sidelink BSR |
| 10111 | Sidelink BSR |
| 11000 | Dual Connectivity Power Headroom Report |
| 11001 | Extended Power Headroom Report |
| 11010 | Power Headroom Report |
| 11011 | C-RNTI |
| 11100 | Truncated BSR |
| 11101 | Short BSR |
| 11110 | Long BSR |
| 11111 | Padding |

<End of modification 2>