3GPP TSG-RAN2 Meeting #102	R2-1806894

# Busan, Korea (Republic Of), 21st May 2018 - 25th May 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | XXXX | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE power class for NR/EN-DC and update of L1/RF capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (If agreed, this draft CR is supposed to be merged into the rapporteur's CR) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 38.331 v15.1.0 does not implement some of L1/RF features due to lack of information. RAN2 has received further inputs from RAN1/4 to implement some of the remaining features. This pCR is aimed at implementing those remaining features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Capability signalling is introduced for the following L1 features according to the input in R2-1806618 (LS from RAN1). - #2-26; SRS carrier switch; - #2-33a; PDSCH RE-mapping for FR1/2; - #2-35; CSI report framework; - #2-40; Type I multi-panel codebook. - #2-43; Type II codebook with port selection; - #2-46; Downlink PTRS density recommendation; - #2-49; Uplink PTRS density recommendation; - #2-51; CSI-RS for tracking. 2. #2-9; phaseCoherenceUL is removed according to R2-1806618. 3. Capability signalling for UE power class and P-Max for NR CA are introduced according to R2-1806639 (LS from RAN4). 4. Capability signalling on maximum duty cycle for SAR is introduced according to R2-1806637 (LS from RAN4). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot report supported power class and some L1 features not present in 38.331 v15.1.0. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This pCR is drafted based on the baseline CR (R2-1806391). |  |  |  |  |  |  |  |  |

6.3.2	Radio resource control information elements

<< skip unchanged part >>

–	FrequencyInfoUL

The IE FrequencyInfoUL provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL information element

-- ASN1START

-- TAG-FREQUENCY-INFO-UL-START

FrequencyInfoUL ::= 				SEQUENCE {

frequencyBandList					MultiFrequencyBandListNR										OPTIONAL,	-- Cond FDD-OrSUL

absoluteFrequencyPointA				ARFCN-ValueNR													OPTIONAL,	-- Cond FDD-OrSUL

scs-SpecificCarrierList				SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

additionalSpectrumEmission			AdditionalSpectrumEmission										OPTIONAL,	-- Need S

p-Max								P-Max															OPTIONAL,	-- Need S

p-MaxCA								P-Max															OPTIONAL,	-- Need S

frequencyShift7p5khz				ENUMERATED {true}												OPTIONAL,	-- Cond FDD-OrSUL-Optional

...

}

-- TAG-FREQUENCY-INFO-UL-STOP

-- ASN1STOP

FrequencyInfoUL field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A. Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList. Corresponds to L1 parameter 'offset-ref-low-scs-ref-PRB' (see 38.211, section FFS_Section)

additionalSpectrumEmission The additional spectrum emission requirements to be applied by the UE on this uplink. If the field is absent, the UE applies the value FFS_RAN4. (see FFS_section, section FFS_Section)

frequencyBandList List of one or multiple frequency bands to which this carrier(s) belongs. Multiple values are only supported in system information but not when the FrequencyInfoDL is provided in dedicated signalling (HO or S(p)Cell addition).

frequencyShift7p5khz Enable the NR UL transmission with a 7.5KHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max FFS_Definition. Corresponds to parameter FFS_RAN4. (see FFS_Spec, section FFS_Section) If the field is absent, the UE applies the value FFS_RAN4.

p-MaxCA FFS_Definition. Corresponds to parameter FFS_RAN4. (see FFS_Spec, section FFS_Section) If the field is absent, the UE applies the value FFS_RAN4.

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. Corresponds to L1 parameter 'offset-pointA-set' (see 38.211, section FFS_Section)

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL | The field is mandatory present if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL) or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent otherwise (if this FrequencyInfoUL is for an unpaired UL (TDD). |
| FDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL) or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent otherwise. |

Next change

6.3.3	UE capability information elements

–	AccessStratumRelease

The IE AccessStratumRelease indicates the release supported by the UE.

AccessStratumRelease information element

-- ASN1START

-- TAG-ACCESSSTRATUMRELEASE-START

AccessStratumRelease ::= ENUMERATED {

rel15, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ... }

-- TAG-ACCESSSTRATUMRELEASE-STOP

-- ASN1STOP

–	BandCombinationList

The IE BandCombinationList contains a list of NR CA and/or MR-DC band combinations (also including DL only or UL only band).

BandCombinationList information element

-- ASN1START

-- TAG-BANDCOMBINATIONLIST-START

BandCombinationList ::=	SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

BandCombination ::= SEQUENCE {

bandAndDL-ParametersList			BandAndDL-ParametersList,

bandCombinationsUL					BIT STRING (SIZE (1.. maxBandComb)),

bandCombinationParametersList		SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParameters 	OPTIONAL

}

BandAndDL-ParametersList ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandAndDL-Parameters

BandAndDL-Parameters ::= CHOICE {

bandAndDL-ParametersEUTRA	BandAndDL-ParametersEUTRA,

bandAndDL-ParametersNR		BandAndDL-ParametersNR

}

BandCombinationParameters ::= SEQUENCE {

ca-ParametersNR					CA-ParametersNR			OPTIONAL,

mrdc-Parameters					MRDC-Parameters			OPTIONAL

}

CA-ParametersNR ::=	SEQUENCE {

multipleTimingAdvances		ENUMERATED {supported}				OPTIONAL,

simultaneousRxTxInterBandCA			ENUMERATED {supported}		OPTIONAL,

supportedBandwidthCombinationSet	BIT STRING (SIZE (1..32))	OPTIONAL,

srs-CarrierSwitch					CHOICE {

intra-Band							ENUMERATED {us0, us30, us100, us140, us200},

inter-Band							ENUMERATED {us0, us30, us100, us200, us300, us500, us900}

}																OPTIONAL,

ue-PowerClassCA-FR1					INTEGER (1..16)				DEFAULT 3

}

MRDC-Parameters ::=	SEQUENCE {

singleUL-Transmission		ENUMERATED {supported}		OPTIONAL,

ul-SharingEUTRA-NR					ENUMERATED {supported}		OPTIONAL,

ul-SwitchingTimeEUTRA-NR			ENUMERATED {type1, type2}	OPTIONAL,

simultaneousRxTxInterBandENDC		ENUMERATED {supported}		OPTIONAL,

asyncIntraBandENDC					ENUMERATED {supported}		OPTIONAL,

ue-PowerClassMRDC-FR1				INTEGER (1..16)				DEFAULT 3,

basebandProcesingCombinationMRDC 	BasebandProcessingCombinationMRDC

}

BandAndDL-ParametersEUTRA ::= SEQUENCE {

bandEUTRA					FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA	CA-BandwidthClassEUTRA		OPTIONAL,

intraBandContiguousCC-InfoDL-EUTRA-List		SEQUENCE (SIZE (1.. maxNrofServingCellsEUTRA)) OF IntraBandContiguousCC-InfoDL-EUTRA		OPTIONAL

}

BandAndDL-ParametersNR ::= SEQUENCE {

bandNR						FreqBandIndicatorNR,

ca-BandwidthClassDL			CA-BandwidthClassNR			OPTIONAL,

intraBandFreqSeparationDL	FreqSeparationClass					OPTIONAL,

intraBandContiguousCC-InfoDL-List		SEQUENCE (SIZE (1.. maxNrofServingCells)) OF IntraBandContiguousCC-InfoDL		OPTIONAL

}

IntraBandContiguousCC-InfoDL ::=	SEQUENCE {

maxNumberMIMO-LayersPDSCH			MIMO-LayersDL					OPTIONAL

}

IntraBandContiguousCC-InfoDL-EUTRA ::=	SEQUENCE {

mimo-CapabilityDL					ENUMERATED {twoLayers, fourLayers, eightLayers}					OPTIONAL

}

-- TAG-BANDCOMBINATIONLIST-STOP

-- ASN1STOP

BandAndDL-ParametersNR field descriptions

intraBandFreqSeparationDL R4 2-3: Non-contiguous intra-band CA frequency separation class for FR2 as in the RAN4 LS R4-1803363

CA-ParametersNR field descriptions

simultaneousRxTxInterBandCA R4 2-5: Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD)

supportedBandwidthCombinationSet BCS related to R4 2-1 and Updated CA BW class in R4-1803374

IntraBandContiguousCC-InfoDL field descriptions

maxNumberMIMO-LayersPDSCH Related to RAN4 LS R2-1804078

IntraBandContiguousCC-InfoDL-EUTRA field descriptions

mimo-CapabilityDL Related to RAN4 LS R2-1804078

MRDC-Parameters field descriptions

asyncIntraBandENDC R4 2-6: Asynchronous FDD-FDD intra-band EN-DC

simultaneousRxTxInterBandENDC R4 2-4: Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD)

ul-SharingEUTRA-NR R4 1-10: Support of EN-DC with LTE-NR coexistence in UL sharing from UE perspective

ul-SwitchingTimeEUTRA-NR R4 1-11: Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective

–	BandCombinationParametersUL-List

The IE BandCombinationParametersUL-List is used to contain list of NR and/or E-UTRA frequency UL band parameters combination for the supported NR CA and/or MR-DC band combinations included in supportedBandCombination in RF-Parameters and/or RF-Parameters-MRDC.

-- ASN1START

-- TAG-BANDCOMBINATIONPARAMETERSULLIST-START

BandCombinationParametersUL-List ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersUL

BandCombinationParametersUL ::= SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF BandParametersUL

BandParametersUL ::= CHOICE {

bandParametersUL-EUTRA		BandParametersUL-EUTRA,

bandParametersUL-NR			BandParametersUL-NR

}

BandParametersUL-EUTRA ::= SEQUENCE {

ca-BandwidthClassUL-EUTRA	CA-BandwidthClassEUTRA		OPTIONAL,

intraBandContiguousCC-InfoUL-EUTRA-List		SEQUENCE (SIZE (1.. maxNrofServingCellsEUTRA)) OF IntraBandContiguousCC-InfoUL-EUTRA		OPTIONAL

}

BandParametersUL-NR ::= SEQUENCE {

ca-BandwidthClassUL			CA-BandwidthClassNR 				OPTIONAL,

intraBandFreqSeparationUL	FreqSeparationClass					OPTIONAL,

intraBandContiguousCC-InfoUL-List		SEQUENCE (SIZE (1.. maxNrofServingCells)) OF IntraBandContiguousCC-InfoUL			OPTIONAL

}

IntraBandContiguousCC-InfoUL ::=	SEQUENCE {

maxNumberMIMO-LayersCB-PUSCH		MIMO-LayersUL					OPTIONAL,

maxNumberMIMO-LayersNonCB-PUSCH		MIMO-LayersUL					OPTIONAL

}

IntraBandContiguousCC-InfoUL-EUTRA ::=	SEQUENCE {

mimo-CapabilityUL						ENUMERATED {twoLayers, fourLayers}					OPTIONAL

}

-- TAG-BANDCOMBINATIONPARAMETERSULLIST-STOP

-- ASN1STOP

BandParametersUL-NR field descriptions

intraBandFreqSeparationUL R4 2-3: Non-contiguous intra-band CA frequency separation class for FR2 as in the RAN4 LS R4-1803363

IntraBandContiguousCC-InfoUL field descriptions

maxNumberMIMO-LayersCB-PUSCH Related to RAN4 LS R2-1804078

IntraBandContiguousCC-InfoUL-EUTRA field descriptions

mimo-CapabilityUL Related to RAN4 LS R2-1804078

–	BasebandCombinationParametersUL-List

-- ASN1START

-- TAG-BASEBANDCOMBINATIONPARAMETERSULLIST-START

BasebandCombinationParametersUL-List ::= SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandCombinationParametersUL

BasebandCombinationParametersUL ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BasebandParametersPerBandUL

BasebandParametersPerBandUL ::= SEQUENCE {

ca-BandwidthClassUL				CA-BandwidthClassNR,

freqRange						ENUMERATED {fr1, fr2},

basebandParametersPerCC-UL		SEQUENCE (SIZE (1.. maxNrofServingCells)) OF BasebandParametersPerCC-UL

}

BasebandParametersPerCC-UL ::= SEQUENCE {

supportedSubcarrierSpacingUL				SubcarrierSpacing,

supportedBandwidthUL		CHOICE {

fr1							ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100},

fr2							ENUMERATED {mhz50, mhz100, mhz200, mhz400}

},

scalingFactor0dot75				ENUMERATED {supported}		OPTIONAL,

maxNumberMIMO-LayersCB-PUSCH		MIMO-LayersUL					OPTIONAL,

maxNumberMIMO-LayersNonCB-PUSCH		MIMO-LayersUL					OPTIONAL,

supportedModulationOrderUL					ModulationOrder					OPTIONAL,

supportedSRS-Resources			SRS-Resources								OPTIONAL,

srs-TxSwitch					SRS-TxSwitch								OPTIONAL,

lowLatencyCSI-Feedback				ENUMERATED {supported}					OPTIONAL,

pusch-DifferentTB-PerSlot		SEQUENCE {

scs-15kHz						ENUMERATED {upto2, upto7}					OPTIONAL,

scs-30kHz						ENUMERATED {upto2, upto7}					OPTIONAL,

scs-60kHz						ENUMERATED {upto2, upto7}					OPTIONAL,

scs-120kHz						ENUMERATED {upto2, upto7}					OPTIONAL

}																			OPTIONAL,

twoPUCCH-Group					ENUMERATED {supported}						OPTIONAL,

diffNumerologyAcrossPUCCH-Group	ENUMERATED {supported}						OPTIONAL,

diffNumerologyWithinPUCCH-Group	ENUMERATED {supported}						OPTIONAL,

crossCarrierScheduling			ENUMERATED {supported}						OPTIONAL,

supportedNumberTAG				ENUMERATED {n2, n3, n4}						OPTIONAL,

dynamicSwitchSUL				ENUMERATED {supported}						OPTIONAL,

simultaneousTxSUL-NonSUL		ENUMERATED {supported}						OPTIONAL,

searchSpaceSharingCA-UL			ENUMERATED {supported}						OPTIONAL,

csi-ReportFramework				CSI-ReportFramework							OPTIONAL

}

-- TAG-BASEBANDCOMBINATIONPARAMETERSULLIST-STOP

-- ASN1STOP

BasebandParametersPerCC-UL field descriptions

crossCarrierScheduling R1 6-10: Cross carrier scheduling

diffNumerologyAcrossPUCCH-Group R1 6-8: Different numerology across PUCCH groups

diffNumerologyWithinPUCCH-Group R1 6-9: Different numerologies across carriers within the same PUCCH group

dynamicSwitchSUL R1 6-18: Supplemental uplink with dynamic switch

lowLatencyCSI-Feedback R1 2-57: Support low latency CSI feedback

maxNumberMIMO-LayersCB-PUSCH R1 2-14: Codebook based PUSCH MIMO transmission. Absence of this field implies that CB-based PUSCH is not supported.

maxNumberMIMO-LayersNonCB-PUSCH R1 2-15: Non-codebook based PUSCH MIMO transmission. Absence of this field implies that Non-CB-based PUSCH is not supported.

pusch-DifferentTB-PerSlot R1 5-12 & 5-12a: Up to 2/7 PUSCHs per slot for different TBs

scalingFactor0dot75 R2-1800012. To be confirmed by RAN1

searchSpaceSharingCA-UL R1 6-22: UL search space sharing for CA

simultaneousTxSUL-NonSUL R1 6-19: Simultaneous transmission of SRS on an SUL/non-SUL carrier and PUSCH/PUCCH/SRS/PRACH on the other UL carrier in the same cell Details on the channel/signal combination are to be described in TS 38.306

srs-TxSwitch R1 2-55: SRS Tx switch

supportedBandwidthUL Accoding to the RAN4 LS R4-1803563, maximum Bandwidth supported per CC is added in BPC FFS how to work together with BCS and max BW for each CC to be defined for each CA band combination in the RAN4 spec.

supportedModulationOrderUL Accoding to the RAN4 LS R4-1803563, modulation order is added per CC granularity in BPC FFS whether all of modulation order specified in the spec need to be signalled. FFS how to address the requirements agreed by RAN4, e.g. mandaotry w/o capabiltiy for 64QAM. mandaotry with capabiltiy for DL 256QAM in FR1.

supportedNumberTAG R1 6-11: Number of supported TAGs

supportedSRS-Resources R1 2-53: SRS resources

supportedSubcarrierSpacingUL R4 2-2: Simultaneous reception or transmission with same or different numerologies in CA It is expressed by the combination of SCS whether simultaneous RxTx is supported or not.

twoPUCCH-Group R1 6-7: Two PUCCH group

–	BasebandProcessingCombinationMRDC

-- ASN1START

-- TAG-BASEBANDPROCESSINGCOMBINATIONMRDC-START

BasebandProcessingCombinationMRDC ::= SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandProcessingCombinationLink

BasebandProcessingCombinationLink ::= SEQUENCE {

basebandProcessingCombinationIndexMN		BasebandProcessingCombinationIndex,

basebandProcessingCombinationLinkedIndexSN	SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandProcessingCombinationIndex

}

BasebandProcessingCombinationIndex ::= INTEGER (1..maxBasebandProcComb)

-- TAG-BASEBANDPROCESSINGCOMBINATIONMRDC-STOP

-- ASN1STOP

–	CA-BandwidthClassNR

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSNR-START

-- Updated based on R4-1803374

CA-BandwidthClassNR ::= ENUMERATED {a, b, c, d, e, f, g, h, i, j, k, l, m, n, o, p, q, ...}

-- TAG-CA-BANDWIDTHCLASSNR-STOP

-- ASN1STOP

–	CA-BandwidthClassEUTRA

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSEUTRA-START

CA-BandwidthClassEUTRA ::=	ENUMERATED {a, b, c, d, e, f, ...}

-- TAG-CA-BANDWIDTHCLASSEUTRA-STOP

-- ASN1STOP

–	FreqBandIndicatorEUTRA

-- ASN1START

-- TAG-FREQ-BAND-INDICATOR-EUTRA-START

FreqBandIndicatorEUTRA ::=	INTEGER (1..maxBandsEUTRA)

-- TAG-FREQ-BAND-INDICATOR-EUTRA-STOP

-- ASN1STOP

–	FreqBandList

The IE FreqBandList is used to contain list of NR and/or E-UTRA frequency bands for which the UE is requested to provide its supported NR CA and/or MR-DC band combinations (i.e. within the UE capability containers for NR and MR-DC, as requested by E-UTRA).

FreqBandList information element

-- ASN1START

-- TAG-FREQBANDLIST-START

FreqBandList ::=	SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation

FreqBandInformation ::= CHOICE {

bandEUTRA				FreqBandIndicatorEUTRA,

bandNR					FreqBandIndicatorNR

}

-- TAG-FREQBANDLIST-STOP

-- ASN1STOP

–	FreqSeparationClass

The IE FreqSeparationClass is used for an intra-band non-contiguous CA band combination to indicate frequency separation between lower edge of lowest CC and upper edge of highest CC in a frequency band.

FreqSeparationClass information element

-- ASN1START

-- TAG-FREQSEPARATIONCLASS-START

FreqSeparationClass ::=	ENUMERATED {c1, c2, c3, ...}

-- TAG-FREQSEPARATIONCLASS-STOP

-- ASN1STOP

–	MIMO-Layers

-- ASN1START

-- TAG-MIMO-LAYERS-START

MIMO-LayersDL ::=	ENUMERATED {twoLayers, fourLayers, eightLayers}

MIMO-LayersUL ::=	ENUMERATED {oneLayer, twoLayers, fourLayers}

-- TAG-MIMO-LAYERS-STOP

-- ASN1STOP

–	ModulationOrder

-- ASN1START

-- TAG-MODULATION-ORDER-START

ModulationOrder ::=	ENUMERATED {bpsk-halfpi, bpsk, qpsk, qam16, qam64, qam256}

-- TAG-MODULATION-ORDER-STOP

-- ASN1STOP

–	RAT-Type

The IE RAT-Type is used to indicate the radio access technology (RAT), including NR, of the requested/transferred UE capabilities.

RAT-Type information element

-- ASN1START

-- TAG-RAT-TYPE-START

RAT-Type ::= ENUMERATED {nr, eutra-nr, spare2, spare1, ...}

-- TAG-RAT-TYPE-STOP

-- ASN1STOP

–	SupportedBasebandProcessingCombination

-- ASN1START

-- TAG-SUPPORTEDBASEBANDPROCESSINGCOMBINATION-START

SupportedBasebandProcessingCombination ::= SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandProcessingCombination

BasebandProcessingCombination ::= SEQUENCE {

basebandParametersDL			SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BasebandParametersPerBandDL,

basebandParametersUL				BIT STRING (SIZE (1..maxBasebandProcComb))}

BasebandParametersPerBandDL ::= SEQUENCE {

ca-BandwidthClassDL			CA-BandwidthClassNR,

freqRange					ENUMERATED {fr1, fr2},

basebandParametersPerCC-DL		SEQUENCE (SIZE (1.. maxNrofServingCells)) OF BasebandParametersPerCC-DL

}

BasebandParametersPerCC-DL ::= SEQUENCE {

supportedSubcarrierSpacingDL				SubcarrierSpacing,

supportedBandwidthDL		CHOICE {

fr1							ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100},

fr2							ENUMERATED {mhz50, mhz100, mhz200, mhz400}

},

scalingFactor0dot75			ENUMERATED {supported}		OPTIONAL,

timeDurationForQCL					SEQUENCE {

scs-60kHz							ENUMERATED {s7, s14, s28}				OPTIONAL,

sch-120kHz							ENUMERATED {s14, s28}					OPTIONAL

}																			OPTIONAL,

scellWithoutSSB					ENUMERATED {supported}						OPTIONAL,

csi-RS-MeasSCellWithoutSSB		ENUMERATED {supported}						OPTIONAL,

maxNumberMIMO-LayersPDSCH			MIMO-LayersDL							OPTIONAL,

supportedModulationOrderDL					ModulationOrder					OPTIONAL,

srs-AssocCSI-RS					ENUMERATED {supported}						OPTIONAL,

type1-3-CSS						ENUMERATED {supported}						OPTIONAL,

pdcchMonitoringAnyOccasions		ENUMERATED {withoutDCI-gap, withDCI-gap}	OPTIONAL,

ue-SpecificUL-DL-Assignment		ENUMERATED {supported}						OPTIONAL,

pdsch-DifferentTB-PerSlot		SEQUENCE {

scs-15kHz						ENUMERATED {upto2, upto7}					OPTIONAL,

scs-30kHz						ENUMERATED {upto2, upto7}					OPTIONAL,

scs-60kHz						ENUMERATED {upto2, upto7}					OPTIONAL,

scs-120kHz						ENUMERATED {upto2, upto7}					OPTIONAL

}																			OPTIONAL,

crossCarrierScheduling			ENUMERATED {supported}						OPTIONAL,

searchSpaceSharingCA-DL			ENUMERATED {supported}						OPTIONAL,

typeI-MultiPanelCodebook			SEQUENCE (SIZE (1..ffsValue)) OF TypeI-MultiPanelCodebook		OPTIONAL,

typeII-CodebookPortSelectionList	SEQUENCE (SIZE (1..ffsValue)) OF TypeII-CodebookPortSelection	OPTIONAL

}

-- TAG-SUPPORTEDBASEBANDPROCESSINGCOMBINATION-STOP

-- ASN1STOP

BasebandParametersPerCC-DL field descriptions

crossCarrierScheduling R1 6-10: Cross carrier scheduling

csi-RS-MeasSCellWithoutSSB R1 1-11: Support of CSI-RS RRM measurement for SCell without SS/PBCH block

maxNumberMIMO-LayersPDSCH R1 2-3: PDSCH MIMO layers. Absence of this field implies support of one layer.

pdcchMonitoringAnyOccasions R1 3-5 & 3-5a: For type 1 with dedicated RRC configuration, type 3, and UE-SS,, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 (with a DCI gap)

pdsch-DifferentTB-PerSlot R1 5-11 & 5-11a: Up to 2/7 unicast PDSCHs per slot for different TBs

scalingFactor0dot75 R2-1800012. To be confirmed by RAN1

scellWithoutSSB R1 1-10: Support of SCell without SS/PBCH block

searchSpaceSharingCA-DL R1 6-21: DL search space sharing for CA

srs-AssocCSI-RS R1 2-15a: Association between CSI-RS and SRS

supportedBandwidthDL Accoding to the RAN4 LS R4-1803563, maximum Bandwidth supported per CC is added in BPC FFS how to work together with BCS and max BW for each CC to be defined for each CA band combination in the RAN4 spec.

supportedModulationOrderDL Accoding to the RAN4 LS R4-1803563, modulation order is added per CC granularity in BPC FFS whether all of modulation order specified in the spec need to be signalled. FFS how to address the requirements agreed by RAN4, e.g. mandaotry w/o capabiltiy for 64QAM. mandaotry with capabiltiy for DL 256QAM in FR1.

supportedSubcarrierSpacingDL R4 2-2: Simultaneous reception or transmission with same or different numerologies in CA It is expressed by the combination of SCS whether simultaneous RxTx is supported or not.

timeDurationForQCL R1 2-2: PDSCH beam switching

type1-3-CSS R1 3-1a: For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 3 OFDM symbols for FR2

ue-SpecificUL-DL-Assignment R1 5-1a: UE specific RRC configure UL/DL assignment

–	UE-CapabilityRAT-ContainerList

The IE UE-CapabilityRAT-ContainerList contains a list of containers, one for each RAT for which UE capabilities are transferred, if any.

UE-CapabilityRAT-ContainerList information element

-- ASN1START

-- TAG-UE-CAPABILITY-RAT-CONTAINER-LIST-START

UE-CapabilityRAT-ContainerList ::=SEQUENCE (SIZE (0.. maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Container

UE-CapabilityRAT-Container ::= SEQUENCE {

rat-Type							RAT-Type,

ue-CapabilityRAT-Container			OCTET STRING

}

-- TAG-UE-CAPABILITY-RAT-CONTAINER-LIST-STOP

-- ASN1STOP

UE-CapabilityRAT-ContainerList field descriptions

ue-CapabilityRAT-Container Container for the UE capabilities of the indicated RAT. The encoding is defined in the specification of each RAT: For NR: the encoding of UE capabilities is defined in UE-NR-Capability. For EUTRA-NR: the encoding of UE capabilities is defined in UE-MRDC-Capability

–	UE-MRDC-Capability

The IE UE-MRDC-Capability is used to convey the UE Radio Access Capability Parameters for MR-DC, see TS 38.306 [yy].

UE-MRDC-Capability information element

-- ASN1START

-- TAG-UE-MRDC-CAPABILITY-START

UE-MRDC-Capability ::=	SEQUENCE {

measParametersMRDC					MeasParametersMRDC					OPTIONAL,

rf-ParametersMRDC					RF-ParametersMRDC,

phy-ParametersMRDC					Phy-ParametersMRDC					OPTIONAL,

generalParametersMRDC				GeneralParametersMRDC-XDD-Diff		OPTIONAL,

fdd-Add-UE-MRDC-Capabilities		UE-MRDC-CapabilityAddXDD-Mode		OPTIONAL,

tdd-Add-UE-MRDC-Capabilities		UE-MRDC-CapabilityAddXDD-Mode		OPTIONAL,

fr1-Add-UE-MRDC-Capabilities		UE-MRDC-CapabilityAddFRX-Mode		OPTIONAL,

fr2-Add-UE-MRDC-Capabilities		UE-MRDC-CapabilityAddFRX-Mode		OPTIONAL,

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode ::=	SEQUENCE {

phy-ParametersMRDC-XDD-Diff		Phy-ParametersMRDC-XDD-Diff		OPTIONAL,

measParametersMRDC-XDD-Diff		MeasParametersMRDC-XDD-Diff		OPTIONAL,

generalParametersMRDC-XDD-Diff		GeneralParametersMRDC-XDD-Diff		OPTIONAL

}

UE-MRDC-CapabilityAddFRX-Mode ::=	SEQUENCE {

phy-ParametersMRDC-FRX-Diff		Phy-ParametersMRDC-FRX-Diff		OPTIONAL,

measParametersMRDC-FRX-Diff		MeasParametersMRDC-FRX-Diff

}

RF-ParametersMRDC ::= SEQUENCE {

supportedBandCombination	BandCombinationList,

bandCombinationParametersUL-List	BandCombinationParametersUL-List

}

Phy-ParametersMRDC ::= SEQUENCE {

phy-ParametersMRDC-XDD-Diff		Phy-ParametersMRDC-XDD-Diff		OPTIONAL,

phy-ParametersMRDC-FRX-Diff		Phy-ParametersMRDC-FRX-Diff		OPTIONAL

}

Phy-ParametersMRDC-XDD-Diff ::=	SEQUENCE {

dynamicPowerSharing					ENUMERATED {supported}				OPTIONAL,

tdm-Pattern							ENUMERATED {supported}				OPTIONAL

}

Phy-ParametersMRDC-FRX-Diff ::=	SEQUENCE {

dynamicPowerSharing					ENUMERATED {supported}				OPTIONAL,

tdm-Pattern							ENUMERATED {supported}				OPTIONAL

}

MeasParametersMRDC ::= SEQUENCE {

measParametersMRDC-Common		MeasParametersMRDC-Common,

measParametersMRDC-XDD-Diff		MeasParametersMRDC-XDD-Diff				OPTIONAL,

measParametersMRDC-FRX-Diff		MeasParametersMRDC-FRX-Diff

}

MeasParametersMRDC-Common ::=	SEQUENCE {

independentGapConfig			ENUMERATED {supported}	OPTIONAL

}

MeasParametersMRDC-XDD-Diff ::=	SEQUENCE {

sftd-MeasPSCell							ENUMERATED {supported}		OPTIONAL,

sftd-MeasNR-Cell						ENUMERATED {supported}		OPTIONAL

}

MeasParametersMRDC-FRX-Diff ::=	SEQUENCE {

simultaneousRxDataSSB-DiffNumerology	ENUMERATED {supported}		OPTIONAL

}

GeneralParametersMRDC-XDD-Diff ::= SEQUENCE {

splitSRB-WithOneUL-Path				ENUMERATED {supported}		OPTIONAL,

splitDRB-withUL-Both-MCG-SCG		ENUMERATED {supported}		OPTIONAL,

srb3									ENUMERATED {supported}		OPTIONAL

}

-- TAG-UE-MRDC-CAPABILITY-STOP

-- ASN1STOP

MeasParametersMRDC-Common field descriptions

independentGapConfig R4 3-1: Independent measurement gap configurations for FR1 and FR2

MeasParametersMRDC-FRX-Diff field descriptions

simultaneousRxDataSSB-DiffNumerology R4 3-2: Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement

Phy-ParametersMRDC-FRX-Diff field descriptions

dynamicPowerSharing R1 8-1: Dynamic power sharing for LTE-NR DC

tdm-Pattern R1 6-13 & 8-2: Case 1 Single Tx UL LTE-NR DC

Phy-ParametersMRDC-XDD-Diff field descriptions

dynamicPowerSharing R1 8-1: Dynamic power sharing for LTE-NR DC

tdm-Pattern R1 6-13 & 8-2: Case 1 Single Tx UL LTE-NR DC

–	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [yy].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::= SEQUENCE {

accessStratumRelease			AccessStratumRelease,

pdcp-Parameters					PDCP-Parameters,

rlc-Parameters					RLC-Parameters						OPTIONAL,

mac-Parameters					MAC-Parameters						OPTIONAL,

phy-Parameters					Phy-Parameters,

rf-Parameters					RF-Parameters,

measParameters					MeasParameters						OPTIONAL,

fdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-Mode			OPTIONAL,

tdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-Mode			OPTIONAL,

fr1-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL,

fr2-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL,

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=	SEQUENCE {

phy-ParametersXDD-Diff			Phy-ParametersXDD-Diff			OPTIONAL,

mac-ParametersXDD-Diff			MAC-ParametersXDD-Diff			OPTIONAL,

measParametersXDD-Diff			MeasParametersXDD-Diff			OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode ::=	SEQUENCE {

phy-ParametersFRX-Diff			Phy-ParametersFRX-Diff			OPTIONAL,

measParametersFRX-Diff			MeasParametersFRX-Diff			OPTIONAL

}

Phy-Parameters ::=	SEQUENCE {

phy-ParametersCommon			Phy-ParametersCommon				OPTIONAL,

phy-ParametersXDD-Diff			Phy-ParametersXDD-Diff				OPTIONAL,

phy-ParametersFRX-Diff			Phy-ParametersFRX-Diff				OPTIONAL,

phy-ParametersFR1				Phy-ParametersFR1					OPTIONAL,

phy-ParametersFR2				Phy-ParametersFR2					OPTIONAL,

supportedBasebandProcessingCombination		SupportedBasebandProcessingCombination,

basebandCombinationParametersUL-List		BasebandCombinationParametersUL-List

}

Phy-ParametersCommon ::=	SEQUENCE {

csi-RS-CFRA-ForHO					ENUMERATED {supported}						OPTIONAL,

dynamicPRB-BundlingDL				ENUMERATED {supported}						OPTIONAL,

sp-CSI-ReportPUCCH					ENUMERATED {supported}						OPTIONAL,

sp-CSI-ReportPUSCH					ENUMERATED {supported}						OPTIONAL,

nzp-CSI-RS-IntefMgmt				ENUMERATED {supported}						OPTIONAL,

type2-SP-CSI-Feedback-LongPUCCH		ENUMERATED {supported}						OPTIONAL,

multipleCORESET						ENUMERATED {supported}						OPTIONAL,

dynamicSFI							ENUMERATED {supported}						OPTIONAL,

precoderGranularityCORESET			ENUMERATED {supported}						OPTIONAL,

dynamicHARQ-ACK-Codebook			ENUMERATED {supported}						OPTIONAL,

semiStaticHARQ-ACK-Codebook			ENUMERATED {supported}						OPTIONAL,

spatialBundlingHARQ-ACK				ENUMERATED {supported}						OPTIONAL,

dynamicBetaOffsetInd-HARQ-ACK-CSI	ENUMERATED {supported}						OPTIONAL,

pucch-Repetition-F1-3-4				ENUMERATED {supported}						OPTIONAL,

ra-Type0-PUSCH						ENUMERATED {supported}						OPTIONAL,

dynamicSwitchRA-Type0-1-PDSCH		ENUMERATED {supported}						OPTIONAL,

dynamicSwitchRA-Type0-1-PUSCH		ENUMERATED {supported}						OPTIONAL,

pdsch-MappingTypeA					ENUMERATED {supported}						OPTIONAL,

pdsch-MappingTypeB					ENUMERATED {supported}						OPTIONAL,

interleavingVRB-ToPRB-PDSCH			ENUMERATED {supported}						OPTIONAL,

interleavingVRB-ToPRB-PUSCH			ENUMERATED {supported}						OPTIONAL,

interSlotFreqHopping-PUSCH			ENUMERATED {supported}						OPTIONAL,

type1-PUSCH-RepetitionOneSlot		ENUMERATED {supported}						OPTIONAL,

type1-PUSCH-RepetitionMultiSlots	ENUMERATED {supported}						OPTIONAL,

type2-PUSCH-RepetitionOneSlot		ENUMERATED {supported}						OPTIONAL,

type2-PUSCH-RepetitionMultiSlots	ENUMERATED {supported}						OPTIONAL,

pusch-RepetitionMultiSlots			ENUMERATED {supported}						OPTIONAL,

pdsch-RepetitionMultiSlots			ENUMERATED {supported}						OPTIONAL,

downlinkSPS							ENUMERATED {supported}						OPTIONAL,

configuredUL-GrantType1				ENUMERATED {supported}						OPTIONAL,

configuredUL-GrantType2				ENUMERATED {supported}						OPTIONAL,

pre-EmptIndication-DL				ENUMERATED {supported}						OPTIONAL,

cbg-TransIndication					BIT STRING (SIZE (2))						OPTIONAL,

cbg-FlushIndication-DL				ENUMERATED {supported}						OPTIONAL,

dynamicHARQ-ACK-CodeB-CBG-Retx-DL	ENUMERATED {supported}						OPTIONAL,

rateMatchingResrcSetSemi-Static		ENUMERATED {supported}						OPTIONAL,

rateMatchingResrcSetDynamic			ENUMERATED {supported}						OPTIONAL,

rateMatchingLTE-CRS					ENUMERATED {supported}						OPTIONAL,

bwp-SwitchingDelay					ENUMERATED {type1, type2}					OPTIONAL

}

Phy-ParametersXDD-Diff ::=	SEQUENCE {

twoPUCCH-F0-2-ConsecSymbols			ENUMERATED {supported}						OPTIONAL,

twoDifferentTPC-Loop-PUSCH			ENUMERATED {supported}						OPTIONAL,

twoDifferentTPC-Loop-PUCCH			ENUMERATED {supported}						OPTIONAL

}

Phy-ParametersFRX-Diff ::=	SEQUENCE {

oneFL-DMRS-TwoAdditionalDMRS		BIT STRING (SIZE (2))						OPTIONAL,

twoFL-DMRS							BIT STRING (SIZE (2))						OPTIONAL,

twoFL-DMRS-TwoAdditionalDMRS		BIT STRING (SIZE (2))						OPTIONAL,

oneFL-DMRS-ThreeAdditionalDMRS		BIT STRING (SIZE (2))						OPTIONAL,

supportedDMRS-TypeDL				ENUMERATED {type1, type2}					OPTIONAL,

supportedDMRS-TypeUL				ENUMERATED {type1, type2}					OPTIONAL,

semiOpenLoopCSI						ENUMERATED {supported}						OPTIONAL,

csi-ReportWithoutPMI				ENUMERATED {supported}						OPTIONAL,

csi-ReportWithCRI					ENUMERATED {supported}						OPTIONAL,

csi-ReportWithoutCQI				ENUMERATED {supported}						OPTIONAL,

onePortsPTRS						BIT STRING (SIZE (2))						OPTIONAL,

twoPUCCH-F0-2-ConsecSymbols			ENUMERATED {supported}						OPTIONAL,

pucch-F2-WithFH						ENUMERATED {supported}						OPTIONAL,

pucch-F3-WithFH						ENUMERATED {supported}						OPTIONAL,

pucch-F4-WithFH						ENUMERATED {supported}						OPTIONAL,

freqHoppingPUCCH-F0-2				ENUMERATED {notSupported}					OPTIONAL,

freqHoppingPUCCH-F1-3-4				ENUMERATED {notSupported}					OPTIONAL,

mux-SR-HARQ-ACK-CSI-PUCCH			ENUMERATED {supported}						OPTIONAL,

uci-CodeBlockSegmentation			ENUMERATED {supported}						OPTIONAL,

onePUCCH-LongAndShortFormat			ENUMERATED {supported}						OPTIONAL,

twoPUCCH-AnyOthersInSlot			ENUMERATED {supported}						OPTIONAL,

intraSlotFreqHopping-PUSCH			ENUMERATED {supported}						OPTIONAL,

pusch-LBRM							ENUMERATED {supported}						OPTIONAL,

pdcch-BlindDetectionCA				ENUMERATED {supported}						OPTIONAL,

tpc-PUSCH-RNTI						ENUMERATED {supported}						OPTIONAL,

tpc-PUCCH-RNTI						ENUMERATED {supported}						OPTIONAL,

tpc-SRS-RNTI						ENUMERATED {supported}						OPTIONAL,

absoluteTPC-Command					ENUMERATED {supported}						OPTIONAL,

twoDifferentTPC-Loop-PUSCH			ENUMERATED {supported}						OPTIONAL,

twoDifferentTPC-Loop-PUCCH			ENUMERATED {supported}						OPTIONAL,

pusch-HalfPi-BPSK					ENUMERATED {supported}						OPTIONAL,

pucch-F3-4-HalfPi-BPSK				ENUMERATED {supported}						OPTIONAL,

oneSymbolGP-TDD						ENUMERATED {supported}						OPTIONAL,

almostContiguousCP-OFDM-UL			ENUMERATED {supported}						OPTIONAL

}

Phy-ParametersFR1 ::=	SEQUENCE {

pdcchMonitoringSingleOccasion		ENUMERATED {supported}						OPTIONAL,

scs-60kHz							ENUMERATED {supported}						OPTIONAL,

pdsch-256QAM-FR1					ENUMERATED {supported}						OPTIONAL,

pdsch-RE-MappingFR1					ENUMERATED {n10, n20}						OPTIONAL

}

Phy-ParametersFR2 ::=	SEQUENCE {

calibrationGapPA					ENUMERATED {supported}						OPTIONAL,

pdsch-RE-MappingFR2					ENUMERATED {n6, n20}						OPTIONAL

}

RF-Parameters ::= SEQUENCE {

supportedBandListNR					SupportedBandListNR,

supportedBandCombination			BandCombinationList,

bandCombinationParametersUL-List	BandCombinationParametersUL-List

}

SupportedBandListNR ::=	SEQUENCE (SIZE (1..maxBands)) OF BandNR

BandNR ::=	SEQUENCE {

bandNR							FreqBandIndicatorNR,

modifiedMPR-Behaviour			BIT STRING (SIZE (8))						OPTIONAL,

maxChannelBW-PerCC				ENUMERATED {mhz400}							OPTIONAL,

mimo-ParametersPerBand			MIMO-ParametersPerBand						OPTIONAL,

extendedCP						ENUMERATED {supported}						OPTIONAL,

scellWithoutSSB					ENUMERATED {supported}						OPTIONAL,

csi-RS-MeasSCellWithoutSSB		ENUMERATED {supported}						OPTIONAL,

srs-AssocCSI-RS					ENUMERATED {supported}						OPTIONAL,

type1-3-CSS						ENUMERATED {supported}						OPTIONAL,

multipleTCI						ENUMERATED {supported}						OPTIONAL,

pdcchMonitoringAnyOccasions		ENUMERATED {withoutDCI-gap, withDCI-gap}	OPTIONAL,

ue-SpecificUL-DL-Assignment		ENUMERATED {supported}						OPTIONAL,

pdsch-DifferentTB-PerSlot		SEQUENCE {

scs-15kHz						ENUMERATED {upto2, upto7}				OPTIONAL,

scs-30kHz						ENUMERATED {upto2, upto7}				OPTIONAL,

scs-60kHz						ENUMERATED {upto2, upto7}				OPTIONAL,

scs-120kHz						ENUMERATED {upto2, upto7}				OPTIONAL

}																			OPTIONAL,

pusch-DifferentTB-PerSlot		SEQUENCE {

scs-15kHz						ENUMERATED {upto2, upto7}				OPTIONAL,

scs-30kHz						ENUMERATED {upto2, upto7}				OPTIONAL,

scs-60kHz						ENUMERATED {upto2, upto7}				OPTIONAL,

scs-120kHz						ENUMERATED {upto2, upto7}				OPTIONAL

}																			OPTIONAL,

bwp-SameNumerology					ENUMERATED {upto2, upto4}				OPTIONAL,

bwp-DiffNumerology					ENUMERATED {upto4}						OPTIONAL,

twoPUCCH-Group					ENUMERATED {supported}						OPTIONAL,

diffNumerologyAcrossPUCCH-Group	ENUMERATED {supported}						OPTIONAL,

diffNumerologyWithinPUCCH-Group	ENUMERATED {supported}						OPTIONAL,

crossCarrierScheduling			ENUMERATED {supported}						OPTIONAL,

supportedNumberTAG				ENUMERATED {n2, n3, n4}						OPTIONAL,

simultaneousTxSUL-NonSUL		ENUMERATED {supported}						OPTIONAL,

searchSpaceSharingCA-DL			ENUMERATED {supported}						OPTIONAL,

searchSpaceSharingCA-UL			ENUMERATED {supported}						OPTIONAL,

pdsch-256QAM-FR2				ENUMERATED {supported}						OPTIONAL,

pusch-256QAM					ENUMERATED {supported}						OPTIONAL,

ue-PowerClass					INTEGER (1..16)								DEFAULT 3,

maxUplinkDutyCycle				INTEGER (1..9)								DEFAULT 5

}

MIMO-ParametersPerBand ::= SEQUENCE {

timeDurationForQCL					SEQUENCE {

scs-60kHz							ENUMERATED {s7, s14, s28}									OPTIONAL,

sch-120kHz							ENUMERATED {s14, s28}										OPTIONAL

}																									OPTIONAL,

maxNumberMIMO-LayersPDSCH			MIMO-LayersDL													OPTIONAL,

maxNumberMIMO-LayersCB-PUSCH		MIMO-LayersUL													OPTIONAL,

maxNumberMIMO-LayersNonCB-PUSCH		MIMO-LayersUL													OPTIONAL,

maxNumberConfiguredTCIstates		ENUMERATED {n4, n8, n16, n32, n64}								OPTIONAL,

maxNumberActiveTCI-PerCC			ENUMERATED {n1, n2, n4, n8}										OPTIONAL,

pusch-TransCoherence				ENUMERATED {nonCoherent, partialNonCoherent, fullCoherent}	OPTIONAL,

beamCorrespondence					ENUMERATED {supported}											OPTIONAL,

periodicBeamReport					ENUMERATED {supported}											OPTIONAL,

aperiodicBeamReport					ENUMERATED {supported}											OPTIONAL,

sp-BeamReportPUCCH					ENUMERATED {supported}											OPTIONAL,

sp-BeamReportPUSCH					ENUMERATED {supported}											OPTIONAL,

beamManagementSSB-CSI-RS			BeamManagementSSB-CSI-RS										OPTIONAL,

maxNumberRxBeam						INTEGER (2..8)													OPTIONAL,

maxNumberRxTxBeamSwitchDL			SEQUENCE {

scs-15kHz							ENUMERATED {n4, n7, n14}									OPTIONAL,

scs-30kHz							ENUMERATED {n4, n7, n14}									OPTIONAL,

scs-60kHz							ENUMERATED {n4, n7, n14}									OPTIONAL,

scs-120kHz							ENUMERATED {n4, n7, n14}									OPTIONAL,

scs-240kHz							ENUMERATED {n4, n7, n14}									OPTIONAL

}																									OPTIONAL,

maxNumberNonGroupBeamReporting		ENUMERATED {n1, n2, n4}											OPTIONAL,

groupBeamReporting					ENUMERATED {supported}											OPTIONAL,

uplinkBeamManagement				SEQUENCE {

maxNumberSRS-ResourcePerSet			ENUMERATED {n8, n16, n32},

maxNumberSRS-ResourceSet			INTEGER (1..8)

}																									OPTIONAL,

maxNumberCSI-RS-BFR					INTEGER (1..64)													OPTIONAL,

maxNumberSSB-BFR					INTEGER (1..64)													OPTIONAL,

maxNumberCSI-RS-SSB-BFR				INTEGER (1..256)												OPTIONAL,

twoPortsPTRS						BIT STRING (SIZE (2))											OPTIONAL,

supportedSRS-Resources			SRS-Resources														OPTIONAL,

srs-TxSwitch					SRS-TxSwitch														OPTIONAL,

maxNumberSimultaneousSRS-PerCC		INTEGER (1..4)													OPTIONAL,

lowLatencyCSI-Feedback				ENUMERATED {supported}											OPTIONAL,

csi-ReportFramework					CSI-ReportFramework												OPTIONAL,

typeI-MultiPanelCodebook			SEQUENCE (SIZE (1..ffsValue)) OF TypeI-MultiPanelCodebook		OPTIONAL,

typeII-CodebookPortSelectionList	SEQUENCE (SIZE (1..ffsValue)) OF TypeII-CodebookPortSelection	OPTIONAL,

ptrs-DensityRecommendationSetDL		SEQUENCE {

scs-15kHz							PTRS-DensityRecommendationDL								OPTIONAL,

scs-30kHz							PTRS-DensityRecommendationDL								OPTIONAL,

scs-60kHz							PTRS-DensityRecommendationDL								OPTIONAL,

scs-120kHz							PTRS-DensityRecommendationDL								OPTIONAL

}																									OPTIONAL,

ptrs-DensityRecommendationSetUL		SEQUENCE {

scs-15kHz							PTRS-DensityRecommendationUL								OPTIONAL,

scs-30kHz							PTRS-DensityRecommendationUL								OPTIONAL,

scs-60kHz							PTRS-DensityRecommendationUL								OPTIONAL,

scs-120kHz							PTRS-DensityRecommendationUL								OPTIONAL

}																									OPTIONAL,

csi-RS-ForTracking					CSI-RS-ForTracking												OPTIONAL

}

-- R1 2-24: SSB/CSI-RS for beam management

BeamManagementSSB-CSI-RS ::=	SEQUENCE {

maxNumberSSB-CSI-RS-ResourceOneTx	ENUMERATED {n8, n16, n32, n64},

maxNumberSSB-CSI-RS-ResourceTwoTx	ENUMERATED {n0, n4, n8, n16, n32, n64},

supportedCSI-RS-Density				ENUMERATED {one, three, oneAndThree}

}

CSI-ReportFramework ::=				SEQUENCE {

maxNumberPeriodicCSI-ReportPerBWP			INTEGER (1..4),

maxNumberAperiodicCSI-ReportPerBWP			INTEGER (1..4),

maxNumberSemiPersistentCSI-ReportPerBWP		INTEGER (0..4),

supportedAdvancedCSI-Proc					BIT STRING (SIZE (8)),

simultaneousCSI-ReportsAllCC				INTEGER (5..32),

supportedCSI-Comuptation					ENUMERATED {typeA, typeB}

}

CSI-RS-ForTracking ::=				SEQUENCE {

supportedBandwidth					ENUMERATED {sameAsBWP, minOfPRB52AndBWP, both},

butstLength							INTEGER (1..2),

maxSimultaneousResourceSetsPerCC	INTEGER (1..8),

maxConfiguredResourceSetsPerCC		INTEGER (1..64),

maxConfiguredResourceSetsAllCC		INTEGER (1..128)

}

PTRS-DensityRecommendationDL ::=	SEQUENCE {

frequencyDensity1					INTEGER (1..276),

frequencyDensity2					INTEGER (1..276),

timeDensity1						INTEGER (0..29),

timeDensity2						INTEGER (0..29),

timeDensity3						INTEGER (0..29)

}

PTRS-DensityRecommendationUL ::=	SEQUENCE {

frequencyDensity1					INTEGER (1..276),

frequencyDensity2					INTEGER (1..276),

timeDensity1						INTEGER (0..29),

timeDensity2						INTEGER (0..29),

timeDensity3						INTEGER (0..29),

sampleDensity1						INTEGER (1..276),

sampleDensity2						INTEGER (1..276),

sampleDensity3						INTEGER (1..276),

sampleDensity4						INTEGER (1..276),

sampleDensity5						INTEGER (1..276)

}

-- R1 2-53: SRS resources

SRS-Resources ::=	SEQUENCE {

maxNumberAperiodicSRS-PerBWP					ENUMERATED {n1, n2, n4, n8, n16},

maxNumberAperiodicSRS-PerBWP-PerSlot			INTEGER (1..6),

maxNumberPeriodicSRS-PerBWP						ENUMERATED {n1, n2, n4, n8, n16},

maxNumberPeriodicSRS-PerBWP-PerSlot				INTEGER (1..6),

maxNumberSemiPersitentSRS-PerBWP				ENUMERATED {n0, n1, n2, n4, n8, n16},

maxNumberSP-SRS-PerBWP-PerSlot					INTEGER (0..6),

maxNumberSRS-Ports-PerResource					ENUMERATED {n1, n2, n4}

}

-- R1 2-55: SRS Tx switch

SRS-TxSwitch ::=	SEQUENCE {

supportedSRS-TxPortSwitch			ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4},

txSwitchImpactToRx					ENUMERATED {true}								OPTIONAL

}

TypeI-MultiPanelCodebook ::=		SEQUENCE {

maxNumberTxPortsPerResource			ENUMERATED {p8, p16, p32},

maxNumberResources					INTEGER (1..64),

totalNumberTxPorts					INTEGER (2..256),

supportedCodebookMode				ENUMERATED {mode1, mode2, both},

supportedNumberPanels				ENUMERATED {n2, n4}

}

TypeII-CodebookPortSelection ::=	SEQUENCE {

maxNumberTxPortsPerResource			ENUMERATED {p4, p8, p12, p16, p24, p32},

maxNumberResources					INTEGER (1..64),

totalNumberTxPorts					INTEGER (2..256),

parameterLx							INTEGER (2..4),

amplitudeScalingType				ENUMERATED {wideband, widebandAndSubband}

}

PDCP-Parameters ::= SEQUENCE {

supportedROHC-Profiles	SEQUENCE {

profile0x0000		BOOLEAN,

profile0x0001		BOOLEAN,

profile0x0002		BOOLEAN,

profile0x0003		BOOLEAN,

profile0x0004		BOOLEAN,

profile0x0006		BOOLEAN,

profile0x0101		BOOLEAN,

profile0x0102		BOOLEAN,

profile0x0103		BOOLEAN,

profile0x0104		BOOLEAN

},

maxNumberROHC-ContextSessions	ENUMERATED {cs2, cs4, cs8, cs12, cs16, cs24, cs32, cs48, cs64, cs128, cs256, cs512, cs1024, cs16384, spare2, spare1},

uplinkOnlyROHC-Profiles			ENUMERATED {supported}	OPTIONAL,

continueROHC-Context			ENUMERATED {supported}	OPTIONAL,

outOfOrderDelivery				ENUMERATED {supported}	OPTIONAL,

shortSN							ENUMERATED {supported} 	OPTIONAL

}

RLC-Parameters ::= SEQUENCE {

am-WithShortSN					ENUMERATED {supported}	OPTIONAL,

um-WithShortSN					ENUMERATED {supported}	OPTIONAL,

um-WIthLongSN					ENUMERATED {supported}	OPTIONAL

}

MAC-Parameters ::= SEQUENCE {

mac-ParametersCommon			MAC-ParametersCommon	OPTIONAL,

mac-ParametersXDD-Diff			MAC-ParametersXDD-Diff	OPTIONAL

}

MAC-ParametersCommon ::=	SEQUENCE {

lcp-Restriction					ENUMERATED {supported}	OPTIONAL,

pucch-SpatialRelInfoMAC-CE		ENUMERATED {supported}	OPTIONAL

}

MAC-ParametersXDD-Diff ::=	SEQUENCE {

skipUplinkTxDynamic				ENUMERATED {supported}	OPTIONAL,

logicalChannelSR-DelayTimer		ENUMERATED {supported}	OPTIONAL,

longDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

shortDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

multipleSR-Configurations		ENUMERATED {supported}	OPTIONAL,

multipleConfiguredGrantConfigurations	ENUMERATED {supported}	OPTIONAL

-- If supported UE supports 16 configured grant configurations, otherwise 1 ConfiguredGrant config is supported.

-- Confirmation is needed whether to align the number to what the configuration signalling can support, and to consider whether the 16 refers

-- to the configurations or the active ones only (as they are within the BWP).

}

MeasParameters ::= SEQUENCE {

measParametersXDD-Diff			MeasParametersXDD-Diff		OPTIONAL,

measParametersFRX-Diff			MeasParametersFRX-Diff		OPTIONAL

}

MeasParametersXDD-Diff ::=	SEQUENCE {

intraAndInterF-MeasAndReport	ENUMERATED {supported}	OPTIONAL,

eventA-MeasAndReport			ENUMERATED {supported}	OPTIONAL

-- Confirmation is needed on the need of capability/IOT signaling in LTE for support of the additional measurement gap configurations.

}

MeasParametersFRX-Diff ::=	SEQUENCE {

ss-SINR-Meas						ENUMERATED {supported}		OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithSSB		ENUMERATED {supported}		OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithoutSSB		ENUMERATED {supported}		OPTIONAL,

csi-SINR-Meas						ENUMERATED {supported}		OPTIONAL,

csi-RS-RLM							ENUMERATED {supported}		OPTIONAL

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

BandNR field descriptions

bwp-DiffNumerology R1 6-4: BWP adaptation (up to 4 BWPs) with different numerologies

bwp-SameNumerology R1 6-2 & 6-3: Type A/B BWP adaptation (up to 2/4 BWPs) with same numerology

crossCarrierScheduling R1 6-10: Cross carrier scheduling

csi-RS-MeasSCellWithoutSSB R1 1-11: Support of CSI-RS RRM measurement for SCell without SS/PBCH block

diffNumerologyAcrossPUCCH-Group R1 6-8: Different numerology across PUCCH groups

diffNumerologyWithinPUCCH-Group R1 6-9: Different numerologies across carriers within the same PUCCH group

extendedCP R1 0-10: Extended CP

maxChannelBW-PerCC R4 2-1: Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC RAN4 agreed that 400 MHz is optional for FR2. The other values defined for FR1/fR2 in TS 38.101 are mandatory w/o capability bit.

modifiedMPR-Behaviour Modified MPR behaviour as in RAN4 LS R2-1804077, which is needed for NSA as well as SA

multipleTCI R1 3-4: More than one TCI state configurations per CORESET

pdcchMonitoringAnyOccasions R1 3-5 & 3-5a: For type 1 with dedicated RRC configuration, type 3, and UE-SS,, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 (with a DCI gap)

pdsch-256QAM-FR2 R4 1-4: 256QAM for PDSCH in FR2

pdsch-DifferentTB-PerSlot R1 5-11 & 5-11a: Up to 2/7 unicast PDSCHs per slot for different TBs

pusch-256QAM R4 1-5: 256QAM for PUSCH

pusch-DifferentTB-PerSlot R1 5-12 & 5-12a: Up to 2/7 PUSCHs per slot for different TBs

scellWithoutSSB R1 1-10: Support of SCell without SS/PBCH block

searchSpaceSharingCA-DL R1 6-21: DL search space sharing for CA

searchSpaceSharingCA-UL R1 6-22: UL search space sharing for CA

simultaneousTxSUL-NonSUL R1 6-19: Simultaneous transmission of SRS on an SUL/non-SUL carrier and PUSCH/PUCCH/SRS/PRACH on the other UL carrier in the same cell Details on the channel/signal combination are to be described in TS 38.306

srs-AssocCSI-RS R1 2-15a: Association between CSI-RS and SRS

supportedNumberTAG R1 6-11: Number of supported TAGs

twoPUCCH-Group R1 6-7: Two PUCCH group

type1-3-CSS R1 3-1a: For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 3 OFDM symbols for FR2

ue-SpecificUL-DL-Assignment R1 5-1a: UE specific RRC configure UL/DL assignment

MAC-ParametersCommon field descriptions

pucch-SpatialRelInfoMAC-CE R1 4-24: PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource

MAC-ParametersXDD-Diff field descriptions

multipleConfiguredGrantConfigurations If supported UE supports 8 SR configurations, otherwise 1 SR config is supported. Confirmation is needed whether to align the number to what the configuration signalling can support.

MeasParametersFRX-Diff field descriptions

csi-RS-RLM R1 1-7: CSI-RS based RLM

csi-RSRP-AndRSRQ-MeasWithoutSSB R1 1-5a: CSI-RS based RRM measurement without associated SS-block

csi-RSRP-AndRSRQ-MeasWithSSB R1 1-5: CSI-RS based RRM measurement with associated SS-block

csi-SINR-Meas R1 1-6: CSI-RS based SINR measurement

ss-SINR-Meas R1 1-3: SSB based SINR measurement

MIMO-ParametersPerBand field descriptions

aperiodicBeamReport R1 2-22: Aperiodic beam report on PUSCH

beamCorrespondence R1 2-20: Beam correspondence

beamManagementSSB-CSI-RS R1 2-24: SSB/CSI-RS for beam management

groupBeamReporting R1 2-29a: Group based beam reporting

lowLatencyCSI-Feedback R1 2-57: Support low latency CSI feedback

maxNumberConfiguredTCIstates R1 2-4: TCI states for PDSCH

maxNumberCSI-RS-BFR R1 2-31: Beam failure recovery

maxNumberMIMO-LayersCB-PUSCH R1 2-14: Codebook based PUSCH MIMO transmission. Absence of this field implies that CB-based PUSCH is not supported.

maxNumberMIMO-LayersNonCB-PUSCH R1 2-15: Non-codebook based PUSCH MIMO transmission. Absence of this field implies that Non-CB-based PUSCH is not supported.

maxNumberMIMO-LayersPDSCH R1 2-3: PDSCH MIMO layers. Absence of this field implies support of one layer.

maxNumberNonGroupBeamReporting R1 2-29: Non-group based beam reporting

maxNumberRxBeam R1 2-26: Receiving beam selection using CSI-RS resource repetition "ON"

maxNumberRxTxBeamSwitchDL R1 2-27: Beam switching (including SSB and CSI-RS)

maxNumberSimultaneousSRS-PerCC R1 2-54a: Simultaneous SRS Tx

periodicBeamReport R1 2-21: Periodic beam report on PUCCH

pusch-TransCoherence R1 2-13: PUSCH transmission coherence

sp-BeamReportPUCCH R1 2-23: Semi-persistent beam report on PUCCH

sp-BeamReportPUSCH R1 2-23a: Semi-persistent beam report on PUSCH

srs-TxSwitch R1 2-55: SRS Tx switch

supportedSRS-Resources R1 2-53: SRS resources

timeDurationForQCL R1 2-2: PDSCH beam switching

twoPortsPTRS R1 2-45 & 2-48: 2 ports of DL/UL PTRS

uplinkBeamManagement R1 2-30: UL beam management

Phy-ParametersCommon field descriptions

bwp-SwitchingDelay R4 1-8: BWP switching delay

csi-RS-CFRA-ForHO R1 1-9: CSI-RS based CFRA for HO

cbg-FlushIndication-DL R1 5-23: CBGFI for CBG-based re-transmission for DL

cbg-TransIndication R1 5-22 & 5-25: CBG-based re-transmission for DL/UL using CBGTI

configuredUL-GrantType1 R1 5-19: Type 1 Configured UL grant

configuredUL-GrantType2 R1 5-20: Type 2 Configured UL grant

downlinkSPS R1 5-18: DL SPS

dynamicBetaOffsetInd-HARQ-ACK-CSI R1 4-21: Dynamic beta-offset configuration and indication for HARQ-ACK and/or CSI

dynamicHARQ-ACK-CodeB-CBG-Retx-DL R1 5-24: Dynamic HARQ-ACK codebook using sub-codebooks for CBG-based re-transmission for DL

dynamicHARQ-ACK-Codebook R1 4-10: Dynamic HARQ-ACK codebook

dynamicPRB-BundlingDL R1 2-11: Downlink dynamic PRB bundling (DL)

dynamicSFI R1 3-6: Dynamic SFI monitoring and dynamic UL/DL determination

dynamicSwitchRA-Type0-1-PDSCH R1 5-3: Dynamic switching between RA type 0 and RA type 1 for PDSCH

dynamicSwitchRA-Type0-1-PUSCH R1 5-4: Dynamic switching between RA type 0 andRA type 1 for PUSCH

interleavingVRB-ToPRB-PDSCH R1 5-7: Interleaving for VRB-to-PRB mapping for PDSCH

interleavingVRB-ToPRB-PUSCH R1 5-8: Interleaving for VRB-to-PRB mapping for PUSCH

interSlotFreqHopping-PUSCH R1 5-10: Inter-slot frequency hopping for PUSCH

multipleCORESET R1 3-3: More than one CORESET per BWP (in addition to CORESET #0)

nzp-CSI-RS-IntefMgmt R1 2-34: NZP-CSI-RS based interference measurement

pdsch-MappingTypeA R1 5-6: PDSCH mapping type A with less than 7 OFDM symbols

pdsch-MappingTypeB R1 5-6a: PDSCH mapping type B

pdsch-RepetitionMultiSlots R1 5-17a: PDSCH repetitions over multiple slots

pre-EmptIndication-DL R1 5-21: Pre-emption indication for DL

precoderGranularityCORESET R1 3-7: Precoder-granularity of CORESET size

pucch-Repetition-F1-3-4 R1 4-23: Repetitions for PUCCH format 1, 3,and 4 over multiple slots with K = 1, 2, 4, 8

pusch-RepetitionMultiSlots R1 5-17: PUSCH repetitions over multiple slots

ra-Type0-PUSCH R1 5-2: RA type 0 for PUSCH

rateMatchingLTE-CRS R1 5-28: Rate-matching around LTE CRS

rateMatchingResrcSetDynamic R1 5-27: Dynamic rate-matching resource set configuration for DL

rateMatchingResrcSetSemi-Static R1 5-26: Semi-static rate-matching resource set configuration for DL

semiStaticHARQ-ACK-Codebook R1 4-11: Semi-static HARQ-ACK codebook

spatialBundlingHARQ-ACK R1 4-12: HARQ-ACK spatial bundling for PUCCH or PUSCH per PUCCH group

sp-CSI-ReportPUCCH R1 2-32a: Semi-persistent CSI report on PUCCH

sp-CSI-ReportPUSCH R1 2-32b: Semi-persistent CSI report on PUSCH

type1-PUSCH-RepetitionMultiSlots R1 5-14: Type 1 configured PUSCH repetitions over multiple slots

type1-PUSCH-RepetitionOneSlot R1 5-13: Type 1 configured PUSCH repetitions within a slot

type2-PUSCH-RepetitionMultiSlots R1 5-16: Type 2 configured PUSCH repetitions over multiple slots

type2-PUSCH-RepetitionOneSlot R1 5-15: Type 2 configured PUSCH repetitions within a slot

type2-SP-CSI-Feedback-LongPUCCH R1 2-42: Support Type II SP-CSI feedback on long PUCCH

Phy-ParametersFR1 field descriptions

pdcchMonitoringSingleOccasion R1 3-2: Unicast PDCCH monitoring following Case 1-2

pdsch-256QAM-FR1 R4 1-4: 256QAM for PDSCH in FR1

scs-60kHz R4 1-1: 60kHz of subcarrier spacing for FR1

Phy-ParametersFR2 field descriptions

calibrationGapPA R4 2-8: PA calibration gap

Phy-ParametersFRX-Diff field descriptions

absoluteTPC-Command R1 8-6: Absolute TPC command mode

almostContiguousCP-OFDM-UL R4 2-7: Almost contiguous UL CP-OFDM

csi-ReportWithCRI R1 2-39: CSI report with CRI

csi-ReportWithoutCQI R1 2-39a: CSI report without CQI

csi-ReportWithoutPMI R1 2-38: CSI report without PMI

freqHoppingPUCCH-F0-2 R1 4-6: Non-frequency hopping for PUCCH formats 0 and 2

freqHoppingPUCCH-F1-3-4 R1 4-7: Non-frequency hopping for PUCCH format 1, 3, and 4

intraSlotFreqHopping-PUSCH R1 5-9: Intra-slot frequency-hopping for PUSCH except for PUSCH scheduled by Type 1 before RRC connection

mux-SR-HARQ-ACK-CSI-PUCCH R1 4-19: SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or piggybacked on a PUSCH)

oneFL-DMRS-ThreeAdditionalDMRS R1 2-9 & 2-19: Support 1+3 DMRS (DL/UL)

oneFL-DMRS-TwoAdditionalDMRS R1 2-6 & 2-16b: Support 1+2 DMRS (DL/UL)

onePortsPTRS R1 2-44 & 2-47: 1 port of DL/UL PTRS

onePUCCH-LongAndShortFormat R1 4-22: 1 long PUCCH format and 1 short PUCCH format in the same slot

oneSymbolGP-TDD R4 1-9: 1-symbol GP in unpaired spectrum

pdcch-BlindDetectionCA R1 6-5a: PDCCH blind detection capability for CA

pucch-F2-WithFH R1 4-3: PUCCH format 2 over 1 – 2 OFDM symbols once per slot with FH

pucch-F3-4-HalfPi-BPSK R4 1-7: pi/2-BPSK for PUCCH format 3/4

pucch-F3-WithFH R1 4-4: PUCCH format 3 over 4 – 14 OFDM symbols once per slot with FH

pucch-F4-WithFH R1 4-5: PUCCH format 4 over 4 – 14 OFDM symbols once per slot with FH

pusch-HalfPi-BPSK R4 1-6: pi/2-BPSK for PUSCH

pusch-LBRM R1 5-29: LBRM for PUSCH

semiOpenLoopCSI R1 2-37: Support Semi-open loop CSI

supportedDMRS-TypeDL R1 2-10: Support DMRS type (DL)

supportedDMRS-TypeUL R1 2-17: Support DMRS type (UL)

tpc-PUCCH-RNTI R1 8-4: TPC-PUCCH-RNTI

tpc-PUSCH-RNTI R1 8-3: TPC-PUSCH-RNTI

tpc-SRS-RNTI R1 8-5: TPC-SRS-RNTI

twoDifferentTPC-Loop-PUCCH R1 8-8: UL power control with 2 PUCCH closed loops

twoDifferentTPC-Loop-PUSCH R1 8-7: UL power control with 2 PUSCH closed loops

twoFL-DMRS-TwoAdditionalDMRS R1 2-8 & 2-18a: Supported 2 symbols front-loaded +2 symbols additional DMRS(DL/UL)

twoFL-DMRS R1 2-7 & 2-18: Supported 2 symbols front-loaded DMRS(DL/UL)

twoPUCCH-AnyOthersInSlot R1 4-22a: 2 PUCCH transmissions in the same slot which are not covered by 4-22 and 4-2

twoPUCCH-F0-2-ConsecSymbols R1 4-2: 2 PUCCH of format 0 or 2 in consecutive symbols

uci-CodeBlockSegmentation R1 4-20: UCI code-block segmentation

Phy-ParametersXDD-Diff field descriptions

twoDifferentTPC-Loop-PUCCH R1 8-8: UL power control with 2 PUCCH closed loops

twoDifferentTPC-Loop-PUSCH R1 8-7: UL power control with 2 PUSCH closed loops

twoPUCCH-F0-2-ConsecSymbols R1 4-2: 2 PUCCH of format 0 or 2 in consecutive symbols