3GPP TSG RAN WG2 Meeting #102                               R1-1806976

Busan, Korea, May 21st – 25th, 2018

Source:	CATT

Title:	Initial self evaluation on User Plane latency

Agenda Item:	11.3

Document for:	Discussion and Decision

- Introduction

At RAN#75 meeting, new Study Item on Self Evaluation towards IMT-2020 submission was approved [1]. Self-Evaluation will provide the performance towards all the ITU-R IMT-2020 requirements as defined in Report ITU-R M.2410 [2]:

The minimum requirements for user plane latency are:

–	4 ms for eMBB

–	1 ms for URLLC

assuming unloaded conditions (i.e. a single user) for small IP packets (e.g. 0 byte payload + IP header), for both downlink and uplink

In this contribution, preliminary evaluation results of NR user plane latency are provided. The evaluation is applied to UE capability #1 with both PDSCH/PUSCH mapping Type A and Type B.

- User plane latency evaluation

  - User plane latency components

Based on the definition and evaluation methodology provided in Report ITU-R M.2412 [2], the procedure and components for user plane latency are listed in Table 1. Also notations are provided for those components.

Table 1: Procedure and components for user plane latency

| Item | Component | Notations |
| --- | --- | --- |
| 1 | Tx processing delay for data | tTx,data |
| 2 | Timing alignment delay | tFA It stands for the waiting time for the start of data transmission. |
| 3 | TTI for data packet transmission | tdata_duration |
| 4 | Rx processing delay for data | tRx_data |
|  | One shot transmission latency | tone_shot= tTx,data+ tFA + tdata_duration+ tRx_data |
| 5 | Rx processing delay for ACK/NACK | tRx_HARQ |
| 6 | ACK/NACK duration time for transmission | tHARQ_duration |
| 7 | Tx processing delay for ACK/NACK | tTx_HARQ |
| 8 | Tx processing delay for retransmission data | tTx,retr |
| 9 | Frame alignment delay | tFA |
| 10 | TTI for retransmission data packet transmission | tdata_duration |
| 11 | Rx processing delay for retransmission data | tRx_retr |
|  | HARQ RTT time | tHARQ_RTT = tRx_HARQ+ tHARQ_duration+ tTx_HARQ+ tTx,retr+ tFA+ tdata_duration+ tRx_retr |

With the one shot transmission latency and HARQ RTT time calculated in Table 1, user plane latency can be obtained for transmission with target error probability p where P is assumed as 0.1:

tp_HARQ= tone_shot + p* tHARQ_RTT.                          (1)

  - Processing time considerations and assumptions

The PDSCH and PUSCH processing procedure time (denoted ad tPDSCH and tPUSCH) from UE capability#1 with PDSCH/PUSCH mapping type A and B are defined in section 5.3 and 6.4 of TS38.214 [3] respectively.

For evaluation simplicity, it is assumed the processing time at gNB is equal to the corresponding processing time at UE side in the following preliminary evaluation, i.e. processing time to prepare DL data (re)transmission plus NACK receiving processing delay (when there is HARQ assumed) at gNB is equal to the time to prepare UL data (re)transmission plus receiving schedule for retransmission (when there is HARQ assumed) and vice versa. In practice, the processing time at eNB side could be less than that at UE side, which can be further discussed for self evaluation. Also for calculation, processing on data channel (Tx or Rx) is assumed equal to the rest processing time, e.g. preparation of NACK.

Waiting time for timing alignment delay is to average all arrival cases (also considering starting time) with the number of symbols to align Tx/Rx according to PDSCH/PUSCH. Actually the waiting delay for e.g. NACK at UE side may not be the same as PUSCH, which could be updated in future.

Proposal 1: Assuming gNB processing delay for NACK reception and DL data (re)transmission preparation is equal to/less than UE PUSCH preparation time[3], and gNB processing delay for PUSCH data reception and preparation for re-transmission schedule is equal to/less than UE PDSCH procedure time[3] in self evaluation.

Proposal 2: Waiting time for timing alignment delay is to average all arrival cases (also considering starting time) with the number of symbols to align Tx/Rx timing.

- User plane latency evaluation results

In this section, preliminary evaluation results of NR user plane latency for UE capability #1 are provided. Both FDD and TDD mode are considered.

Considering the valid starting symbol S and the number of consecutive symbols L combinations for PDSCH/PUSCH mapping type A and B defined in TS38.214 (as shown in Table 2 and Table 3), 2/4/7-os mini-slot for type B and 3/4/7/14-os mini-slot/slot for type A can be evaluated for DL user plane latency evaluation. For UL user plane latency evaluation, 2/4/7/14-os mini-slot/slot for type B and 4/7/14-os mini-slot/slot for type A can be evaluated.

Table 2: Valid S and L combinations for DL

| PDSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | {0,1,2,3} | {3,…,14} | {3,…,14} | {0,1,2,3} | {3,…,12} | {3,…,12} |
| Type B | {0,…,12} | {2,4,7} | {2,…,14} | {0,…,10} | {2,4,6} | {2,…,12} |

Table 3: Valid S and L combinations for UL

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} | {0,…,12} | {1,…,12} | {1,…,12} |

Proposal 3: For DL user plane latency evaluation, 2/4/7-os mini-slot for PDSCH mapping type B and 3/4/7/14-os mini-slot/slot for type A can be evaluated. For UL user plane latency evaluation, 2/4/7/14-os mini-slot/slot for PUSCH mapping type B and 4/7/14-os mini-slot/slot for type A can be evaluated.

  - NR FDD

For FDD, the DL and UL user plane latency results for mapping type B and A are given in Table 4, Table 5, Table 6, and Table 7, respectively.

Table 4: DL user plane latency results for NR FDD (UE capability #1 and mapping type B)

| Slot / non-slot duration | Error probability | SCS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15kHz | 30kHz | 60kHz | 120kHz |
| 2 OS non-slot | p=0 | 0.93 | 0.54 | 0.43 | 0.29 |
|  | p=0.1 | 1.11 | 0.64 | 0.51 | 0.34 |
| 4 OS non-slot | p=0 | 1.10 | 0.62 | 0.47 | 0.31 |
|  | p=0.1 | 1.29 | 0.73 | 0.56 | 0.37 |
| 7 OS non-slot | p=0 | 1.29 | 0.71 | 0.52 | 0.33 |
|  | p=0.1 | 1.49 | 0.83 | 0.61 | 0.39 |

Table 5: UL user plane latency results for NR FDD (UE capability #1 and mapping type B)

| Slot / non-slot duration | Error probability | SCS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15kHz | 30kHz | 60kHz | 120kHz |
| 2 OS non-slot | p=0 | 0.93 | 0.54 | 0.43 | 0.29 |
|  | p=0.1 | 1.11 | 0.64 | 0.51 | 0.34 |
| 4 OS non-slot | p=0 | 1.10 | 0.62 | 0.47 | 0.31 |
|  | p=0.1 | 1.29 | 0.73 | 0.56 | 0.37 |
| 7 OS non-slot | p=0 | 1.29 | 0.71 | 0.52 | 0.33 |
|  | p=0.1 | 1.49 | 0.83 | 0.61 | 0.39 |
| 14 OS slot | p=0 | 2.14 | 1.14 | 0.73 | 0.44 |
|  | p=0.1 | 2.47 | 1.32 | 0.85 | 0.51 |

Table 6: DL user plane latency results for NR FDD (UE capability #1 and mapping type A)

| Slot / non-slot duration | Error probability | SCS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15kHz | 30kHz | 60kHz | 120kHz |
| 3 OS non-slot | p=0 | 1.32 | 0.73 | 0.53 | 0.33 |
|  | p=0.1 | 1.58 | 0.87 | 0.63 | 0.40 |
| 4 OS non-slot | p=0 | 1.35 | 0.75 | 0.53 | 0.34 |
|  | p=0.1 | 1.61 | 0.89 | 0.64 | 0.41 |
| 7 OS non-slot | p=0 | 1.46 | 0.80 | 0.56 | 0.35 |
|  | p=0.1 | 1.72 | 0.95 | 0.67 | 0.42 |
| 14 OS slot | p=0 | 2.14 | 1.14 | 0.73 | 0.44 |
|  | p=0.1 | 2.47 | 1.32 | 0.85 | 0.51 |

Table 7: UL user plane latency results for NR FDD (UE capability #1 and mapping type A)

| Slot / non-slot duration | Error probability | SCS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15kHz | 30kHz | 60kHz | 120kHz |
| 4 OS non-slot | p=0 | 1.54 | 0.84 | 0.58 | 0.36 |
|  | p=0.1 | 1.80 | 0.98 | 0.68 | 0.43 |
| 7 OS non-slot | p=0 | 1.64 | 0.89 | 0.61 | 0.38 |
|  | p=0.1 | 1.90 | 1.04 | 0.71 | 0.44 |
| 14 OS slot | p=0 | 2.14 | 1.14 | 0.73 | 0.44 |
|  | p=0.1 | 2.47 | 1.32 | 0.85 | 0.51 |

Observation 1: PDSCH/PUSCH mapping type B will result in lower latency than mapping type A for NR FDD non-slot transmission.

Observation 2: For PDSCH/PUSCH mapping type A and B

-NR FDD can fulfil user plane latency requirement of IMT-2020 for eMBB with UE capability #1 under all types of SCS.

- NR FDD can fulfil user plane latency requirement of IMT-2020 for URLLC with UE capability #1 under SCS of 30 kHz, 60kHz and 120 kHz.

  - NR TDD

For NR TDD, two DL/UL configurations are considered:

Configuration 1. DDSUU

Configuration 2. DU.

    - TDD configuration 1

For the “S” slot in UL/DL configuration 1 with DDSUU structure, slot format with 11 DL symbols and 3 UL symbols are assumed. The DL and UL user plane latency results for mapping type B and A are given in Table 8, Table 9, Table 10 and Table 11, respectively.

Table 8: DL user plane latency results for NR TDD-DDSUU (UE capability #1 and mapping type B)

| Slot / non-slot duration | Error probability | SCS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15kHz | 30kHz | 60kHz | 120kHz |
| 2 OS non-slot | p=0 | 1.37 | 0.76 | 0.54 | 0.34 |
|  | p=0.1 | 1.68 | 0.92 | 0.66 | 0.41 |
| 4 OS non-slot | p=0 | 1.59 | 0.87 | 0.59 | 0.37 |
|  | p=0.1 | 1.91 | 1.04 | 0.71 | 0.44 |
| 7 OS non-slot | p=0 | 1.83 | 0.99 | 0.65 | 0.40 |
|  | p=0.1 | 2.17 | 1.17 | 0.78 | 0.47 |

Table 9: UL user plane latency results for NR TDD-DDSUU (UE capability #1 and mapping type B)

| Slot / non-slot duration | Error probability | SCS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15kHz | 30kHz | 60kHz | 120kHz |
| 2 OS non-slot | p=0 | 1.77 | 0.96 | 0.64 | 0.39 |
|  | p=0.1 | 2.07 | 1.12 | 0.75 | 0.46 |
| 4 OS non-slot | p=0 | 2.53 | 1.34 | 0.83 | 0.49 |
|  | p=0.1 | 2.92 | 1.54 | 0.97 | 0.57 |
| 7 OS non-slot | p=0 | 2.80 | 1.47 | 0.90 | 0.52 |
|  | p=0.1 | 3.21 | 1.69 | 1.04 | 0.61 |
| 14 OS slot | p=0 | 3.74 | 1.94 | 1.13 | 0.64 |
|  | p=0.1 | 4.35 | 2.26 | 1.32 | 0.75 |

Table 10: DL user plane latency results for NR TDD-DDSUU (UE capability #1 and mapping type A)

| Slot / non-slot duration | Error probability | SCS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15kHz | 30kHz | 60kHz | 120kHz |
| 3 OS non-slot | p=0 | 1.90 | 1.02 | 0.67 | 0.41 |
|  | p=0.1 | 2.38 | 1.28 | 0.83 | 0.50 |
| 4 OS non-slot | p=0 | 1.94 | 1.04 | 0.68 | 0.41 |
|  | p=0.1 | 2.42 | 1.30 | 0.84 | 0.51 |
| 7 OS non-slot | p=0 | 2.05 | 1.09 | 0.71 | 0.43 |
|  | p=0.1 | 2.53 | 1.35 | 0.87 | 0.52 |
| 14 OS slot | p=0 | 3.34 | 1.74 | 1.03 | 0.59 |
|  | p=0.1 | 3.95 | 2.06 | 1.22 | 0.70 |

Table 11: UL user plane latency results for NR TDD-DDSUU (UE capability #1 and mapping type A)

| Slot / non-slot duration | Error probability | SCS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15kHz | 30kHz | 60kHz | 120kHz |
| 4 OS non-slot | p=0 | 3.14 | 1.64 | 0.98 | 0.56 |
|  | p=0.1 | 3.61 | 1.89 | 1.14 | 0.66 |
| 7 OS non-slot | p=0 | 3.24 | 1.69 | 1.01 | 0.58 |
|  | p=0.1 | 3.72 | 1.95 | 1.17 | 0.67 |
| 14 OS slot | p=0 | 3.74 | 1.94 | 1.13 | 0.64 |
|  | p=0.1 | 4.35 | 2.26 | 1.32 | 0.75 |

Observation 3: PDSCH/PUSCH mapping type B will result in lower latency than mapping type A for NR TDD non-slot transmission.

Observation 4: For PDSCH/PUSCH mapping type A and B

- NR TDD (DDSUU) can fulfil user plane latency requirement of IMT-2020 for eMBB with UE capability #1 under all types of SCS.

-NR TDD (DDSUU) can fulfil user plane latency requirement of IMT-2020 for URLLC with UE capability #1 under SCS of 120 kHz for type A, while 60kHz and 120kHz for type B.

    - TDD configuration 2

Another DL/UL configuration DU is also considered. The DL and UL user plane latency results for mapping type B and A are given in Table 12, Table 13, Table 14 and Table 15, respectively.

Table 12: DL user plane latency results for NR TDD-DU (UE capability #1 and mapping type B)

| Slot / non-slot duration | Error probability | SCS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15kHz | 30kHz | 60kHz | 120kHz |
| 2 OS non-slot | p=0 | 1.23 | 0.69 | 0.51 | 0.32 |
|  | p=0.1 | 1.47 | 0.82 | 0.60 | 0.39 |
| 4 OS non-slot | p=0 | 1.46 | 0.80 | 0.56 | 0.35 |
|  | p=0.1 | 1.73 | 0.95 | 0.67 | 0.42 |
| 7 OS non-slot | p=0 | 1.71 | 0.93 | 0.63 | 0.38 |
|  | p=0.1 | 2.01 | 1.09 | 0.74 | 0.45 |

Table 13: UL user plane latency results for NR TDD-DU (UE capability #1 and mapping type B)

| Slot / non-slot duration | Error probability | SCS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15kHz | 30kHz | 60kHz | 120kHz |
| 2 OS non-slot | p=0 | 1.23 | 0.69 | 0.51 | 0.32 |
|  | p=0.1 | 1.47 | 0.82 | 0.60 | 0.39 |
| 4 OS non-slot | p=0 | 1.46 | 0.80 | 0.56 | 0.35 |
|  | p=0.1 | 1.73 | 0.95 | 0.67 | 0.42 |
| 7 OS non-slot | p=0 | 1.71 | 0.93 | 0.63 | 0.38 |
|  | p=0.1 | 2.01 | 1.09 | 0.74 | 0.45 |
| 14 OS slot | p=0 | 2.64 | 1.39 | 0.86 | 0.50 |
|  | p=0.1 | 3.07 | 1.62 | 1.00 | 0.59 |

Table 14: DL user plane latency results for NR TDD-DU (UE capability #1 and mapping type A)

| Slot / non-slot duration | Error probability | SCS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15kHz | 30kHz | 60kHz | 120kHz |
| 3 OS non-slot | p=0 | 1.80 | 0.97 | 0.65 | 0.39 |
|  | p=0.1 | 2.16 | 1.17 | 0.78 | 0.47 |
| 4 OS non-slot | p=0 | 1.84 | 0.99 | 0.66 | 0.40 |
|  | p=0.1 | 2.20 | 1.18 | 0.78 | 0.48 |
| 7 OS non-slot | p=0 | 1.94 | 1.04 | 0.68 | 0.41 |
|  | p=0.1 | 2.30 | 1.24 | 0.81 | 0.49 |
| 14 OS non-slot | p=0 | 2.64 | 1.39 | 0.86 | 0.50 |
|  | p=0.1 | 3.07 | 1.62 | 1.00 | 0.59 |

Table 15: UL user plane latency results for NR TDD-DU (UE capability #1 and mapping type A)

| Slot / non-slot duration | Error probability | SCS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15kHz | 30kHz | 60kHz | 120kHz |
| 4 OS non-slot | p=0 | 2.04 | 1.09 | 0.71 | 0.42 |
|  | p=0.1 | 2.39 | 1.28 | 0.83 | 0.50 |
| 7 OS non-slot | p=0 | 2.14 | 1.14 | 0.73 | 0.44 |
|  | p=0.1 | 2.50 | 1.34 | 0.86 | 0.52 |
| 14 OS slot | p=0 | 2.64 | 1.39 | 0.86 | 0.50 |
|  | p=0.1 | 3.07 | 1.62 | 1.00 | 0.59 |

Observation 5: For PDSCH/PUSCH mapping type A and B

-NR TDD (DU) can fulfil user plane latency requirement of IMT-2020 for eMBB with UE capability #1 under all types of SCS.

-NR TDD (DU) can fulfil user plane latency requirement of IMT-2020 for URLLC with UE capability #1 under SCS of 30 kHz, 60kHz and 120 kHz for type B.

- Conclusions

In this contribution, user plane latency evaluation procedure is discussed and we have the following proposal:

Proposal 1: Assuming gNB processing delay for NACK reception and DL data re-transmission preparation is equal to/less than PUSCH preparation time[3], and gNB processing delay for PUSCH data reception and preparation for re-transmission schedule is equal to/less than PDSCH procedure time[3] in self evaluation.

Proposal 2: Waiting time for timing alignment delay is to average all arrival cases (also considering starting time) with the number of symbols to align Tx/Rx timing.

Proposal 3: For DL user plane latency evaluation, 2/4/7-os mini-slot for PDSCH mapping type B and 3/4/7/14-os mini-slot/slot for type A can be evaluated. For UL user plane latency evaluation, 2/4/7/14-os mini-slot/slot for PUSCH mapping type B and 4/7/14-os mini-slot/slot for type A can be evaluated.

Preliminary evaluation results of NR user plane latency are provided. The evaluation is applied to UE capability #1 with both PDSCH/PUSCH mapping Type A and Type B. The following observations are made:

Observation 1: PDSCH/PUSCH mapping type B will result in lower latency than mapping type A for NR FDD non-slot transmission.

Observation 2: For PDSCH/PUSCH mapping type A and B

-NR FDD can fulfil user plane latency requirement of IMT-2020 for eMBB with UE capability #1 under all types of SCS

- NR FDD can fulfil user plane latency requirement of IMT-2020 for URLLC with UE capability #1 under SCS of 30 kHz, 60kHz and 120 kHz

Observation 3: PDSCH/PUSCH mapping type B will result in lower latency than mapping type A for NR TDD non-slot transmission.

Observation 4: For PDSCH/PUSCH mapping type A and B

- NR TDD (DDSUU) can fulfil user plane latency requirement of IMT-2020 for eMBB with UE capability #1 under all types of SCS.

-NR TDD (DDSUU) can fulfil user plane latency requirement of IMT-2020 for URLLC with UE capability #1 under SCS of 120 kHz for type A, while 60kHz and 120kHz for type B.

Observation 5: For PDSCH/PUSCH mapping type A and B

-NR TDD (DU) can fulfil user plane latency requirement of IMT-2020 for eMBB with UE capability #1 under all types of SCS.

-NR TDD (DU) can fulfil user plane latency requirement of IMT-2020 for URLLC with UE capability #1 under SCS of 30 kHz, 60kHz and 120 kHz for type B.

- References

- RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- R1-1805796, “draft CR to 38.214 capturing the RAN1#92bis meeting agreements”, April. 2018.