3GPP TSG-RAN WG2 Meeting #102	R2-1807126

Busan, South Korea, 21st - 25th May 2018	R2-18xxxxx

Agenda item:	11.2

Source:	Nokia, Nokia Shanghai Bell

Title:	RLM/RLF measurement on NR-U

WID/SID:	FS_NR_unlic – Release 16

Document for:	Discussion and Decision

# 1	Introduction

During RAN plenary #78, the release 15 NR specifications supporting licensed band operation were approved. Before that a NR Study Item [1] dealing with NR-based access to unlicensed spectrum has been approved in RAN plenary #75. One objective is to:

- Study NR-based operation in unlicensed spectrum (RAN1, RAN2, RAN4) including

  - Physical channels inheriting the choices of duplex mode, waveform, carrier bandwidth, subcarrier spacing, frame structure, and physical layer design made as part of the NR study and avoiding unnecessary divergence with decisions made in the NR WI

    - Consider unlicensed bands both below and above 6GHz, up to 52.6GHz

    - Consider unlicensed bands above 52.6GHz to the extent that waveform design principles remain unchanged with respect to below 52.6GHz bands

    - Consider similar forward compatibility principles made in the NR WI

  - Initial access, channel access. Scheduling/HARQ, and mobility including connected/inactive/idle mode operation and radio-link monitoring/failure

In this contribution, we will discuss radio-link monitoring/failure (RLM/RLF) framework in NR-U, highlight the potential challenges of RLM/RLF handling in NR-U, and also give some possible solutions to overcome the challenges.

# 2	Discussion

For the purpose of monitoring downlink radio link quality to indicate out-of-sync/in-sync status indications to higher layers in LTE, the UE shall measure the reference signal (RS) and compare it to the thresholds Qout and Qin [2]. When the estimated downlink radio link quality becomes worse than threshold Qout, the physical layer in the UE shall send an out-of-sync (OOS) indication to higher layers. When the estimated downlink radio link quality becomes better than threshold Qin, the physical layer in the UE shall send an in-sync (IS) indication to higher layers. After reception of enough consecutive OOS indications, a timer is started. If no enough consecutive IS indications are received before the timer expiry, the radio link failure (RLF) is declared.

In NR licensed spectrum, both Synchronization Signal Block (SSB) and Channel Status Information Reference Signal (CSI-RS) can be used as RLM reference signals [3]. Both RLM-RS resources can be transmitted periodically at specific time instants. The UE shall monitor the downlink quality based on the SSB and/or CSI-RS, and assess the downlink radio link quality per every evaluation period. However, in NR unlicensed spectrum, the periodical transmission of RLM-RS may be interrupted due to LBT failure as shown in Fig.1. Once the gNB misses the RS transmission due to the failed LBT, it must wait until the next transmission opportunity. More seriously, the gNB may fail to access the channel for several consecutive periods. In this case, the UE may not exactly know the radio link status because the UE cannot distinguish whether the radio quality is poor or the reference signals are not transmitting due to LBT failure. In this case, if the UE always reports OOS indication to higher layer even if the channel condition is good, the inaccurate measurement result may lead to unnecessary RLF. On the other hand, if the UE always doesn’t report OOS indications when it cannot detect any of its configured RLM-RS resources, this will result in unacceptable delays in declaring RLF. Therefore, the RLM/RLF mechanism may be impacted by the possible lack of reference signals due to LBT.

- Observation 1: the RLM/RLF measurement and report may be impacted by the transmission uncertainty of the RLM reference signals due to LBT.

Figure1: Legacy periodical RLM-RS transmission in NR-U

- In order to improve the success probability of the RLM-RS transmission on NR-U, the RLM-RS transmission opportunity within one period shall be increased. One possible solution is to use a specific reference signal, like the Discovery Reference Signal (DRS) in LAA, for radio link monitoring purposes. Such reference signal is transmitted in a predictable timing configuration, here referred to as the DRS transmission timing configuration (DTTC). The presence of DRS in unlicensed spectrum can be expected because the gNB would always attempt to transmit the DRS with multiple transmission opportunities in the DTTC; on the other hand, high-priority LBT defined for DRS in LAA may also be used to improve the success probability of DRS transmission. The DRS in NR-U may consist of SSB and optionally CSI-RS resources for the purpose of RLM. The RAN2 RLM measurement and reporting relies on the exact structure of the DRS signal and the LBT design for DRS signal transmission in NR-U, which can be studied by RAN1.

- Proposal 1: DRS-like signal in one configured window (DRS transmission timing configuration, DTTC) should be assumed for radio link monitoring purposes.

As mentioned above, measurements on DRS signals within the DTTC window are more reliable than the periodical RLM-RS (i.e., SSB or CSI-RS) outside of the DTTC window. Therefore, L1 samples within the DTTC window should primarily be used for RLM purposes - including both out-of-sync and in-sync evaluations.

Proposal 2: L1 samples based on DRS transmission within the DTTC window are used for both in-sync and out-of-sync evaluations.

In case the UE cannot detect any DRS transmission during the DTTC window, it cannot distinguish whether this is due to poor link quality or because the DRS transmission is block by LBT. Therefore, the event of the UE not detecting any DRS transmissions within the DTTC window should also be counted in the out-of-synch evaluations. How to count the missing DRS samples within the DTTC window in the out-of-synch evaluations is for RAN4 to be discussed.

Proposal 3: The event of the UE not detecting a DRS transmission within the DTTC window is also counted in the out-of-synch evaluations. How to count the missing DRS samples within the DTTC window in the out-of-synch evaluations is for RAN4 to be discussed.

As by proposal 3, LBT blocking within the DTTC window may result in unnecessary declaration of RLF. Though periodic RLM-RS transmission outside the DTTC window are less reliable that DRS transmissions within the DTTC, they can still be used for RLM purposes. To allow for faster RLM recovery in case DRS transmission within the DTTC is blocked, we propose that detected L1 samples outside the DTTC in correspondence of configured RLM-RS resources can also be used in the in-sync evaluations.

Proposal 4: L1 samples outside the DTTC window are also used for in-sync evaluations (upon detection of RLM-RS transmission from the gNB).

As by proposal 4, outside of the DTTC window the samples based on the detection of periodical RLM-RS (e.g. SSB or CSI-RS) can be used for the in-sync evaluations. It seems that the number of detected RLM-RS signals, which relies on the RLM-RS periodicity and the success probability of LBT, may impact the in-sync evaluation performance. The RLM-RS signals used for in-sync evaluations might not have sufficient density when the periodical RLM-RS signal could not always be transmitted periodically at the configured time resource due to the unsuccessful channel access. Further enhancements for RLM-RS signal outside of the DTTC window need to be investigated, especially for long periodicity of RLM-RS resources.

Proposal 5: For RLM measurements outside the DTTC, further enhancements should consider the impact of LBT.

In NR, the UE shall be able to evaluate the downlink quality on the configured RLM-RS resources over a certain evaluation period, which is based on RLM-RS period and the number of RLM-RS samples. However, in NR-U, considering that the DRS signal may be introduced for RLM as proposed above, the evaluation period may be redefined in terms of DTTC period and the number of DRS samples. Lower number of DRS samples will impact the accuracy of RLM. However, for a given DTTC period, the evaluation period increases as the number of DRS samples increases. The larger evaluation period may result in slow response times in the L1 sync evaluation. Therefore, the detailed number of DRS samples under various DTTC periods needs to be specified after evaluation.

Proposal 6: The definition of evaluation period should depend on the DTTC period and the number of DRS samples. The specific number of DRS samples under various DTTC periods needs to be evaluated.

# 3	Conclusion

The proposals and observations made in this contribution are summarized below.

- Observation 1: the RLM/RLF measurement and report may be impacted by the transmission uncertainty of the RLM reference signals due to LBT.

- Proposal 1: DRS-like signal in one configured window (DRS transmission timing configuration, DTTC) should be assumed for radio link monitoring purposes.

Proposal 2: L1 samples based on DRS transmission within the DTTC window are used for both in-sync and out-of-sync evaluations.

Proposal 3: The event of the UE not detecting a DRS transmission within the DTTC window is also counted in the out-of-synch evaluations. How to count the missing DRS samples within the DTTC window in the out-of-synch evaluations is for RAN4 to be discussed.

Proposal 4: L1 samples outside the DTTC window are also used for in-sync evaluations (upon detection of RLM-RS transmission from the gNB).

Proposal 5: For RLM measurements outside the DTTC, further enhancements should consider the impact of LBT.

Proposal 6: The definition of evaluation period should depend on the DTTC period and the number of DRS samples. The specific number of DRS samples under various DTTC periods needs to be evaluated.

# References

- RP-172021, “New SID on NR-based Access to Unlicensed Spectrum”, Qualcomm

- 3GPP TS 36.133 V15.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management”

- Final Report of 3GPP TSG RAN WG1 #91