3GPP TSG-RAN WG2 Meeting #102	R2-1807172

Busan, South Korea, 21st - 25th May 2018

Agenda item:	10.4.1.3.3

Source:	Nokia, Nokia Shanghai Bell

Title:	NR RRC signalling for slice assistance information with TP

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

This contribution provides stage 3 details of the slice assistance information and text proposal on signalling NSSAI to the running CR to TS38.331.

# 2	Discussion

There have been three complementary NR agreements that resolve detailed implementation of the NSSAI signaling in TS38.331:

### 2.1 S-NSSAI format

- RAN2#101bis agreed CR to TS38.300 in [1] defining the following contents of NSSAI:

The slice assistance information consists of one or a list of S-NSSAIs, where an S-NSSAI is a combination of:

-	mandatory SST (Slice/Service Type) field, which identifies the slice type and consists of 8 bits (with range is 0-255);

-	optional SD (Slice Differentiator) field, which differentiates among Slices with same SST field and consist of 24 bits.

Proposal 1: RRCSetupComplete message type supports conveying the list of S-NSSAIs, with ability to signal each S-NSSAI with mandatory SST field and optionally included SD field.

Proposal 2: UE actions upon reception of RRCSetup message type support provision of NSSAI (when the associated procedures are mature in TS38.331)

### 2.2 S-NSSAI support by RRC_INACTIVE

In the response to RAN2#101 point left open:

FFS Whether the NSSAI info needs to be included in MSG5 in the case of resume.

SA2 clarified that inclusion of NSSAI in MSG5 for RRC_INACTIVE is not required. The rationale was, that if RRC Connection Resume procedure (MSG3) is sent, retrieval of the context in the anchor NG-RAN node is triggered and the UE needs not pass NSSAI to the NG-RAN node in MSG5 as the Allowed NSSAI is already in the context it obtains from the anchor RAN node (see [3]).

Proposal 3: RRCResumeComplete message type does not support provision of NSSAI.

### 2.3 Slicing support

SA2#126 agreed that network slicing is mandatory [4]:

-	The 5G System deployed in a PLMN shall always support the procedures, information and configurations specified for network slicing in the present document, TS 23.502 and TS 23.503.

The UE capable of 5G System shall support network slicing.

Proposal 4: NSSAI support is mandatory (based on provision by NAS).

# 3	Conclusion

Proposal 1: RRCSetupComplete message type supports conveying the list of S-NSSAIs, with ability to signal each S-NSSAI with mandatory SST field and optionally included SD field.

Proposal 2: UE actions upon reception of RRCSetup message type support provision of NSSAI (when the associated procedures are mature in TS38.331)

Proposal 3: RRCResumeComplete message type does not support provision of NSSAI.

Proposal 4: NSSAI support is mandatory (based on provision by NAS).

Proposal 5: Agree Text Proposal in the appended Annex.

# References

- R2-1806385, Slicing assistance information, Nokia, Nokia Shanghai Bell

- S2-184498, Reply LS on clarification of NSSAI information exchange during Inactive to connected mode transition, SA2

- S2-184219, Output of NSSAI handling in Access Stratum drafting sessions, SA2

- S2-182549, 5GS support for network slicing, Telecom Italia, China Mobile, Nokia, Nokia Shanghai Bell, Orange, AT&T, SK Telecom, Verizon, Deutsche Telekom, Charter Communications, Oracle

# Annex

Modified Subclause

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NR; Overall description; Stage 2".

[3]	3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

[4]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[5]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) protocol specification".

[6]	ITU-T Recommendation X.680 (08/2015) "Information Technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation" (Same as the ISO/IEC International Standard 8824-1).

[7]	ITU-T Recommendation X.681 (08/2015) "Information Technology - Abstract Syntax Notation One (ASN.1): Information object specification" (Same as the ISO/IEC International Standard 8824-2).

[8]	ITU-T Recommendation X.691 (08/2015) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[9]	3GPP TS 38.215: "NR; Physical layer measurements".

[10]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol Specification".

[11]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[14]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[15]	3GPP TS 38.101: "NR; User Equipment (UE) radio transmission and reception".

[16]	3GPP TS 38.211: "NR; Physical channels and modulation".

[17]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[18]	ITU-T Recommendation X.683 (08/2015) "Information Technology - Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 specifications" (Same as the ISO/IEC International Standard 8824-4).

[19]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[xx]	3GPP TS 23.003: "Numbering, addressing and identification".

Modified Subclause

### 6.3.4	Other information elements

#### –	S-NSSAI

The IE S-NSSAI is used to convey S-NSSAI (Single Network Slice Selection Assistance Information), see TS 23.003 [xx].

S-NSSAI information element

-- ASN1START

-- TAG-S-NSSAI-START

S-NSSAI ::=						SEQUENCE {

sst							BIT STRING (SIZE (8)),

sd							BIT STRING (SIZE (16))		OPTIONAL

}

-- TAG-S-NSSAI-START

-- ASN1STOP

S-NSSAI field descriptions

sst Slice/Service Type, see TS 23.003 [xx].

sd Slice Differentiator, see TS 23.003 [xx].

Next Modified Subclause

## 11.4	Inter-node RRC multiplicity and type constraint values

#### –	Multiplicity and type constraints definitions

-- ASN1START

-- TAG_NR-MULTIPLICITY-AND-CONSTRAINTS-START

maxMeasFreqsMN				INTEGER ::= 32	-- Maximum number of MN-configured measurement frequencies

maxCellPrep					INTEGER ::= 32	-- Maximum number of cells prepared for handover

maxNSSAI					INTEGER ::= 8	-- Maximum number of S-NSSAIs

-- TAG_NR-MULTIPLICITY-AND-CONSTRAINTS-STOP

-- ASN1STOP

End of Modified Subclause