3GPP TSG-RAN WG2 #102	R2-1807196

# Busan, Republic of Korea, 21st – 25th May 2018		 	  Revision of R2-1805434

Agenda Item:	10.3.2.4

Source:	Ericsson

Title:	Clarification of RLC poll handling for DC and duplication

Document for:	Discussion, Decision

# Introduction

In this contribution we discuss a remaining open issue regarding the RLC poll handling when no data for the logical channel is available anymore due to split bearer reconfiguration or PDCP duplication deactivation.

# Discussion

Packet duplication and switching between RLC bearers introduces additional use cases which require additional clarifications.

From current section 5.3.3.2, below, it is clear that a poll shall be set when at time of submission to lower layers it is clear that the RLC PDU is the last available for transmission (including higher layers). However, there may however be use cases when this is not the case:

#### 5.3.3.2	Transmission of a AMD PDU

…

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or

-	if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

-	include a poll in the AMD PDU as described below.

NOTE:	Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation.

In some instances, PDCP shall discontinue transmissions on one RLC entity even though PDCP PDUs are available for transmission. This applies for protocol architectures where a higher layer PDCP is associated with multiple lower layer RLC entities. These include:

- Dual Connectivity Split bearers

  - applies to when the transmission on the RLC entities are changed (for uplink transmission this is triggered by RRC signaling), leading to data not being available anymore for the old RLC entity. Similarly, in DL, F1-U procedures, this is triggered DL Flush or DL Block Discard.

  - applies to when a splitting threshold is configured and buffered data changes from more than the threshold to less than the threshold, leading to data not being available anymore for the secondary associated RLC.

- Duplication is configured in Carrier Aggregation and DC protocol architectures, and for where duplication becomes deactivated or PDUs discarded, leading to data not being available anymore for the secondary associated RLC entity where previously duplicates were sent.

The RLC entity may, when any of above situations occur already have emptied its buffer and because at the time the RLC PDU was sent the upper layer had data pending for this RLC entity, a poll was not included with the last RLC PDU.

In current specifications there is no support to transmit further RLC PDUs including a poll.

In such situations, the receiving RLC entity may have correctly received PDUs, but since no poll for RLC status report is sent, these RLC PDUs will not be acknowledged and the transmitting RLC entity will not know if the data was received or not.  When this RLC entity is re-used for transmission again, polls will be sent. If the old unacknowledged RLC PDU SN was correctly received it is now acknowledged to the transmitting RLC entity. If the old unacknowledged RLC PDU SN instead was incorrectly received, this requires RLC retransmission before it can be acknowledged.

The RLC PDU SN acknowledgement correspond to specific PDCP PDU SN. Due to continued data transmission on another RLC entity, the PDCP entity in the above described scenario has continued to transmit PDUs and may therefore have moved its PDCP SN window further. The acknowledge for the PDCP PDU SN related the old correctly received RLC PDU SN may therefore, when arriving in the CU, be outside the current PDCP RX SN window. This includes a risk that the PDCP HFN gets out-of-sync between the transmitter and the receiver, resulting in a dropped radio bearer. The RLC entity does not have all required information to conclude at what time an acknowledgement to PDCP will trigger a PDCP HFN out-of-sync which is why a solution must be robust to any delay.

38.323 do not cover all new RLC poll use cases when PDCP shall discontinue the transmission to one RLC entity suddenly

The specifications should therefore be updated with clarifications to cover also these use cases similarly to not setting the poll bit too frequently is captured. One potential solution would be to leave this to UE implementation and introduce NOTEs according to below:

Update 38.323 to cover use cases when PDCP turns off transmission to one RLC entity suddenly. This by adding below note in section 5.2.1 Transmit Operation. 

NOTE:
When there is no PDCP data for an associated RLC entity expected and the RLC entity is not released or reset, PDCP indicates this (i.e. no PDCP data) to the RLC entity at the time when the last PDCP PDU is submitted to the RLC entity

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Update 38.323 to cover use cases when PDCP turns off transmission to one RLC entity suddenly. This by adding below note in section 5.2.1 Transmit Operation.

NOTE: When there is no PDCP data for an associated RLC entity expected and the RLC entity is no released or reset, PDCP indicates this (i.e. no PDCP data) to the RLC entity at the time when the last PDCP PDU is submitted to the RLC entity