3GPP TSG-RAN WG2 #102	R2-1807197

# Busan, Republic of Korea, 21st – 25th May 2018

Agenda Item:	10.3.4.4

Source:	Ericsson

Title:	Maximum Data Burst Volume (MDBV)

Document for:	Discussion, Decision

# 1	Introduction

In the LS S2-184613, SA2 writes:

SA2 has specified delay critical 5QI(s) (with QoS characteristics including Maximum Data Burst Volume) as follows in TS 23.501:

- Maximum Data Burst Volume denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB (i.e. 5G-AN part of the PDB). The Maximum Data Burst Volume may be signalled with 5QIs to the (R)AN, and if it is not received, a standardized value applies (for standardized 5QIs the value in the QoS characteristics Table 5.7.4) applies.

For the QoS flows not exceeding the GFBR, SA2 has discussed the use cases where the UE has multiple QoS flows with delay critical 5QIs. SA2 expect that the PDB and PER requirements are satisfied in the UL and DL for all delay critical QoS flows that do not exceed the MDBV in the presence of any competing traffic. If other delay critical QoS Flows exceeds the MDBV, it is not required that for these QoS flows the PDB and PER targets are met.

SA2 would like to ask RAN2 to confirm the above behaviour is supported by the RAN specifications and whether any specific action is required in order to deliver predictable behaviour to compliant flows, in presence other delay critical QoS flows.

2. Actions:

To RAN2 group.

ACTION: 	SA2 respectfully asks RAN2 to confirm that the PDB and PER requirements are satisfied in the UL and DL for all delay critical QoS flows that do not exceed the MDBV, even if other delay critical QoS Flows (which may also be of higher priority) for the same UE or other UE exceeds the MDBV.

# 2	Discussion

## 2.1	Usage of MDBV in RAN

It should be clear that the MDBV parameter is not a requirement on RAN, but a limitation of the PDB and PER requirements. RAN is only required to fulfil the PDB and PER requirement for QoS Flows with bursts that are within the MDBV. In that aspect, it is similar to GFBR, which defines that PDB and PER is not required to be fulfilled for QoS Flows with bitrates above the GFBR.

RAN is expected to use the MDBV parameter for observability of fulfilment of QoS requirements. Also, in Admission Control it will be useful for RAN to know how large bursts it is required to serve without exceeding the PDB.

In the case when there is a high GBR load, and GBR flows with burst that exceed the MDBV may take resources needed by other QoS Flows, it is also useful to use the MDBV parameter in scheduling and prioritize QoS Flows that comply with the MDBV.

With current functionality it is possible to take MDBV into account for scheduling between all DL QoS Flows that are placed in different DRB’s, and for UL Flows to different UE’s. For the special use case described in the LS, where there are several Delay Critical GBR Flows to the same UE, there are several options to guarantee the PDB, as described in next section.

## 2.1	Methods to Guarantee PDB when there are several UL GBR Flows to same UE.

RAN have several options how to ensure that PDB is fulfilled even if there are competing GBR Flows with bursts larger than MDBV.

- Use different Logical Channels for which resources are restricted, e.g. Configured Grants, cell restrictions etc.

The GBR Flows are assigned such that the logical channel resource distribution is controlled by RAN which may prioritize Flows with bursts smaller than MDBV, in order to guarantee PDB for GBR Flows which comply with MDBV.

- Admission control

When considering requests for several UL GBR Flows to the same UE, new Flows are only admitted if PDB can be guaranteed even if other GBR Flows with same or higher priority exceed the MDBV.

- Specify Application behaviour in SLA (Service Level Agreement)

For most Delay Critical Applications, the service provider will have control of the terminal and application design, and it will be possible to control the behaviour of the applications. It is therefore possible for the operator to require that applications comply with MDBV if there are several UL delay critical GBR Flows to same UE, when writing agreements with service providers.

- Use PRB to prioritize QoS Flows that comply with MDBV.

As described in [1], the PBR parameter can be set to MDBV/PDB, and the Buffer Size Duration parameter can be set to PDB so that the UE will prioritize QoS Flows that comply with MDBV. The drawback of this method is that UE prioritization based on GFBR will not be possible, and QoS Flows that comply with MDBV will be prioritized in the UE, even if the bitrate exceeds GFBR.

Methods 1-3 can be used with current 5G standard, while method 4 can be used if new values are standardized for Bucket Size Duration. In release 15 timespan it is not expected that the load from GBR Flows with tight delay requirements will be high, and it is therefore our view that current functionality is sufficient.

RAN may place delay critical QoS Flows in different Logical Channels in order to control the resource distribution from each Flow. If there are no Logical Channels available (w.r.t restricted LCH resources), Admission Control may be used to limit the number of UL Delay Critical Flows from one UE.

As a result, we propose that:

RAN has sufficient functionality in current standard to ensure that RAN will be able to fulfil PDB and PER requirements for delay critical QoS flows.

A Reply LS is sent to SA2, stating the above as a confirmation of the question from SA2

# Conclusion

In the previous sections we made the following observation:

Observation 1	RAN may place delay critical QoS Flows in different Logical Channels in order to control the resource distribution from each Flow. If there are no Logical Channels available (w.r.t restricted LCH resources), Admission Control may be used to limit the number of UL Delay Critical Flows from one UE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN has sufficient functionality in current standard to ensure that RAN will be able to fulfil PDB and PER requirements for delay critical QoS flows.

Proposal 2	A Reply LS is sent to SA2, stating the above as a confirmation of the question from SA2

# References

R2-1804911, MDBV in Uplink, Nokia, Nokia Shanghai Bell, Sanya, April 2018