3GPP TSG-RAN WG2 #102	R2-1807200

# Busan, Republic of Korea, 21st – 25th May 2018

Agenda Item:	10.3.2.3

Source:	Ericsson, LG Electronics Inc.

Title:	Suspending RLC in duplication

Document for:	Discussion, Decision

# Introduction

In RAN2#101bis, the impact of PDCP duplication to RLC was discussed and in this contribution,  we discuss the remaining open issues on RLC failure handling in case of duplication.

# Discussion

An open issue in relation of duplication is how the RLC logical channel triggering maximum number of retransmissions reached should be considered. This had been discussed also in [1].

It is specified that RLC indicates the maximum number of retransmissions reached to higher layers. This leads to triggering RLF in case of RLC associated with PCell, SCG-RLF for RLC associated with PSCell in SCG, and (newly introduced in Rel-15) SCell-RLF in case of duplication and RLC associated with SCell.

It is noteworthy that this case previously lead in both RLF and SCG RLF to reset of the corresponding MAC entity (both or at least SCG-MAC). This is not the case in duplication with RLC associated with SCell, i.e. transmission of data via the RLC entity not reaching the maximum retransmissions continues, i.e. MAC must not reset, otherwise the gain of continued transmissions in duplication vanishes.

This leads to the new situation that an RLC entity that is suspended due to maximum number of retransmission reached continues to be considered somehow by MAC. It should be clarified that data of the suspended RLC logical channel is not considered on MAC, i.e. simply ignored as if there was not such a logical channel.

This should be considered in both LCP and BSR procedures.

When an RLC logical channel is suspended due to maximum number of retransmissions reached, remaining data on that logical channel is ignored by MAC, i.e. MAC ignores suspended logical channels in LCP and BSR calculation.

This ensures also that further retransmissions of the failed RLC entity beyond the retransmission limit do no take place.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	When an RLC logical channel is suspended due to maximum number of retransmissions reached, remaining data on that logical channel is ignored by MAC, i.e. MAC ignores suspended logical channels in LCP and BSR calculation.

# References

- R2-1806090, RLC failure handling, LG, 3GPP TSG-RAN WG2 Meeting #101, Athens, Greece, 26th February – 2nd March 2018