3GPP TSG-RAN WG2 Meeting #102 	                   R2-1807735

Busan, Korea, 20th – 25th May 2018              (re-submission of R2-1804397)

Source: 			ZTE Corporation

Title: 	Measurement and gap configuration framework in NGEN-DC

Agenda item:		10.2.6

Document for: 	Discussion and Decision

- Introduction

For EN-DC, RAN2 made a lot of progress on measurement configuration &coordination as well as gap configuration &coordination, we summarized as below, in this contribution, we provide more views on NGEN-DC case.

- Measurement configuration and reporting:

  - MN(LTE) supports B1 and B2 event measurement on NR frequencies;

  - MN(LTE) only support SSB based NR measurement, doesn’t support CSI-RS based NR measurement;

  - SN(NR) can only configure intra-RAT measurement, doesn’t support inter-RAT measurement;

  - The measure results of NR serving cell and best neighbour cells can be included in MN(LTE) A3/A4/A5 events and B1/B2 events reporting.

- Measurement coordination:

  - MN(LTE) indicates the number of frequency layers that can be used in the SN;

  - Re-negotiation (SN signaling to MN for the purpose to ask for more number of frequency layer) is not supported (at least in Rel-15).

- Gap configuration:

  - Per-UE gap or independent LTE/FR1 gap is configured by MN, and UE performs gap calculation based on the SFN and subframe of PCell;

  - Independent FR2 gap is configured by SN, and UE performs gap calculation based on the SFN and subframe of PSCell;

  - UE can report the gap capability to network. For independent gap capable UE, the network can configure either per-UE gap or per-FR gap, and MN indicates the used gap type in Uu interface.

- Gap coordination:

  - For per-UE gap or independent LTE/FR1 gap, MN transmit the gap pattern to SN via CG-ConfigInfo;

  - For per-UE gap, if SN hasn’t received the gap pattern from MN, and SN wants to configure gap assisted FR1/FR2 measurements, SN can send the FR1/FR2 frequencies to MN via CG-Config;

  - For independent LTE/FR1 gap, if SN hasn’t received the gap pattern from MN, and SN wants to configure gap assisted FR1 measurements, SN can send the FR1 frequencies to MN via CG-Config;

  - For independent FR2 gap, if MN wants to configure gap assisted FR2 measurements, MN can send the FR1 frequencies to SN via CG-ConfigInfo.

- Consideration on measurement framework

The architecture of EN-DC and NGEN-DC are illustrated in figure1:

Figure1 Architecture of EN-DC and NGEN-DC

- Measurement configuration and reporting

Considering the architecture of NGEN-DC is quite similar with EN-DC, after finalizing the specification work for EN-DC, the procedure and ASN.1 coding design of measurement part can be almost reused in NGEN-DC, and in our understanding, stage3 modification is not needed except some minor terminology changes.

Proposal 1	In NGEN-DC, reuse the same measurement framework as defined in EN-DC:

- SN configured measurement can be transmitted via SRB3, or embedded in LTE RRC message via SRB1; Once SRB3 is configured, the SN configured measurement report can only be transmitted via SRB3;

- Both intra-RAT and inter-RAT measurement are supported at MN side, while only intra-RAT measurement is supported at SN side;

- Only SSB based measurement is supported in MN configured NR measurement;

- The available measurement results of NR serving cell and best neighbour cells can be included in LTE configured A3/A4/A5 events and NR B1/B2 events.

- Measurement coordination

Similar with measurement configuration, in NGEN-DC, although Xn interface is used instead of X2 interface between MN and SN, but as we agreed in RAN2, the RRC inter-node messages(i.e. CG-ConfigInfo and CG-Config) which defined in 38.331 will be reused in NGEN-DC. If proposal1 is adopted, there’s no obvious difference between EN-DC and NGEC-DC on the aspect of measurement coordination. No matter whether further enhancement will be introduced in EN-DC measurement coordination(eg. introduce re-negotiation mechanism from SN), at least for now, the current conclusions in EN-DC can be reused.

Proposal 2	In NGEN-DC, reuse the measurement coordination framework as defined in EN-DC:

- MN indicates a maximum number of measured NR frequency layers that can be used in SN via CG-ConfigInfo;

- Re-negotiation(SN signaling to MN for the purpose to ask for more number of frequency layer) is not supported (at least in Rel-15)

- Consideration on gap configuration and coordination

- Gap configuration

As we discussed above, considering the architecture of NGEN-DC and EN-DC are nearly same, the gap configuration framework in EN-DC can also be directly reused in NGEN-DC, and stage3 modification is not needed except some minor terminology change in text description.

Proposal 3	In NGEN-DC, reuse the same gap configuration framework as defined in EN-DC:

- Either per-UE gap or independent LTE/FR1 gap is configured by MN, and UE performs gap calculation based on the SFN and subframe of PCell;

- Independent FR2 gap is configured by SN, and UE performs gap calculation based on the SFN and subframe of PSCell;

- UE can report the gap capability(i.e. whether supports per-FR gap) to network.

- Gap coordination

Similar with measurement coordination, in NGEN-DC, although Xn interface is used instead of X2 interface between MN and SN, but as we agreed in RAN2, the inter-node messages which defined in 38.331 are reused. If proposal3 is adopted, then the similar gap coordination mechanism can be reused as well.

Proposal 4	In NGEN-DC, reuse the gap coordination framework as defined in EN-DC:

- For either per-UE gap or independent LTE/FR1 gap, MN transmits the gap pattern to SN via CG-ConfigInfo;

- For per-UE gap, if SN has not received the gap pattern from MN, and SN wants to configure gap assisted FR1/FR2 measurements, SN can send the FR1/FR2 frequencies to MN via CG-Config;

- For independent LTE/FR1 gap, if SN hasn’t received the gap pattern from MN, and SN wants to configure gap assisted FR1 measurements, SN can send the FR1 frequencies to MN via CG-Config;

- For independent FR2 gap, if MN wants to configure gap assisted FR2 measurements, MN can send the FR2 frequencies to SN via CG-ConfigInfo.

By adopting P1~P4, the ASN.1 design in EN-DC can be directly reused for NGEN-DC with some minor terminology change.

Proposal 5 	Stage3 ASN.1 design in EN-DC can be directly reused for NGEN-DC, only minor terminology change is needed in text procedure and IE descriptions. (eg. Change “EN-DC” into “EN-DC or NGEN-DC”)

- Conclusion and proposals

RAN2 is kindly asked to discuss and adopt the following proposal:

Proposal 1	In NGEN-DC, reuse the same measurement framework as defined in EN-DC:

- SN configured measurement can be transmitted via SRB3, or embedded in LTE RRC message via SRB1; Once SRB3 is configured, the SN configured measurement report can only be transmitted via SRB3;

- Both intra-RAT and inter-RAT measurement are supported at MN side, while only intra-RAT measurement is supported at SN side;

- Only SSB based measurement is supported in MN configured NR measurement;

- The available measurement results of NR serving cell and best neighbour cells can be included in LTE configured A3/A4/A5 events and NR B1/B2 events.

Proposal 2	In NGEN-DC, reuse the measurement coordination framework as defined in EN-DC:

- MN indicates a maximum number of measured NR frequency layers that can be used in SN via CG-ConfigInfo;

- Re-negotiation(SN signaling to MN for the purpose to ask for more number of frequency layer) is not supported (at least in Rel-15)

Proposal 3	In NGEN-DC, reuse the same gap configuration framework as defined in EN-DC:

- Either per-UE gap or independent LTE/FR1 gap is configured by MN, and UE performs gap calculation based on the SFN and subframe of PCell;

- Independent FR2 gap is configured by SN, and UE performs gap calculation based on the SFN and subframe of PSCell;

- UE can report the gap capability(i.e. whether supports per-FR gap) to network.

Proposal 4	In NGEN-DC, reuse the gap coordination framework as defined in EN-DC:

- For either per-UE gap or independent LTE/FR1 gap, MN transmits the gap pattern to SN via CG-ConfigInfo;

- For per-UE gap, if SN has not received the gap pattern from MN, and SN wants to configure gap assisted FR1/FR2 measurements, SN can send the FR1/FR2 frequencies to MN via CG-Config;

- For independent LTE/FR1 gap, if SN hasn’t received the gap pattern from MN, and SN wants to configure gap assisted FR1 measurements, SN can send the FR1 frequencies to MN via CG-Config;

- For independent FR2 gap, if MN wants to configure gap assisted FR2 measurements, MN can send the FR2 frequencies to SN via CG-ConfigInfo.

Proposal 5 	Stage3 ASN.1 design in EN-DC is directly reused for NGEN-DC, only minor terminology change is needed in text procedure and IE descriptions. (eg. Change “EN-DC” into “EN-DC or NGEN-DC”)