3GPP TSG-RAN2 Meeting #102	R2-1807498

# Busan, Korea, 21st May 2018 - 25th May 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0008 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on total layer2 buffer size |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - For NR, PDCP layer is responsible for reordering and thus reordering packets in PDCP layer should be taken into account for all bearer types. - Current text states that total L2 buffer may consider the pre-processed RLC PDUs and this may lead to NW’s under-estimation of buffer usage in the UE. - For MR-DC, it is not clear what value of RLC RTT and MaxULDataRate used considering different RLC RTT for different CGs. - For MR-DC for non-split bearer operation, it is not clear what value of RLC RTT and MaxDLDataRate used considering different RLC RTT for different CGs. - The concrete equation of total L2 buffer size of MR-DC with split bearer (taking into both MN terminated and SN terminated into account) is missing. - Values of RLC RTT per SCS are still TBD. - It is not clear which RLC RTT is used if the NR CA composes of CCs configured with different SCS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following changes are done: 1. It is clarified that the total layer2 buffer is used to store PDCP reordering data in receiving side also for non-split bearer. 2. It is clarified that UE shall not take the pre-processed RLC PDUs into account for total L2 buffer size 3. It is clarifed that for MR-DC, buffer size from UL buffering components is the sum of MaxULDataRate_MN * RTT_MN and MaxULDataRate_SN * RTT_SN. 4. It is clarifed that for MR-DC for non-split bearer operation, buffer size from DL buffering components is the sum of MaxDLDataRate_MN * RTT_MN and MaxDLDataRate_SN * RTT_SN. 5. Added the equation to calculate total L2 buffer size considering both MN terminated and SN terminated split bearer. 6. Remove TBDs in table 4.1.4-1 7. It is clarified that the RLC RTT for the smallest SCS is used to calcualte buffer size for the for given band combination of NR CG. Impact analysis: Impacted functionality: UE total L2 buffer size calucation Inter-operability: If the proposed change is implemented by the UE and not by the gNB, gNB may transmit PDCP PDU larger than UE can store in the buffer. If the proposed change is implemented by the eNB and not by the UE, there is no interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE and NW cannot be have the same understanding of the UE L2 buffer size and NW may over-schedule resulting in packet drop in the UE due to buffer over-flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

### 4.1.4	Total layer 2 buffer size

The total layer 2 buffer size is defined as the sum of the number of bytes that the UE is capable of storing in the RLC transmission windows (which are transmitted) and RLC reception and reordering windows and also in PDCP reordering windows for all radio bearers.

The required total layer 2 buffer size for split bearer operation in MR-DC is calculated by maximizing following:

- MaxULDataRate_MN * RTT_MN + MaxULDataRate_SN * RTT_SN + MaxDLDataRate_SN * RTT_SN + MaxDLDataRate_MN * (RTT_SN + X2 delay + Queuing in SN)

- MaxULDataRate_MN * RTT_MN + MaxULDataRate_SN * RTT_SN + MaxDLDataRate_MN * RTT_MN + MaxDLDataRate_SN * (RTT_MN + X2 delay + Queuing in MN)

The required total layer 2 buffer size for non-split bearer operation in MR-DC is calculated by (MaxULDataRate_MN + MaxDLDataRate_MN)* RTT_MN + (MaxULDataRate_SN + MaxULDataRate_SN) * RTT_SN. Otherwise it is calculated by MaxDLDataRate * RLC RTT + MaxULDataRate * RLC RTT.

The required total layer 2 buffer size is determined as the maximum total layer 2 buffer size of all the calculated ones for each band combination in the supported MR-DC or NR band combinations. When NR band combination part of supported MR-DC or NR band combination complies with multiple SCSs, the RLC RTT is that for the smallest SCS.

wherein

X2 delay + Queuing in SN = 25ms

X2 delay + Queuing in MN =  [TBD] ms

RLC RTT for EUTRA cell group = 75ms

RLC RTT for NR cell group is defined in Table 4.1.4-1

Table 4.1.4-1: RLC RTT for NR cell group per SCS.

| SCS (KHz) | RLC RTT (ms) |
| --- | --- |
| 15KHz | 55 |
| 30KHz | 55 |
| 60KHz | 35 |
| 120KHz | 25 |

End of change