3GPP TSG-RAN2 Meeting #102	R2-1807701

# Busan, Korea (Republic Of), 21st May 2018 - 25th May 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0150 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for handling the configuredGrantTimer during BWP switch |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | If the BWP switch happens when the configuredGrantTimer is running for one or more HARQ processes, the UE behaviour is ambiguous. The configuration of the configuredGrantTimer could be different in the old and new BWP. To address this issue, the following agreement has been reached in the RAN2#101b meeting: The configured grant timer, once started, runs to completion with no change to its original duration if a BWP switch takes place. This CR implements the above agreement in 38.321. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the configuredGrantTimer for a HARQ process is running at the time of the BWP switch, it runs to completion with no change to its original duration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior can be unpredictable. Some UEs might stop the timer, others might adjust the duration based on the configuredGrantTimer configuration in the new BWP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

## 5.15	Bandwidth Part (BWP) operation

In addition to clause 12 of TS 38.213 [6], this subclause specifies requirements on BWP operation.

A Serving Cell may be configured with one or multiple BWPs, and the maximum number of BWP per Serving Cell is specified in TS 38.213 [6].

The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time. The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bandwidthPartInactivityTimer, or by the MAC entity itself upon initiation of Random Access procedure. Upon addition of SpCell or activation of an SCell, one BWP is initially active without receiving PDCCH indicating a downlink assignment or an uplink grant. The active BWP for a Serving Cell is indicated by either RRC or PDCCH (as specified in TS 38.213 [6]). For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.

If the configuredGrantTimer for a HARQ process is running when the BWP is switched, it runs to completion with no change to its original duration.

On the active BWP for each activated Serving Cell configured with a BWP, the MAC entity shall apply normal operations including:

1>	transmit on UL-SCH;

1>	transmit on RACH;

1>	monitor the PDCCH;

1>	transmit PUCCH;

1>	transmit SRS;

1>	receive DL-SCH;

1>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 according to the stored configuration, if any, and to start in the symbol according to rules in subclause 5.8.2.

On the inactive BWP for each activated Serving Cell configured with a BWP, the MAC entity shall:

1>	not transmit on UL-SCH;

1>	not transmit on RACH;

1>	not monitor the PDCCH;

1>	not transmit PUCCH;

1>	not transmit SRS;

1>	not receive DL-SCH;

1>	clear any configured downlink assignment and configured uplink grant of configured grant Type 2;

1>	suspend any configured uplink grant of configured Type 1.

<End of modification>