3GPP TSG RAN2 Meeting #102	R2-1807733

# Busan, South Korea, 21st – 25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.355 | CR | 0200 | rev | - | Current version: | 14.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | - X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR CID addition to LPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To provide NR CID support for NR Rel15 devices |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extending the CID reporting to accomodated NR CID in both CommonIEsRequestAssistanceData and ECID-SignalMeasurementInformation . Adding the following items to E-CID support: - NR physical cell ID - NR cell global ID - NR ARFCN - NR SFN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not having any NR support in LPP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 6.4.1, 6.4.2, 6.5.2.2, 6.5.2.6, 6.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of Change

# 2	References

[x1]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[x2]	3GPP TS 38.101: “NR; User Equipment (UE) radio transmission and reception”.

Next Change

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply.

ADR	Accumulated Delta-Range

A-GNSS	Assisted-GNSS

AP	Access Point

ARFCN	Absolute Radio Frequency Channel Number

BDS	BeiDou Navigation Satellite System

BSSID	Basic Service Set Identifier

BTS	Base Transceiver Station (GERAN)

CID	Cell-ID (positioning method)

CNAV	Civil Navigation

CRS	Cell-specific Reference Signals

ECEF	Earth-Centered, Earth-Fixed

ECGI	Evolved Cell Global Identifier

ECI	Earth-Centered-Inertial

E-CID	Enhanced Cell-ID (positioning method)

EGNOS	European Geostationary Navigation Overlay Service

E-SMLC	Enhanced Serving Mobile Location Centre

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

EOP	Earth Orientation Parameters

EPDU	External Protocol Data Unit

FDMA	Frequency Division Multiple Access

FEC	Forward Error Correction

FTA	Fine Time Assistance

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

ICD	Interface Control Document

IOD	Issue of Data

IS	Interface Specification

LLA	Latitude Longitude Altitude

LPP	LTE Positioning Protocol

LPPa	LTE Positioning Protocol Annex

LSB	Least Significant Bit

MBS	Metropolitan Beacon System

MO-LR	Mobile Originated Location Request

MSAS	Multi-functional Satellite Augmentation System

MSB	Most Significant Bit

msd	mean solar day

MT-LR	Mobile Terminated Location Request

NAV	Navigation

NB-IoT	NarrowBand Internet of Things

NICT	National Institute of Information and Communications Technology

NI-LR	Network Induced Location Request

NPRS	Narrowband Positioning Reference Signals

NR	New Radio

NRSRP	Narrowband Reference Signal Received Power

NRSRQ	Narrowband Reference Signal Received Quality

NTSC	National Time Service Center of Chinese Academy of Sciences

OTDOA	Observed Time Difference Of Arrival

PDU	Protocol Data Unit

Next Change

## 6.4	Common IEs

Common IEs comprise IEs that are applicable to more than one LPP positioning method.

### 6.4.1	Common Lower-Level IEs

#### –	AccessTypes

The IE AccessTypes is used to indicate several cellular access types using a bit map.

-- ASN1START

# AccessTypes ::= SEQUENCE {

accessTypes		BIT STRING {	eutra		(0),

utra		(1),

gsm			(2),

nb-iot		(3),

nr			(4) } (SIZE (1..8)),

...

}

-- ASN1STOP

AccessTypes field descriptions

accessTypes This field specifies the cellular access type(s). This is represented by a bit string, with a one-value at the bit position means the particular access type is addressed; a zero-value means not addressed.

### […]

#### –	ARFCN-ValueNR

The IE ARFCN-ValueNR is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR global frequency raster, as defined in TS 38.101-2 [x2].

-- ASN1START

ARFCN-ValueNR ::=				INTEGER (0..3279165)

-- ASN1STOP

### […]

#### – NR-CGI

The IE NR-CGI  specifies the NR Cell Global Identifier (NR-CGI), the globally unique identity of a cell in NR [x1].

-- ASN1START

# NR-CGI ::= SEQUENCE {

mcc-r15					SEQUENCE (SIZE (3)) 	OF INTEGER (0..9),

mnc-r15					SEQUENCE (SIZE (2..3)) 	OF INTEGER (0..9),

nr-cellidentity-r15		BIT STRING (SIZE (36))

}

-- ASN1STOP

Next Change

### 6.4.2	Common Positioning

### […]

#### –	CommonIEsRequestAssistanceData

The CommonIEsRequestAssistanceData carries common IEs for a Request Assistance Data LPP message Type.

-- ASN1START

# CommonIEsRequestAssistanceData ::= SEQUENCE {

primaryCellID		ECGI		OPTIONAL,	-- Cond EUTRA

... ,

[[

primaryNRCellID-r15		NR-CGI		OPTIONAL	-- Cond NR

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EUTRA | The field is mandatory present for E-UTRA or NB-IoT access. The field shall be omitted for non-EUTRA and non-NB-IoT user plane support. |
| NR | The field is only mandatory present for NR access. |

CommonIEsRequestAssistanceData field descriptions

primaryCellID This parameter identifies the current primary cell for the target device.

primaryNRCellID This parameter identifies the current primary NR cell for the target device.

Next Change

### 6.5.2	A-GNSS Positioning

### […]

#### NetworkTime

-- ASN1START

# NetworkTime ::= SEQUENCE {

secondsFromFrameStructureStart				INTEGER(0..12533),

fractionalSecondsFromFrameStructureStart	INTEGER(0..3999999),

frameDrift									INTEGER (-64..63)	OPTIONAL,	-- Cond GNSSsynch

cellID		CHOICE {

eUTRA		SEQUENCE {

physCellId			INTEGER (0..503),

cellGlobalIdEUTRA	CellGlobalIdEUTRA-AndUTRA	OPTIONAL,	-- Need ON

earfcn				ARFCN-ValueEUTRA,

...,

[[ earfcn-v9a0		ARFCN-ValueEUTRA-v9a0 OPTIONAL	-- Cond EARFCN-max

]]

},

uTRA		SEQUENCE {

mode	CHOICE {

fdd		SEQUENCE {

primary-CPICH-Info	INTEGER (0..511),

...

},

tdd		SEQUENCE {

cellParameters		INTEGER (0..127),

...

}

},

cellGlobalIdUTRA	CellGlobalIdEUTRA-AndUTRA	OPTIONAL,	-- Need ON

uarfcn				ARFCN-ValueUTRA,

...

},

gSM			SEQUENCE {

bcchCarrier			INTEGER (0..1023),

bsic				INTEGER (0..63),

cellGlobalIdGERAN	CellGlobalIdGERAN			OPTIONAL,	-- Need ON

...

},

...,

nBIoT-r14	SEQUENCE {

nbPhysCellId-r14	INTEGER (0..503),

nbCellGlobalId-r14	ECGI						OPTIONAL,	-- Need ON

nbCarrierFreq-r14	CarrierFreq-NB-r14,

...

},

nr-r15		SEQUENCE {

nrPhysCellId-r15	INTEGER (0..1007),

CellGlobalIdNR-r15 	NR-CGI-r15					OPTIONAL,  	-- Need ON

nrarfcn-r15			ARFCN-ValueNR-r15,

...

}

},

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EARFCN-max | The field is mandatory present if the corresponding earfcn (i.e. without suffix) is set to maxEARFCN. Otherwise the field is not present. |
| GNSSsynch | The field is present and set to 0 if NetworkTime is synchronized to gnss-SystemTime; otherwise the field is optionally present, need OR. |

NetworkTime field descriptions

secondsFromFrameStructureStart This field specifies the number of seconds from the beginning of the longest frame structure in the corresponding air interface. In case of E-UTRA, the SFN cycle length is 10.24 seconds. In case of UTRA, the SFN cycle length is 40.96 seconds. In case of GSM, the hyperfame length is 12533.76 seconds. In case of NB-IoT, the Hyper-SFN cycle lengths is 10485.76 seconds. In case of NR, the SFN cycle length is 10.24 seconds.

fractionalSecondsFromFrameStructureStart This field specifies the fractional part of the secondsFromFrameStructureStart in 250 ns resolution. The total time since the particular frame structure start is secondsFromFrameStructureStart + fractionalSecondsFromFrameStructureStart

frameDrift This field specifies the drift rate of the GNSS-network time relation with scale factor 2-30 seconds/second, in the range from -5.9605e-8 to +5.8673e-8 sec/sec.

cellID This field specifies the cell for which the GNSS–network time relation is provided.

physCellId This field specifies the physical cell identity of the reference cell (E-UTRA), as defined in [12], for which the GNSS network time relation is provided.

cellGlobalIdEUTRA This field specifies the Evolved Cell Global Identifier (ECGI), the globally unique identity of a cell in E-UTRA, of the reference cell for the GNSS-network time relation, as defined in [12].

earfcn This field specifies E-ARFCN of the reference cell for the GNSS-network time relation (E-UTRA). In case the server includes earfcn-v9a0, the server shall set the corresponding earfcn (i.e. without suffix) to maxEARFCN.

primary-CPICH-Info This field specifies the physical cell identity of the reference cell (UTRA) for the GNSS-network time relation, as defined in [13].

cellParameters This field specifies the physical cell identity of the reference cell (UTRA) for the GNSS-network time relation, as defined in [13].

cellGlobalIdUTRA The filed specifies the global UTRAN Cell Identifier, the globally unique identity of a cell in UTRA, of the reference cell for the GNSS-network time relation, as defined in [13].

uarfcn This field specifies ARFCN of the reference cell for the GNSS-network time relation (UTRA).

bcchCarrier This field specifies the absolute GSM RF channel number of the BCCH of the reference base station (GERAN) for the GNSS-network time relation, as defined in [14].

bsic This field specifies the Base Station Identity Code of the reference base station (GERAN) for the GNSS-network time relation, as defined in [14].

cellGlobalIdGERAN This field specifies the Cell Global Identification (CGI), the globally unique identity of a cell in GERAN, of the reference base station for the GNSS-network time relation.

nbPhysCellId This field specifies the narrowband physical layer cell identity of the NB-IoT reference cell, as defined in [12], for which the GNSS network time relation is provided.

nbCellGlobalId This field specifies the global cell identifier of the NB-IoT reference cell for which the GNSS-network time relation is provided, as defined in [12].

nbCarrierFreq This field specifies the carrier frequency of the NB-IoT reference cell for which the GNSS-network time relation is provided.

nrPhysCellId This field specifies the physical cell identity of the reference cell (NR), as defined in [x1], for which the GNSS network time relation is provided.

cellGlobalIdNR This field specifies the NR Cell Global Identifier (NR-CGI), the globally unique identity of a cell in NR, of the reference cell for the GNSS-network time relation, as defined in [x1].

nrarfcn This field specifies NR-ARFCN of the reference cell (NR) for the GNSS-network time relation.

Next Change

#### 6.5.2.6	GNSS Location Information Elements

#### –	GNSS-SignalMeasurementInformation

The IE GNSS-SignalMeasurementInformation is used by the target device to provide GNSS signal measurement information to the location server and GNSS-network time association if requested by the location server. This information includes the measurements of code phase, Doppler, C/No and optionally accumulated carrier phase, also called accumulated deltarange (ADR), which enable the UE-assisted GNSS method where position is computed in the location server. Figure 6.5.2.6-1 illustrates the relation between some of the fields.

-- ASN1START

# GNSS-SignalMeasurementInformation ::= SEQUENCE {

measurementReferenceTime		MeasurementReferenceTime,

gnss-MeasurementList			GNSS-MeasurementList,

...

}

-- ASN1STOP

GNSS-SignalMeasurementInformation field descriptions

measurementReferenceTime This field specifies the GNSS system time for which the information provided in gnss-MeasurementList is valid. It may also include network time, if requested by the location server and supported by the target device.

gnss-MeasurementList This field provides GNSS signal measurement information for up to 16 GNSSs.

#### –	MeasurementReferenceTime

The IE MeasurementReferenceTime is used to specify the time when the measurements provided in A-GNSS-ProvideLocationInformation are valid.  It may also include GNSS-network time association, in which case reported measurements shall be valid for the cellular frame boundary defined in the network time association.

-- ASN1START

# MeasurementReferenceTime  ::= SEQUENCE {

gnss-TOD-msec		INTEGER (0..3599999),

gnss-TOD-frac		INTEGER (0..3999)			OPTIONAL,

gnss-TOD-unc		INTEGER (0..127)			OPTIONAL,

gnss-TimeID			GNSS-ID,

networkTime			CHOICE {

eUTRA	SEQUENCE {

physCellId			INTEGER (0..503),

cellGlobalId		CellGlobalIdEUTRA-AndUTRA		OPTIONAL,

systemFrameNumber	BIT STRING (SIZE (10)),

...

},

uTRA	SEQUENCE {

mode					CHOICE {

fdd			SEQUENCE {

primary-CPICH-Info	INTEGER (0..511),

...

},

tdd			SEQUENCE {

cellParameters		INTEGER (0..127),

...

}

},

cellGlobalId			CellGlobalIdEUTRA-AndUTRA		OPTIONAL,

referenceSystemFrameNumber

INTEGER (0..4095),

...

},

gSM		SEQUENCE {

bcchCarrier			INTEGER (0..1023),

bsic				INTEGER (0..63),

cellGlobalId		CellGlobalIdGERAN					OPTIONAL,

referenceFrame		SEQUENCE {

referenceFN		INTEGER (0..65535),

referenceFNMSB		INTEGER (0..63)		OPTIONAL,

...

},

deltaGNSS-TOD		INTEGER (0 .. 127)		OPTIONAL,

...

},

...,

nbIoT-r14

SEQUENCE {

nbPhysCellId-r14	INTEGER (0..503),

nbCellGlobalId-r14	ECGI						OPTIONAL,

sfn-r14				BIT STRING (SIZE (10)),

hyperSFN-r14		BIT STRING (SIZE (10))		OPTIONAL,

...

},

nr-r15	SEQUENCE {

nrPhysCellId-r15	INTEGER (0..1007),

cellGlobalIdNR-r15	NR-CGI-r15					OPTIONAL,

nrSystemReferenceNumber-r15

BIT STRING (SIZE (10)),

...

}

}		OPTIONAL,

...

}

-- ASN1STOP

MeasurementReferenceTime field descriptions

gnss-TOD-msec This field specifies the GNSS TOD for which the measurements and/or location estimate are valid. The 22 bits of GNSS TOD are the least significant bits. The most significant bits shall be derived by the location server to unambiguously derive the GNSS TOD. The value for GNSS TOD is derived from the GNSS specific system time indicated in gnss-TimeID rounded down to the nearest millisecond unit. Scale factor 1 millisecond.

gnss-TOD-frac This field specifies the fractional part of the GNSS TOD in 250 ns resolution. The total GNSS TOD is given by gnss-TOD-msec + gnss-TOD-frac. Scale factor 250 nanoseconds.

gnss-TOD-unc This field provides the accuracy of the relation GNSS-network time when GNSS-network time association is provided. When GNSS-network time association is not provided, this element can be included to provide the accuracy of the reported gnss-TOD-msec. If GNSS TOD is the given GNSS time, then the true GNSS time, corresponding to the provided network time if applicable, as observed at the target device location, lies in the interval [GNSS TOD – gnss-TOD-unc, GNSS TOD + gnss-TOD-unc]. The uncertainty r, expressed in microseconds, is mapped to a number K, with the following formula: r = C*(((1+x)K)-1) with C = 0.5 and x = 0.14. To encode any higher value of uncertainty than that corresponding in the above formula to K=127, the same value, K=127, shall also be used. The uncertainty is then coded on 7 bits, as the binary encoding of K. Examples of gnss-TOD-unc value are as in the table Value of K to Value of uncertainty relation below. This field shall be included if the target device provides GNSS-network time relationship.

gnss-TimeID This field specifies the GNSS system time for which the gnss-TOD-msec (and gnss-TOD-frac if applicable) is provided.

networkTime These fields specify the network time event which the GNSS TOD time stamps. This field shall be included if the target device provides GNSS-network time relationship.

physCellId This field identifies the reference cell, as defined in [12], that is used for the GNSS-network time relation.

cellGlobalId This field specifies the globally unique cell identifier (Evolved Cell Global Identifier (ECGI) in E-UTRA, global UTRAN Cell Identifier in UTRA, or Cell Global Identification (CGI) in GERAN) of the reference cell, as defined in [12] for E-UTRA and [13] for UTRA, for which the GNSS network time relation is provided.

systemFrameNumber This field specifies the system frame number in E-UTRA which the GNSS time time stamps, as defined in [12].

mode This field identifies the reference cell for the GNSS-network time relation, as defined in [13].

referenceSystemFrameNumber This field specifies the system frame number in UTRA, as defined in [13], which is used for time stamping.

bcchCarrier, bsic This field identifies the reference cell for the GNSS-network time relation in GERAN, as defined in [14].

referenceFN, referenceFNMSB These fields specify the frame number in GERAN which the GNSS time time stamps, as defined in [14]. The time of the reference frame boundary is as observed by the target device, i.e. without Timing Advance compensation. The referenceFNMSB field indicates the most significant bits of the frame number of the reference BTS corresponding to the GNSS-MeasurementList. Starting from the complete GSM frame number denoted FN, the target device calculates Reference FN MSB as Reference FN MSB = floor(FN/42432) The complete GSM frame number FN can then be reconstructed in the location server by combining the fields referenceFN with referenceFNMSB in the following way FN = referenceFNMSB *42432 + referenceFN

deltaGNSS-TOD This field specifies the difference in milliseconds between gnss-TOD-msec reported and the milli-second part of the SV time tsv_1 of the first SV in the list reported from the target device, as defined in [14]. The deltaGNSS-TOD is defined as deltaGNSS-TOD = gnss-TOD-msec - fix(tsv_1) where fix() denotes rounding to the nearest integer towards zero.

nbPhysCellId This field identifies the reference cell, as defined in [12] that is used for the GNSS-network time relation.

nbCellGlobalId This field specifies the global cell identifier of the NB-IoT reference cell, as defined in [12], for which the GNSS network time relation is provided.

sfn This field specifies the system frame number in NB-IoT which the GNSS time time stamps, as defined in [12].

hyperSFN This field specifies the hyper-SFN in NB-IoT which the GNSS time time stamps, as defined in [12].

nrPhysCellId This field identifies the reference cell, as defined in [x1], that is used for the GNSS-network time relation.

cellGlobalIdNR This field specifies the globally unique cell identifier (NR-CGI) in NR, of the reference cell, as defined in [x1] for NR, for which the GNSS network time relation is provided.

nrSystemFrameNumber This field specifies the system frame number in NR which the GNSS time time stamps, as defined in [x1].

# Value of K to Value of uncertainty relation

| Value of K | Value of uncertainty |
| --- | --- |
| 0 | 0 microseconds |
| 1 | 0.07 microseconds |
| 2 | 0.1498 microseconds |
| - | - |
| 50 | 349.62 microseconds |
| - | - |
| 127 | ≥ 8430000 microseconds |

Next Change

### 6.5.3	Enhanced Cell ID Positioning

#### 6.5.3.1	E-CID Location Information

#### –	ECID-ProvideLocationInformation

The IE ECID-ProvideLocationInformation is used by the target device to provide E-CID location measurements to the location server. It may also be used to provide ECID positioning specific error reason.

-- ASN1START

# ECID-ProvideLocationInformation ::= SEQUENCE {

ecid-SignalMeasurementInformation	ECID-SignalMeasurementInformation		OPTIONAL,

ecid-Error							ECID-Error								OPTIONAL,

...

}

-- ASN1STOP

#### 6.5.3.2	E-CID Location Information Elements

#### –	ECID-SignalMeasurementInformation

The IE ECID-SignalMeasurementInformation is used by the target device to provide various UE-measurements to the location server.

-- ASN1START

# ECID-SignalMeasurementInformation ::= SEQUENCE {

primaryCellMeasuredResults	MeasuredResultsElement	OPTIONAL,

measuredResultsList			MeasuredResultsList,

...

}

# MeasuredResultsList ::= SEQUENCE (SIZE(1..32)) OF MeasuredResultsElement

# MeasuredResultsElement ::= SEQUENCE {

physCellId		INTEGER (0..503),

cellGlobalId	CellGlobalIdEUTRA-AndUTRA			OPTIONAL,

arfcnEUTRA 		ARFCN-ValueEUTRA,

systemFrameNumber

BIT STRING (SIZE (10))				OPTIONAL,

rsrp-Result		INTEGER (0..97)						OPTIONAL,

rsrq-Result		INTEGER (0..34)						OPTIONAL,

ue-RxTxTimeDiff	INTEGER (0..4095)					OPTIONAL,

...,

[[ arfcnEUTRA-v9a0		ARFCN-ValueEUTRA-v9a0 		OPTIONAL		-- Cond EARFCN-max

]],

[[ nrsrp-Result-r14		INTEGER (0..113)			OPTIONAL,

nrsrq-Result-r14		INTEGER (0..74)				OPTIONAL,

carrierFreqOffsetNB-r14

CarrierFreqOffsetNB-r14		OPTIONAL,		-- Cond NB-IoT

hyperSFN-r14			BIT STRING (SIZE (10))		OPTIONAL

]]

[[ nrPhysCellId-r15		INTEGER (0..1007),

cellGlabalIdNR-r15	NR-CGI				  		OPTIONAL,

arfcnNR-r15			ARFCN-ValueNR,

nrSystemFrameNumber-r15

BIT STRING (SIZE (10))		OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EARFCN-max | The field is mandatory present if the corresponding arfcnEUTRA (i.e. without suffix) is set to maxEARFCN. Otherwise the field is not present. |
| NB-IoT | The field is mandatory present if the measured cell is a NB-IoT cell. Otherwise it is not present. |

ECID-SignalMeasurementInformation field descriptions

primaryCellMeasuredResults This field contains measurements for the primary cell, when the target device reports measurements for both primary cell and neighbour cells. This field shall be omitted when the target device reports measurements for the primary cell only, in which case the measurements the primary cell is reported in the measuredResultsList.

measuredResultsList This list contains the E-CID measurements for up to 32 cells.

physCellId This field specifies the physical cell identity of the measured cell.

cellGlobalId This field specifies cell global ID of the measured cell. The target device shall provide this field if it was able to determine the ECGI of the measured cell at the time of measurement.

arfcnEUTRA This field specifies the ARFCN of the measured E-UTRA carrier frequency, as defined in [12]. In case the target device includes arfcnEUTRA-v9a0, the target device shall set the corresponding arfcnEUTRA (i.e. without suffix) to maxEARFCN.

systemFrameNumber This field specifies the system frame number of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the SFN of the cell at the time of measurement.

rsrp-Result This field specifies the reference signal received power (RSRP) measurement, as defined in [12], [17].

rsrq-Result This field specifies the reference signal received quality (RSRQ) measurement, as defined in [12], [17].

ue-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in [17]. It is provided only for measurements on the UE’s primary cell. Measurement report mapping is according to 3GPP TS 36.133 [18].

nrsrp-Result This field specifies the narrowband reference signal received power (NRSRP) measurement, as defined in [17]. Measurement report mapping is according to TS 36.133 [18].

nrsrq-Result This field specifies the narrowband reference signal received quality (NRSRQ) measurement, as defined in [17]. Measurement report mapping is according to TS 36.133 [18].

carrierFreqOffsetNB This field specifies the offset of the NB-IoT channel number to ARFCN given by arfcnEUTRA as defined in TS 36.101 [21].

hyperSFN This field specifies the hyper-SFN of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the hyper-SFN of the cell at the time of measurement.

nrPhysCellId This field specifies the physical cell identity of the measured NR cell.

nrCellGlobalId This field specifies cell global ID of the measured NR cell. The target device shall provide this field if it was able to determine the CGI of the measured NR cell at the time of measurement.

arfcnNR This field specifies the ARFCN of the measured NR carrier frequency, as defined in [x1].

nrSystemFrameNumber This field specifies the system frame number of the measured NR cell during which the measurements have been performed. The target device shall include this field if it was able to determine the SFN of the NR cell at the time of measurement.

End of Changes