3GPP TSG-RAN WG2 #102	Tdoc R2-1807919

Busan, Korea, 21st – 25th May 2018

Agenda Item:	10.4.1.3.5

Source:	Ericsson

Title:	Inter-RAT cell selection upon RRC Re-establishment

Document for:	Discussion

# Introduction

In LTE, timer T311 is started upon the initiation of the RRC connection re-establishment procedure. UE shall stop the timer when it selects a suitable LTE cell or a cell using another RAT. Upon the expiry of T311, the UE informs the higher layer and enters RRC_IDLE.

For NR, inter-RAT cell selection upon RRC Re-establishment has not been discussed. That is reflected in the following FFS captured in the TP on connection control [2]:

Editor’s Note: FFS how to treat inter-RAT cell selection

This contribution addresses that FFS and proposes a way forward.

# Discussion

In LTE, the timer T311 is started upon the initiation of the RRC Connection Re-establishment procedure. As part of the initiation, the UE perform cell selection in accordance with the cell selection process as specified in TS 36.304.

If the UE finds a suitable EUTRA cell, the UE stops the timer T311 and initiates the transmission of an RRCConnectionReestablishmentRequest message (procedure also applies the UE returns to PCell) i.e. UE tries to resume the connection by continuing the re-establishment procedure. However, if the UE selects an inter-RAT cell instead the UE performs the actions upon leaving RRC_CONNECTED with release cause 'RRC connection failure'. In other words, the UE basically goes to RRC_IDLE.

In the TP on connection control for 38.331 a similar behaviour is assumed for the actions following an NR cell selection while T311 is running. However, as shown below, the case where the UE selects an inter-RAT cell is not described yet.

***************************************************************************************************************************

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

1>	stop timer T311;

1>	start timer T301;

1>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4

NOTE:	This procedure applies also if the UE returns to the source PCell.

Editor’s Note: FFS how to treat inter-RAT cell selection

***************************************************************************************************************************

In theory, it could be possible to perform an inter-RAT RRC Re-establishment especially if LTE is capable of interpret the UE AS Context identifier and possibly resume the SRBs and DRBs without necessarily entering RRC_IDLE first. Notice that to certain extent, that would be possible as long as RAN based handover is supported, which is the case if LTE is connected to 5GC. To certain extent, such an optimization relates to previous discussions on the support of inter-RAT mobility in RRC_INACTIVE between LTE/5GC and NR cells. In RAN2#101 in Athens:

=>	RAN2 will not specify inter-RAT mobility between LTE and NR in INACTIVE state in Rel-15. Can be considered for Rel-16 (to be concluded by RAN plenary)

=>	RAN2 aim to define INACTIVE in LTE and NR in a way that this can be possible to introduce in future.

Agreement

1: 	Inter-RAT re-selection from NR INACTIVE  to an LTE/5GC cell, UE moves to Idle and informs NAS to trigger NAS recovery.

In our view, considering that the support for inter-RAT RRC_INACTIVE mobility is postponed, it also makes sense to not optimize inter-RAT re-establishment in Rel-15. Hence, if the UE selects to an LTE cell connected to EPC or 5GC while T311 is running, the UE perform actions upon going to RRC_IDLE with release cause ‘RRC connection failure’. Consequently, the UE releases all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity for all established RBs; Then, it indicates the release of the RRC connection to upper layers together with the release cause.

As in LTE, upon performing inter-RAT cell selection while T311 is running the UE perform actions upon going to RRC_IDLE with release cause ‘RRC connection failure’. Consequently, the UE releases all radio resources (e.g. RLC, MAC, PDCP, etc.) and indicates the release of the RRC connection to upper layers together with the release cause.

# Conclusion

Based on the discussion in section 2 we propose the following:

- As in LTE, upon performing inter-RAT cell selection while T311 is running the UE perform actions upon going to RRC_IDLE with release cause ‘RRC connection failure’. Consequently, the UE releases all radio resources (e.g. RLC, MAC, PDCP, etc.) and indicates the release of the RRC connection to upper layers together with the release cause.

# References

3GPP TS 36.331, Radio Resource Control (RRC), V15.0.1 (2018-01).

R2-1805355, CR on Connection Control, Ericsson, 3GPP TSG-RAN WG2 #101bis, Sanya, P.R. of China, 16th – 20th April 2018.