3GPP TSG-RAN WG2 #102	R2-1807984
Busan, Korea, 21-25 May 2018	(revision of R2-1805537)

Agenda Item:	10.4.1.3.8

Source: 	Huawei, HiSilicon

Title:  	Procedures for enabling security per bearer

Document for:	Discussion and Decision

# 1. Introduction

At the RAN2 ad hoc of June 2017, it was observed (e.g. in [1]) that SA3 expect to support security on a per PDU session basis.  Further discussion in SA3#88 ([3], endorsed) and SA3#89 ([5], endorsed) reached several conclusions relevant to RAN2:

(#4) It is agreed to have a single UP confidentiality algorithm.

(#5) It is agreed to have a single UP integrity protection algorithm (excluding discussion about no MAC-I) in phase 1, but not precluding per PDU in phase 2.

(#6) For single connectivity, it is agreed to use AS SMC for negotiating UP confidentiality algorithm, similar to LTE, meaning that all PDU sessions will be protected using the same UP integrity protection algorithm. Dual connectivity case is FFS and will be based on RAN2 progress.

(#7) For single connectivity, it is agreed to use AS SMC for negotiating UP integrity protection algorithm. Dual connectivity case is FFS and will be based on RAN2 progress.

(#8) It is agreed to use RRC signalling (similar to dual connectivity) for negotiating UP integrity protection activation, meaning that UP integrity is activated per DRB. This allows UP integrity to be activated for one DRB while not activated for another DRB. (requirements for UP integrity need to adapted).

(#9) It is agreed to use RRC signalling (similar to dual connectivity) for negotiating UP confidentiality activation, meaning that UP confidentiality is activated per DRB. This allows UP confidentiality to be activated for one DRB while not activated for another DRB. (requirements for UP confidentiality need to adapted).

(#10) It is agreed that same algorithms are used for RRC security and user plane security in phase 1. This does not preclude different algorithms in later phases.

Further work at SA3#90 ([6], endorsed) captured these principles into TS 33.501 in the form of requirements on the AS Security Mode Command procedure (section 6.7.4 of TS 33.501) and the UP security activation procedure (section 6.6.2 of TS 33.501).

This paper discusses the implications of the SA3 decisions and the needed procedures.

# 2. Discussion

  - 2.1. Security activation at DRB granularity

In accordance with the SA3 decision, security activation should be specified at DRB granularity (and then the network can be responsible for assuring consistent settings at the PDU session level).  The requirement from TS 33.501 is that “AS UP integrity protection and ciphering activation shall be done as part of the DRB addition procedure using RRC Connection Reconfiguration procedure”.  A corresponding message flow is provided in TS 33.501, as shown in Figure 1.

Figure 1: UP security activation (from TS 33.501, Figure 6.6.2-1)

This decision includes ciphering as well as integrity, and it was accordingly agreed at RAN2#101bis to have an indication of ciphering in the PDCP-Config structure similar to the existing integrity indication.  This could be done in a backward compatible fashion through the extension container, or non-backward compatibly by adding a field in the main body; considering the existence of other non-backward compatible changes from RAN2#101bis, we suggest that it is not important to maintain backward compatibility.

Proposal 1: The security setting for ciphering activation is added to PDCP-Config at the same level as the currently existing integrity indication.

Similar to integrity protection, we suggest that changing the setting of ciphering for a DRB should be an unusual operation.  It can be supported by DRB release/add procedure using a different DRB ID; if it is necessary to change the setting while maintaining the DRB ID, it seems sufficient to have it supported at reconfiguration with sync.

Proposal 2: Ciphering for a DRB can be enabled/disabled during reconfiguration with sync or by DRB release/add procedure.

At this writing the DRB portion of PDCP-Config is not extensible; we suggest that it would be preferable to have it be extensible, to accommodate the possibility of a future release with additional per-DRB settings such as an integrity algorithm (another non-backward-compatible change).

Proposal 3: The DRB structure in PDCP-Config should be extensible.

  - 2.2. Security Mode Command

Per the SA3 agreements above, it is agreed to use the AS SMC to indicate the user plane ciphering and integrity algorithms.  It is also agreed that for Rel-15 the same algorithms are used for the control and user planes.  This means that for this release, the SMC only needs to indicate a single pair of algorithms (as it does in LTE), trigger key derivation, and trigger activation of security on the control plane (only).  In essence the SMC keeps the same functions it has today.  SA3 provided a message flow as shown in Figure 2.

Figure 2: AS SMC procedure (from TS 33.501, Figure 6.7.4-1)

Accordingly, we propose for RAN2 to capture the SA3 decisions as follows.

Proposal 4: Maintain the structure of the SMC from LTE, i.e. it contains the per UE algorithm identifiers that will be used for both control and user plane.

Proposal 5: As in LTE, receiving the SMC activates security on SRBs, configures the algorithms for both control and user plane security, and triggers derivation of the AS control plane keys (KUPint and KUPenc may be derived at this time or as needed, when a DRB with integrity protection or ciphering is configured).

If a future release introduced algorithm selection at PDU session granularity, as suggested by the SA3 agreements, it should be possible to use the SMC to indicate default algorithms and override them on a per-bearer basis for bearers associated with PDU sessions using different algorithms (which would require some extension to the DRB structure in PDCP-Config).  The network would be responsible for coordinating the assignment of algorithms so that the different bearers of a PDU session were consistent.

# 3. Conclusion

This document raises the following proposals:

Proposal 1: The security setting for ciphering activation is added to PDCP-Config at the same level as the currently existing integrity indication.

Proposal 2: Ciphering for a DRB can be enabled/disabled during reconfiguration with sync or by DRB release/add procedure.

Proposal 3: The DRB structure in PDCP-Config should be extensible.

Proposal 4: Maintain the structure of the SMC from LTE, i.e. it contains the per UE algorithm identifiers that will be used for both control and user plane.

Proposal 5: As in LTE, receiving the SMC activates security on SRBs, configures the algorithms for both control and user plane security, and triggers derivation of the AS control plane keys (KUPint and KUPenc may be derived at this time or as needed, when a DRB with integrity protection or ciphering is configured).

# 4. References

- R2-1706845, “NR Security: DRB based Enabling/Disabling of UP Security”, Samsung, RAN2 ad hoc June 2017

- TR 33.899, “Study on the security aspects of the next generation system”

- S3-172164, “Discussion on AS security setup and negotiation”, NTT DOCOMO, SA3#88

- TS 38.331 v1.0.0

- S3-173516, “Agreements on user plane (UP) security”, Ericsson et al., SA3#89

# 5. TP

### 5.3.4	Initial security activation

#### 5.3.4.1	General

Figure 5.3.4.1-1: Security mode command, successful

Figure 5.3.4.1-2: Security mode command, failure

The purpose of this procedure is to activate AS security upon RRC connection establishment.

#### 5.3.4.2	Initiation

RAN initiates the security mode command procedure to a UE in RRC_CONNECTED. RAN applies the procedure when only SRB1 is established, i.e. prior to establishment of SRB2 and/ or DRBs.

#### 5.3.4.3	Reception of the SecurityModeCommand by the UE

The UE shall:

1>	derive the KgNB key, as specified in TS 33.501 [11], if not already derived;

1>	derive the KRRCint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

1>	request lower layers to verify the integrity protection of the SecurityModeCommand message, using the algorithm indicated by the integrityProtAlgorithm as included in the SecurityModeCommand message and the KRRCint key;

1>	if the SecurityModeCommand message passes the integrity protection check:

2>	derive the KRRCenc key associated with the cipheringAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

2>	configure lower layers to apply ciphering using the indicated algorithm and the KRRCenc key after completing the procedure, i.e. ciphering shall be applied to all subsequent RRC messages received and sent by the UE, except for the SecurityModeComplete message which is sent unciphered;

2>	configure lower layers to apply integrity protection using the indicated algorithm and the KRRCint key immediately, i.e. integrity protection shall be applied to all subsequent RRC messages received and sent by the UE, including the SecurityModeComplete message;

2>	consider AS security to be activated;

1>	else:

2>	continue using the configuration used prior to the reception of the SecurityModeCommand message, i.e. neither apply integrity protection nor ciphering;

2>	submit the SecurityModeFailure message to lower layers for transmission, upon which the procedure ends;

##### […]

##### 5.3.5.6.5	DRB addition/modification

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

2>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

2>	if the received pdcp-Config contains integrityProtection set to enabled:

3> derive the key KUPint, if not already derived;

2>	if the received pdcp-Config contains ciphering set to enabled:

3> derive the key KUPenc, if not already derived;

2>	if the UE is operating in EN-DC:

3>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KUPenc) associated with the KeNB/S-KgNB as indicated in keyToUse;

2>	else:

3>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KUPenc and KUPint) associated with the KgNB;

2>	if the DRB was configured with the same eps-BearerIdentity either by NR or E-UTRA prior to receiving this reconfiguration:

3> associate the established DRB with the corresponding eps-BearerIdentity;

2> else:

3>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration:

2>	if reestablishPDCP is set:

3>	if the UE is operating in EN-DC:

4>	configure the PDCP entity of this RadioBearerConfig to apply the ciphering algorithm and KUPenc key associated with the KeNB/S-KgNB as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

3>	else:

4>	if the received pdcp-Config contains ciphering set to enabled:

5>	configure the PDCP entity of this RadioBearerConfig to apply the ciphering algorithm and KUPenc key associated with the KgNB, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE on this DRB;

4>	else:

5>	configure the PDCP entity of this RadioBearerConfig not to apply ciphering on this DRB;

4>	if the received pdcp-Config contains integrityProtection set to enabled:

5>	configure the PDCP entity of this RadioBearerConfig to apply the integrity algorithm and KUPint key associated with the KgNB, i.e. the integrity configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE on this DRB;

4>	else:

5>	configure the PDCP entity of this RadioBearerConfig not to apply integrity protection on this DRB;

3>	re-establish the PDCP entity of this DRB as specified in 38.323 [5], section 5.1.2;

2>	else, if recoverPDCP is set:

3>	trigger the PDCP entity of this DRB to perform data recovery as specified in 38.323;

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config.

NOTE 1:	Removal and addition of the same drb-Identity in a single radioResourceConfig is not supported. In case drb-Identity is removed and added due to reconfiguration with sync or re-establishment with the full configuration option, the network can use the same value of drb-Identity.

NOTE 2:	When determining whether a drb-Identity value is part of the current UE configuration, the UE does not distinguish which RadioBearerConfig and DRB-ToAddModList that DRB was originally configured in.  To re-associate a DRB with a different key (KeNB to S-KeNB or vice versa), the network provides the drb-Identity value in the (target) drb-ToAddModList and sets the reestablishPDCP flag. The network does not list the drb-Identity in the (source) drb-ToReleaseList.

NOTE 3:	When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. It does that e.g. by triggering a reconfiguration with sync of the cell group hosting the old RLC entity or by releasing the old RLC entity.

NOTE 4: 	In this specification, UE configuration refers to the parameters configured by NR RRC unless otherwise stated.

#### […]

#### –	SecurityModeCommand

The SecurityModeCommand message is used to command the activation of AS security.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

SecurityModeCommand message

-- ASN1START

-- TAG-SECURITYMODECOMMAND-START

SecurityModeCommand ::= 			SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions						CHOICE {

securityModeCommand						SecurityModeCommand-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

SecurityModeCommand-IEs ::= SEQUENCE {

securityAlgorithmConfig					SecurityAlgorithmConfig,

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

-- TAG-SECURITYMODECOMMAND-STOP

-- ASN1STOP

#### –	SecurityModeComplete

The SecurityModeComplete message is used to confirm the successful completion of a security mode command.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SecurityModeComplete message

-- ASN1START

-- TAG-SECURITYMODECOMPLETE-START

SecurityModeComplete ::= 			SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions						CHOICE {

securityModeComplete					SecurityModeComplete-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

SecurityModeComplete-IEs ::= SEQUENCE {

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

-- TAG-SECURITYMODECOMPLETE-STOP

-- ASN1STOP

#### –	SecurityModeFailure

The SecurityModeFailure message is used to indicate an unsuccessful completion of a security mode command.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SecurityModeFailure message

-- ASN1START

-- TAG-SECURITYMODEFAILURE-START

SecurityModeFailure ::= 			SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions						CHOICE {

securityModeFailure						SecurityModeFailure-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

SecurityModeFailure-IEs ::= SEQUENCE {

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

-- TAG-SECURITYMODEFAILURE-STOP

-- ASN1STOP

#### […]

#### –	PDCP-Config

The IE PDCP-Config is used to set the configurable PDCP parameters for signalling and data radio bearers.

PDCP-Config information element

-- ASN1START

-- TAG-PDCP-CONFIG-START

PDCP-Config ::=			SEQUENCE {

drb						SEQUENCE {

discardTimer			ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300, ms500, ms750, ms1500, infinity} 													OPTIONAL, -- Cond Setup

pdcp-SN-SizeUL			ENUMERATED {len12bits, len18bits}				OPTIONAL, -- Cond Setup2

pdcp-SN-SizeDL			ENUMERATED {len12bits, len18bits}				OPTIONAL, -- Cond Setup2

headerCompression		CHOICE {

notUsed					NULL,

rohc					SEQUENCE {

maxCID					INTEGER (1..16383)				DEFAULT 15,

profiles				SEQUENCE {

profile0x0001			BOOLEAN,

profile0x0002			BOOLEAN,

profile0x0003			BOOLEAN,

profile0x0004			BOOLEAN,

profile0x0006			BOOLEAN,

profile0x0101			BOOLEAN,

profile0x0102			BOOLEAN,

profile0x0103			BOOLEAN,

profile0x0104			BOOLEAN

},

drb-ContinueROHC			BOOLEAN

},

uplinkOnlyROHC			SEQUENCE {

maxCID					INTEGER (1..16383)				DEFAULT 15,

profiles				SEQUENCE {

profile0x0006			BOOLEAN

},

drb-ContinueROHC			BOOLEAN

},

...

},

integrityProtection		ENUMERATED { enabled }														OPTIONAL,	-- Cond ConnectedTo5GC

ciphering				ENUMERATED { enabled }														OPTIONAL,	-- Cond ConnectedTo5GC

statusReportRequired	ENUMERATED { true }															OPTIONAL,	-- Cond Rlc-AM

outOfOrderDelivery		BOOLEAN,

...

}																											OPTIONAL,	-- Cond DRB

moreThanOneRLC			SEQUENCE {

primaryPath				SEQUENCE {

cellGroup				CellGroupId											OPTIONAL,	-- Need R

logicalChannel			LogicalChannelIdentity								OPTIONAL	-- Need R

},

ul-DataSplitThreshold	UL-DataSplitThreshold 									OPTIONAL, -- Cond SplitBearer

pdcp-Duplication			ENUMERATED { true }													OPTIONAL	-- Need R

}																										OPTIONAL, -- Cond MoreThanOneRLC

t-Reordering				ENUMERATED {

ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220,

ms240, ms260, ms280, ms300,	ms500, ms750, ms1000, ms1250, ms1500, ms1750, ms2000, ms2250, ms2500, ms2750,

ms3000, spare28, spare27, spare26, spare25, spare24, spare23, spare22, spare21, spare20,

spare19, spare18, spare17, spare16, spare15, spare14, spare13, spare12, spare11, spare10, spare09,

spare08, spare07, spare06, spare05, spare04, spare03, spare02, spare01 }		OPTIONAL, -- Need S

...

}																										OPTIONAL, -- Cond MoreThanOneRLC

t-Reordering				ENUMERATED {

ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220,

ms240, ms260, ms280, ms300,	ms500, ms750, ms1000, ms1250, ms1500, ms1750, ms2000, ms2250, ms2500, ms2750,

ms3000, spare28, spare27, spare26, spare25, spare24, spare23, spare22, spare21, spare20,

spare19, spare18, spare17, spare16, spare15, spare14, spare13, spare12, spare11, spare10, spare09,

spare08, spare07, spare06, spare05, spare04, spare03, spare02, spare01 }		OPTIONAL, -- Need S

...

}

UL-DataSplitThreshold ::= ENUMERATED {

b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800,

b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400,

b6553600, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

-- TAG-PDCP-CONFIG-STOP

-- ASN1STOP

PDCP-Configfield descriptions

ciphering Indicates whether or not ciphering is configured for this radio bearer. The value of ciphering for a DRB can only be changed using reconfiguration with sync.

discardTimer Value in ms of discardTimer specified in TS 38.323 [5]. Value ms50 corresponds to 50 ms, ms100 corresponds to 100 ms and so on.

drb-ContinueROHC Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment. This field is set to true only in case of reconfiguration with sync where the PDCP termination point is not changed.

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. If uplinkOnlyROHC is configured, the UE shall apply the configure ROHC profile(s) in uplink (there is no header compression in downlink). ROHC can be configured for any bearer type. ROHC should be configured at reconfiguration involving PDCP re-establishment if the RB was previously configured with ROHC. Header compression should not be configured when out-of-order delivery is allowed for PDCP SDUs.

integrityProtection Indicates whether or not integrity protection is configured for this radio bearer. The value of integrityProtection for a DRB can only be changed using reconfiguration with sync. FFS: text to indicate where to find the key.

maxCID Indicates the value of the MAX_CID parameter as specified in TS 38.323 [5] FFS: need to specify something with respect to UE capabilities.

outOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured. Out-of-order delivery is configured only when the radio bearer is established

pdcp-Duplication Indicates whether or not uplink duplication is configured as specified in TS 38.323 [5]. This field is absent in this version of the specification.

pdcp-SN-Size PDCP sequence number size, 12 or 18 bits.

primaryPath Indicates the cell group ID and LCID of the primary RLC entity as specified in TS 38.323 clause 5.2.1 for UL data tranmission when more than one RLC entity is associated with the PDCP entity. In this version of the specification, only cell group ID corresponding to MCG is supported for SRBs.

pdcp-SN-Size PDCP sequence number size, 12 or 18 bits.

statusReportRequired For AM DRBs, indicates whether the DRB is configured to send a PDCP status report in the uplink, as specified in TS 38.323 [5]. For UL DRBs, the value shall be ignored by the UE.

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms0 corresponds to 0ms, value ms20 corresponds to 20ms, value ms40 corresponds to 40ms, and so on. When the field is absent the UE applies the value infinity.

ul-DataSplitThreshold Parameter specified in TS 38.323 [5]. Value b0 corresponds to 0 bits, value b100 corresponds to 100 bits, value b200 corresponds to 200 bits, and so on.

| Conditional presence | Explanation |
| --- | --- |
| DRB | This field is mandatory present when the corresponding DRB is being set up, not present for SRBs. Otherwise this field is optionally present, need M. |
| MoreThanOneRLC | This field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer with more than one associated logical channel and upon RRC reconfiguration with the association of an additional logical channel to the PDCP entity. Upon RRC reconfiguration when a PDCP entity is associated with multiple logical channels, this field is optionally present need M. Otherwise, this field is absent and all its included parameters are released. |
| Rlc-AM | The field is mandatory present upon setup of a PDCP entity for a radio bearer configured with RLC AM. Otherwise, the field is optionally present, need R. |
| Setup | The field is mandatory present in case of radio bearer setup. Otherwise the field is optionally present, need M. |
| SplitBearer | The field is optional present, need M, in case of radio bearer with more than one associated RLC mapped to different cell groups. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| ConnectedTo5GC | The field is optionally present, need R, if EN-DC is not configured, and absent if EN-DC is configured. |
| Setup2 | This field is mandatory present in case for radio bearer setup for RLC-AM and RLC-UM. This field is optionally present in case for handover and reestablishment for for RLC-UM..Otherwise, ths field is not present. |