3GPP TSG-WG2 Meeting #101bis 	R2-1807996

# Busan, Republic of Korea, 21st – 25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev |  | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to TDD-UL-DL configuration. |
| --- | --- |
| Source to WG: | Ericsson; NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (CR to be merged into Rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) tdd-UL-DL-ConfigurationCommon1 & tdd-UL-DL-ConfigurationCommon2: The TDD-UL-DL-ConfigCommon may be instantiated twice. The second pattern may only be present if the first is present. If the second pattern is present, it shall not contain the referenceSubcarrierSpacing (RAN1 agreed in their April meeting that "UE does not expect the reference SCS in TDD UL/DL configuration common and common2 to be different"). In that case, the referenceSubcarrierSpacing of the first pattern applies. Describing these dependencies properly in the current structure is unnecessarily complex. 2) dl-UL-TransmissionPeriodicity: The field is currently OPTIONAL but it is unclear how the UE behaves if it is absent. Furthermore, a need code is missing. 3) nrofDownlinkSlots, nrofUplinkSlots: The field is flagged marked as OPTIONAL but the field description does not explain what the UE does when it is absent. Furthermore, the need code is missing. 4) nrofDownlinkSymbols, nrofUplinkSymbols: The field is OPTIONAL with Need R. The field description explains that there are no downlink/uplink symbols when the respective field is absent. However, the value range starts anyway at 0 which means obviously the same as absence. Hence, there are two options for signalling the same information which should generally be avoided. 5) TDD-UL-DL-ConfigCommon and TDD-UL-DL-ConfigDedicated: Extension markers are missing (as highlighted by the Rapporteur). 6) slotSpecificConfigurationsToAddModList: The field description lists FFSs which have either been already addressed (add/mod list; introduce constants) or which may not require further discussion right now (signalling overhead). 7) TDD-UL-DL-Config: References to L1 specs: The current field descriptions refer to the old L1 parameter names ("Corresponds to L1 parameter ...") and point to 38.211. However, the fields are actually used in 38.213 and in that specification the old names therein have been updated to match the RRC field names. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) tdd-UL-DL-ConfigurationCommon1 & tdd-UL-DL-ConfigurationCommon2: Remove tdd-UL-DL-ConfigurationCommon2 and create the two patterns instead inside the TDD-UL-DL-ConfigCommon IE. 2) dl-UL-TransmissionPeriodicity: Make the field mandatory to ensure that the NW provides. 3) nrofDownlinkSlots, nrofUplinkSlots: Make the field mandatory present. 4) nrofDownlinkSymbols, nrofUplinkSymbols: To keep the handling of the information simple and aligned with the other fields (see 3), make the fields mandatory present and adjust the field description accordingly. 5) TDD-UL-DL-ConfigCommon and TDD-UL-DL-ConfigDedicated: Add the extension markers. 6) slotSpecificConfigurationsToAddModList: Remove obsolete FFSs 7) TDD-UL-DL-Config: References to L1 specs: Remove the references to the old L1 parameter names and correct the specification number to 38.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in signalling (changes 1, 2, 3) remain; unnecessary signalling overhead and complexity due to unused optionality bits (changes 2, 3, 4); extensibility is not ensured (change 5); open issues that were already addressed seem to remain in the spec (change 6) |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### ======================== Section provided for reference only ========================

#### –	SIB1

Editor’s Note: Targeted for completion in Sept 2018. Not used in EN-DC.

SIB1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. It also contains radio resource configuration information that is common for all UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH and BR-BCCH

Direction: Network to UE

SIB1 message

-- ASN1START

-- TAG-SIB1-START

SIB1 ::=		SEQUENCE {

-- FFS / TODO: Add other parameters.

ssb-PositionsInBurst				SEQUENCE {

inOneGroup							BIT STRING (SIZE (8)),

groupPresence						BIT STRING (SIZE (8))												OPTIONAL -- Cond above6GHzOnly

},

ssb-PeriodicityServingCell			ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare1, spare2},

ss-PBCH-BlockPower					INTEGER (-60..50),

uplinkConfigCommon					UplinkConfigCommon														OPTIONAL,

supplementaryUplink					SEQUENCE {

uplinkConfigCommon					UplinkConfigCommon													OPTIONAL

-- FFS: Add additional (selection) criteria determining when/whether the UE shall use the SUL frequency

}																											OPTIONAL, -- Cond SUL

tdd-UL-DL-ConfigurationCommon		TDD-UL-DL-ConfigCommon													OPTIONAL, -- Cond TDD

pdcch-ConfigCommon					PDCCH-ConfigCommon														OPTIONAL,

pucch-ConfigCommon					PUCCH-ConfigCommon														OPTIONAL,

lateNonCriticalExtension			OCTET STRING															OPTIONAL,

nonCriticalExtension				SEQUENCE{}																OPTIONAL

}

-- TAG-SIB1-STOP

-- ASN1STOP

SIB1 field descriptions

groupPresence For above 6 GHz: indicates which groups of SSBs is present

inOneGroup Indicates the presence of the up to 8 SSBs in one group

ss-PBCH-BlockPower TX power that the NW used for SSB transmission. The UE uses it to estimate the RA preamble TX power. (see 38.213, section 7.4)

ssb-PeriodicityServingCell The SSB periodicity in msec for the rate matching purpose (see 38.211, section [7.4.3.1])

ssb-PositionsInBurst Time domain positions of the transmitted SS-blocks in an SS-Burst-Set (see 38.213, section 4.1)

supplementaryUplink FFS: How to indicate the FrequencyInfoUL for the SUL

| Conditional Presence | Explanation |
| --- | --- |
| TDD | The field is optionally present, Need R, for TDD cells; otherwise it is not present. |

#### ======================== Section provided for reference only ========================

#### –	ServingCellConfigCommon

The ServingCellConfigCommon IE is used to configure cell specific parameters of a UE’s serving cell. The IE contains parameters which a UE would typically acquire from SSB, MIB or SIBs when accessing the cell from IDLE. With this IE, the network provides this information in dedicated signalling when configuring a UE with a SCells or with an additional cell group (SCG). It also provides it for SpCells (MCG and SCG) upon reconfiguration with sync.

ServingCellConfigCommon information element

-- ASN1START

-- TAG-SERVING-CELL-CONFIG-COMMON-START

ServingCellConfigCommon ::=			SEQUENCE {

physCellId							PhysCellId													OPTIONAL, -- Cond HOAndServCellAdd,

frequencyInfoDL						FrequencyInfoDL												OPTIONAL, -- Cond InterFreqHOAndServCellAdd

initialDownlinkBWP					BWP-DownlinkCommon											OPTIONAL,	-- Cond ServCellAdd

uplinkConfigCommon					UplinkConfigCommon											OPTIONAL, -- Cond ServCellAdd-UL

supplementaryUplinkConfig					UplinkConfigCommon									OPTIONAL, -- Cond ServCellAdd-SUL

n-TimingAdvanceOffset				ENUMERATED { n25560, n39936 }								OPTIONAL,-- Need S

ssb-PositionsInBurst				CHOICE {

shortBitmap							BIT STRING (SIZE (4)),

mediumBitmap						BIT STRING (SIZE (8)),

longBitmap							BIT STRING (SIZE (64))

}																								OPTIONAL, -- Need R,

ssb-periodicityServingCell			ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }		OPTIONAL,	-- Need S

dmrs-TypeA-Position					ENUMERATED {pos2, pos3},

lte-CRS-ToMatchAround				SetupRelease { RateMatchPatternLTE-CRS } 								OPTIONAL,	-- Need M

rateMatchPatternToAddModList		SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern		OPTIONAL, -- Need N

rateMatchPatternToReleaseList		SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId		OPTIONAL, -- Need N

subcarrierSpacing					SubcarrierSpacing														OPTIONAL, -- Need S

tdd-UL-DL-ConfigurationCommon		TDD-UL-DL-ConfigCommon													OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower					INTEGER (-60..50),

...

}

UplinkConfigCommon ::=				SEQUENCE {

frequencyInfoUL						FrequencyInfoUL												OPTIONAL, -- Cond InterFreqHOAndServCellAdd

initialUplinkBWP					BWP-UplinkCommon											OPTIONAL	-- Cond ServCellAdd

}

-- TAG-SERVING-CELL-CONFIG-COMMON-STOP

-- ASN1STOP

UplinkConfigCommon field descriptions

frequencyInfoUL Absolute uplink frequency configuration and subcarrier specific virtual carriers.

initialUplinkBWP The initial uplink BWP configuration for a SpCell (PCell of MCG or SCG). Corresponds to L1 parameter 'initial-UL-BWP'. (see 38.331, section FFS_Section).

ServingCellConfigCommon field descriptions

dmrs-TypeA-Position Position of (first) DL DM-RS (see 38.211, section 7.4.1.1.1)

initialDownlinkBWP The initial downlink BWP configuration for a SpCell (PCell of MCG or SCG).

longBitmap bitmap for above 6 GHz

lte-CRS-ToMatchAround Parameters to determine an LTE CRS pattern that the UE shall rate match around.

mediumBitmap bitmap for 3-6 GHz

n-TimingAdvanceOffset The N_TA-Offset to be applied for random access on this serving cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency rangeof this serving cell. See 38.133, table 7.1.2-2.

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the nested bitmaps. Rate match patterns defined here on cell level apply only to PDSCH of the same numerology. Corresponds to L1 parameter 'Resource-set-cekk' (see 38.214, section 5.1.2.2.3)

shortBitmap bitmap for sub 3 GHz

ss-PBCH-BlockPower TX power that the NW used for SSB transmission. The UE uses it to estimate the RA preamble TX power. (see 38.213, section 7.4)

ssb-periodicityServingCell The SSB periodicity in msec for the rate matching purpose. If the field is absent, the UE applies the value ms5. (see 38.211, section [7.4.3.1])

ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks in an SS-burst. The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted. Corresponds to L1 parameter 'SSB-Transmitted' (see 38.213, section 4.1)

subcarrierSpacing Subcarrier spacing of SSB. Used only for non-initial access (e.g. SCells, PCell of SCG). If the field is absent the UE shall assume the default value of the band. Only the values 15 or 30 kHz (<6GHz), 120 or 240 kHz (>6GHz) are applicable.

tdd-UL-DL-ConfigurationCommon A cell-specific TDD UL/DL configuration. see 38.213, section 11.1

| Conditional Presence | Explanation |
| --- | --- |
| HOAndServCellAdd | This field is mandatory present for inter-cell handover and upon serving cell (PSCell/SCell) addition. Otherwise, the field is absent, Need M. |
| InterFreqHOAndServCellAdd | This field is mandatory present for inter-frequency handover and upon serving cell (PSCell/SCell) addition. Otherwise, the field isoptionally present, Need M. |
| ServCellAdd | This field is mandatory present upon serving cell addition (for PSCell and SCell). It is optionally present, Need M otherwise. |
| ServCellAdd-UL | This field is mandatory present upon serving cell addition (for PSCell and SCell) provided that the serving cell is configured with uplink. It is optionally present, Need M otherwise. |
| ServCellAdd-SUL | This field is mandatory present upon serving cell addition (for PSCell and SCell) provided that the serving cell is configured with a supplementary uplink. It is optionally present, Need M otherwise. |
| TDD | The field is optionally present, Need R, for TDD cells; otherwise it is not present. |

#### ======================== Actual changes below ========================

#### –	TDD-UL-DL-Config

The TDD-UL-DL-Config IEs determines the Uplink/Downlink TDD configuration. There are both, UE- and cell specific IEs.

TDD-UL-DL-Config information element

-- ASN1START

-- TAG-TDD-UL-DL-CONFIG-START

TDD-UL-DL-ConfigCommon ::=			SEQUENCE {

referenceSubcarrierSpacing			SubcarrierSpacing															,

pattern1							TDD-UL-DL-Pattern,

pattern2							TDD-UL-DL-Pattern															OPTIONAL, -- Need R

...

}

TDD-UL-DL-Pattern ::= 				SEQUENCE {

dl-UL-TransmissionPeriodicity		ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10}				,

nrofDownlinkSlots					INTEGER (0..maxNrofSlots)													,

nrofDownlinkSymbols					INTEGER (0..maxNrofSymbols-1)

nrofUplinkSlots						INTEGER (0..maxNrofSlots)													,

nrofUplinkSymbols					INTEGER (0..maxNrofSymbols-1)

}

TDD-UL-DL-ConfigDedicated ::=		SEQUENCE {

slotSpecificConfigurationsToAddModList		SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotConfig			OPTIONAL, -- Need N

slotSpecificConfigurationsToreleaseList		SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex			OPTIONAL, -- Need N

...

}

TDD-UL-DL-SlotConfig ::=			SEQUENCE {

slotIndex							TDD-UL-DL-SlotIndex,

symbols								CHOICE {

allDownlink							NULL,

allUplink							NULL,

explicit							SEQUENCE {

nrofDownlinkSymbols					INTEGER (1..maxNrofSymbols-1)										OPTIONAL,	-- Need S

nrofUplinkSymbols					INTEGER (1..maxNrofSymbols-1)										OPTIONAL	-- Need S

}

}

}

TDD-UL-DL-SlotIndex ::=				INTEGER (0..maxNrofSlots-1)

-- TAG-TDD-UL-DL-CONFIG-STOP

-- ASN1STOP

TDD-UL-DL-ConfigCommon field descriptions

referenceSubcarrierSpacing Reference SCS used to determine the time domain boundaries in the UL-DL pattern which must be common across all subcarrier specific carriers, i.e., independent of the actual subcarrier spacing using for data transmission. Only the values 15 or 30 kHz (<6GHz), 60 or 120 kHz (>6GHz) are applicable. Corresponds to L1 parameter 'reference-SCS' (see 38.211, section FFS_Section)

TDD-UL-DL-Pattern field descriptions

dl-UL-TransmissionPeriodicity Periodicity of the DL-UL pattern. (see 38.213, section FFS_Section)

nrofDownlinkSlots Number of consecutive full DL slots at the beginning of each DL-UL pattern. (see 38.213, Table 4.3.2-1)

nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot following the last full DL slot (as derived from nrofDownlinkSlots). The value 0 indicates that there is no partial-downlink slot. (see 38.213, section FFS_Section).

nrofUplinkSlots Number of consecutive full UL slots at the end of each DL-UL pattern. (see 38.213, Table 4.3.2-1)

nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot preceding the first full UL slot (as derived from nrofUplinkSlots). The value 0 indicates that there is no partial-uplink slot. (see 38.213, section FFS_Section)

pattern1 A recurring uplink- downlink TDD pattern.

pattern2 A second recurring uplink- downlink TDD pattern. If configured, the cell specific DL/UL pattern is a concatenation of pattern1 and pattern2.

TDD-UL-DL-ConfigDedicated field descriptions

slotSpecificConfigurationsToAddModList The slotSpecificConfiguration allows overriding UL/DL allocations provided in tdd-UL-DL-configurationCommon.

TDD-UL-DL-SlotConfig field descriptions

nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot identified by slotIndex. If the field is absent the UE assumes that there are no leading DL symbols. (see 38.213, section FFS_Section)

nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot identified by slotIndex. If the field is absent the UE assumes that there are no trailing UL symbols. (see 38.213, section FFS_Section)

slotIndex Identifies a slot within a dl-UL-TransmissionPeriodicity (given in tdd-UL-DL-configurationCommon)

symbols The direction (downlink or uplink) for the symbols in this slot. "allDownlink" indicates that all symbols in this slot are used for downlink; "allUplink" indicates that all symbols in this slot are used for uplink; "explicit" indicates explicitly how many symbols in the beginning and end of this slot are allocated to downlink and uplink, respectively.