3GPP TSG-RAN2 Meeting #102 	R2-1808529

# Busan, South Korea, 21st - 25th May 2018				         Resubmission of R2-1806045

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | 0228 | rev | 1 | Current version: | 14.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of PDCP data recovery for UM DRBs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  |  | Date: |  | 2018-04-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In current specification, the PDCP data recovery is only applied to the AM DRBs. This is because the split bearer for UM DRB is not suppored so far. However, in RAN2#101 meeting, RAN2 agreed that the split bearer can be applied to the UM DRBs. Hence, the PDCP data recovery procedure for UM DRB is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the PDCP data recovery is performed, the PDCP entity for UM DRB retransmits the PDCP PDUs which sucssessful delivery has not been confirmed by lower layers. Impact analysis: Impacted functionality: PDCP data recovery procedure Inter-operability: If the network is implemented according to this CR while the UE is not, the receiving side of PDCP entity for UM DRB does not receive the PDCP PDUs transmitted by the re-established RLC entity. If the UE is implemented according to this CR while the network is not, the receiving side of PDCP entity for UM DRB does not receive the PDCP PDUs transmitted by the re-established RLC entity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Data recovery procedure for UM DRBs is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS36.322 CR 0134 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of changes

5.9	PDCP Data Recovery procedure

When upper layers request a PDCP Data Recovery for a radio bearer, the UE shall:

-	if the radio bearer is configured by upper layers to send a PDCP status report in the uplink (statusReportRequired [3]), compile a status report as described in subclause 5.3.1, and submit it to lower layers as the first PDCP PDU for the transmission;

-	perform retransmission of all the PDCP PDUs previously submitted to re-established AM RLC entity or re-established UM RLC entity in ascending order of the associated COUNT values from the first PDCP PDU for which the successful delivery has not been confirmed by lower layers.

After performing the above procedures, the UE shall follow the procedures in subclause 5.1.1.

End of changes