3GPP TSG-RAN WG2 Meeting #101bis	R2-1808636

Sanya, China, 16th – 20th April 2018

Agenda Item	: 9.20 (INOBEAR-Core)

Source	: LG Electronics Inc.

Title	: LCID extension for increased number DRB

Document for	: Discussion and Decision

1.	Introduction

In RAN2#101bis, RAN2 discussed how to extend the LCID and how many bits of eLCID is used. The conclusion are recaptured below:

Agreements

=>	Use of special value of LCID to indicate presence of extended LCID field after the octet containing the legacy LCID field.  eLCID is in addition to the exiting LCID space.

=>	10000 is used for extended LCID field

=>	eLCID is 5 bits long

=>	DRB configuration info should have an extension to address higher logical channel ID values

This document addresses remaining issues on extension of LCID field.

2.	Discussion

Issue 1. Length of eLCID field

In the last meeting, it was agreed that eLCID field is 5 bits long, however it has not been captured yet to give some more time to check if it is ok.

Having 5 bits of eLCID field, 32 more eLCID index are introduced. However, it seems that there are different understanding on this. For example, in the current MAC running CR, the table 6.2.1-2 shows that eLCID field is 6 bits long and provides 32 more LCID index by always setting the MSB of eLCID index to 1, i.e., half of the lower values of eLCID index is unused at all.

Table 6.2.1-2 Values of LCID and eLCID for UL-SCH

| Index | LCID and eLCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 | Identity of the logical channel |
| 01011 | CCCH |
| 01100 | CCCH |
| 01101-01111 | Reserved |
| 10000 | Extended logical channel indicator |
| 10001-10011 | Reserved |
| 10100 | Recommended bit rate query |
| 10101 | SPS confirmation |
| 10110 | Truncated Sidelink BSR |
| 10111 | Sidelink BSR |
| 11000 | Dual Connectivity Power Headroom Report |
| 11001 | Extended Power Headroom Report |
| 11010 | Power Headroom Report |
| 11011 | C-RNTI |
| 11100 | Truncated BSR |
| 11101 | Short BSR |
| 11110 | Long BSR |
| 11111 | Padding |
| 100000-100110 | Identity of logical channel |
| 101000-111111 | Reserved |

The presence of eLCID field is indicated by LCID field set to 10000. If LCID field is set to 10000, it means that eLCID index, instead of LCID index, is to be used to indicate the type or identity of logical channel of corresponding MAC subPDU.

For example, CCCH is indicated by 00000 of LCID index. If there is a CCCH SDU, LCID field will be included which is set to 00000. When there is a CCCH SDU, it is not an option to set LCID index to 10000 and include eLCID field indicating CCCH, e.g., 000000.

It would be redundant and waste of half of lower values of eLCID to use the same value of LCID index and eLCID index for the same purpose. In this light, if RAN2 considers 32 more values are sufficient for eLCID index, it is straightforward that eLCID field is 5 bits long, as already agreed.

Proposal 1. Logical channel instance or type of MAC CE which are already indicated by LCID index are not indicated by eLCID index.

Proposal 2. eLCID field is 5 bits long and eLCID index is from 00000 to 11111.

With Proposal 1 and 2, we need a separate table for LCID index and eLCID index.

Proposal 3. Separate tables are used, i.e., one for LCID index and the other one for eLCID index.

Issue 2. Number of newly added identities of logical channel

Currently in MAC running CR, 7 more identities are added by allocating 100000-100110 to Identity of logical channel. Given that logicalChannelIdentity from 4 to 10 is used for DRB (3 is for SRB1bis), we need at least 8 more identities for logical channel. The question is whether we need more than 8 identities.

It was explained by the rapporteur that the scope of INOBEAR is to support up to 15 DRB and duplication is not a consideration in this WI. However, packet duplication in CA is to be supported in R15 LTE HRLLC which is to be completed in June 2018. If packet duplication in CA is used, it is straightforward that each logical channel of duplicated DRB is identified by unique identity of logical channel. Otherwise, RLC/PDCP processing cannot be done properly. If we stick to total 15 identity of logical channel, it means that up to 7 DRBs can be duplicated in CA. However, in the current HRLLC WI, up to 8 DRB needs to be duplicated in CA and hence duplication MAC CE includes total 8 bits of D field.

We understand that it is a clean way in general to focus on the scope of the item, however, we should take care of parallel ongoing items which may have impact each other.

It wouldn’t be a critical sacrifice to have more than 8 identities of logical channel in eLCID index because we are going to have at least 32 more eLCID index now. Thus, we propose to determine the number of identities of logical channel by considering packet duplication supported in R15 HRLLC.

Proposal 4. Decide the number of newly added identities of logical channel in eLCID index by considering the packet duplication in R15 HRLLC.

Issue 3. Identity of logical channel

Currently, LCID index 00001 - 01010 are used to indicate Identity of logical channel and RRC configures logicalChannelIdentity which can be [3…10], i.e., there is correspondence between LCID index and logicalChannelIdentity.

RRC doesn’t take care of LCID index in MAC. Thus, from RRC point of view, it seems natural the value of newly added logicalChannelIdentity are [11…x]. In this case, eLCID index for newly added Identity of logical channel would be [01011…xxxxx].

Another approach suggested during [101bis#79] is to add logicalChannelIdentity from [32…x]. The reason was to consider maximum LCID index which is used in MAC is 31. Accordingly, in the current MAC running CR, eLCID index for newly added Identity of logical channel is [100000…xxxxxx]. However, if 5 bits of eLCID index is used as proposed in Proposal 1, there is no way to have a correspondence between logicalChannelIdentity and eLCID index.

Although LCID stands for Logical Channel ID, LCID index is not only to indicate Identity of logical channel but also to indicate the type of MAC control element. Therefore, we don’t need to make a linkage between LCID/eLCID index and logicalChannelIdentity. Thus, we propose that newly added logicalChannelIdentity are [11…x] and [01011…xxxxx] are used for identity of logical channel in eLCID index.

Proposal 5. In RRC, newly added logicalChannelIdentity are [11…x]. In MAC, [01011…xxxxx] are used for identity of logical channel in eLCID index

3.	Conclusion

In this document, we discussed the remaining issues for INOBEAR and proposed:

Proposal 1. Logical channel instance or type of MAC CE which are already indicated by LCID index are not indicated by eLCID index.

Proposal 2. eLCID field is 5 bits long and eLCID index is from 00000 to 11111.

Proposal 3. Separate tables are used, i.e., one for LCID index and the other one for eLCID index.

Proposal 4. Decide the number of newly added identities of logical channel in eLCID index by considering the packet duplication in R15 HRLLC.

Proposal 5. In RRC, newly added logicalChannelIdentity are [11…x]. In MAC, [01011…xxxxx] are used for identity of logical channel in eLCID index