3GPP TSG-RAN WG2 Meeting #102			Tdoc R2-1808926

Busan, Korea, 21st – 25th May 2018.

Title:	[DRAFT] Reply LS on PDCCH common search space configurations

Response to:	LS (R2-1807819) on PDCCH common search space configurations from RAN1

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	NTT DOCOMO, INC. (to be RAN2)

To:	RAN1

Cc:

Contact Person:

Name:	Hideaki Takahashi

Tel. Number:	+81 46 840 3190

E-mail Address:	hideaki dot takahashi dot vx at nttdocomo dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 would like to thank RAN1 for their LS on PDCCH common search configuration. Based on the observations provide in the LS, RAN2 agreed:

1.	Introduce a new field for controlResourceSet0 as an INTEGER (0..15). The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Add a condition that this field may only be configured in the initial BWP (BWP#0). Clarify that "The UE acquires CORESET#0 irrespective of the currently active BWP as described in 38.xxx, section x.x.x.”. Other CORESETs are configured as defined currently.

2.	Introduce a new field for searchSpaceZero as an INTEGER (0..15). The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Add a condition that this field may only be configured in the initial BWP (BWP#0). Clarify that "The UE acquires SearchSpaceZero irrespective of the currently active BWP as described in 38.xxx, section x.x.x.”.

3.	For other search spaces: Add a "duration" to the SearchSpace. The "duration" defines the number of slots that a SearchSpace lasts in every occasion (at every period).

3a.	Add the missing periodicities as described in R2-1806895.

4.	Consider splitting the definition of the field pdcch-ConfigSIB1 in MIB into CORESET and SearchSpace if that is OK for RAN1.

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to take the above agreements into account.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 NR-AH 1807	2nd – 6th July 2018	Montreal,	Canada.

TSG-RAN WG2 Meeting #103	20th – 24th August 2018	Gothenburg, Sweden.