R 3GPP TSG-RAN WG2 Meeting #102	R2-1808948

# Busan, S. Korea, 21 – 25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0018 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 CR on combined bearer type and termination point change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is currently an inconsistency in 37.340 when it comes to bearer type changes allowed. In Annex A, the table of bearer type changes with and without security key change shows that bearer type change is supported together with termination point change. However, in section 8.3, only change of bearer type OR bearer termination point are listed, which is only a subset of the allowed bearer type changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified desciption in section 8.3 for Bearer type change together with termination point change. Impact analysis Impacted functionality: EN-DC Inter-operability: 1. If the UE is implemented according to this CR but the network is not, there is no backward compatibility issue; 2. If the network is implemented according to this CR but the UE is not, there is backward compatibility issue if the UE is not able to support simultaneous bearer type and termination point change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may not support bearer type change together with termination point change. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

# 8	Bearer handling aspects

## 8.3	Bearer type change

In MR-DC, all the possible bearer type change options are supported:

-	MCG bearer to/from split bearer;

-	MCG bearer to/from SCG bearer;

-	SCG bearer to/from split bearer.

Bearer termination point change is supported for all bearer types, and can be performed with or without bearer type change:

-	MN terminated bearer to/from SN terminated bearer.

For EN-DC:

-	when the security key is changed for a bearer, the associated PDCP and RLC entities are re-established, while MAC behavior might depend on the solution selected by the network, e.g. MAC reset, change of LCID, etc. (see Annex A);

-	for MCG bearer, split bearer and SCG bearer, during handover MCG/SCG PDCP and RLC are re-established and MCG/SCG MAC is reset;

-	if a bearer type change happens through handover procedure then for MCG bearer, split bearer and SCG bearer, MCG/SCG PDCP/RLC are re-established and MCG/SCG MAC is reset;

-	if a bearer type change happens through SN change procedure, then SN terminated PDCP /RLC are re-established and SCG MAC is reset;

-	one step (direct) bearer type change between MN terminated bearer types without using the handover procedure is supported;

-	one step (direct) bearer type change between SN terminated bearer types without using the handover or SN change procedure is supported;

-	for bearer type change between MCG bearer and SCG bearer MAC is not reset; the original RLC entity is released and new RLC entity is established;

-	PDCP version change for a DRB is only performed via handover procedure, using a release and add of the DRBs (in a single message) or full configuration;

-	One step (direct) bearer type change with PDCP version change is supported;

-	Upon bearer type change from MCG bearer to SCG bearer or from split bearer to SCG bearer, the associated LTE RLC entity is first re-established and then released.

NOTE:	L2 handling for bearer type change in EN-DC is also summarized in Annex A.

END OF CHANGES