3GPP TSG-RAN WG2 #102	R2-1809111

# Busan, Republic of Korea, 21st – 25th May 2018

Agenda Item:	10.XXX

Source:	Ericsson

Title:	Offline #34 on Msg3

Document for:	Discussion, Decision

# Introduction

This email discussion discusses Msg3 sizes. First proposal for UL-CCCH structure is given and then coding of each message is discussed separately.

Agreements

If RAN1 is not able to provide a larger min grant size than 56bits with same coverage :

-	RRC messages for msg3 will be defined to fit 56 bit min grant size

-	Possibility to have a larger min grant size in a cell will be supported and indicated via system info.

-	To be progress offline (focus first on the coding and 3i, 5 as a second step of the discussion):

1/	Coding for RRCRequest to fit the 56bit requirement assuming 5G-S-TMSI is split between msg3 and msg5

2/	Coding for RRCRe-establishmentRequest to fit the 56bit requirement

3/	Coding for ResumeRequest to fit the 56bit requirement and details of truncation of I-RNTI;

3i/ 	Can also consider the option that if system information indicates the minimum grant size is 56bits then resume procedure will fall back to RRC connection request (i.e. resume does not work in this cell).

4/	Coding for ResumeRequest to fit the 72 bits

5/	Associated procedure text

=>	Offline discussion based on above agreements. Offline discussion #34, Ericsson

Assumptions for the parameters

- I-RNTI size 40 bits

- 5G-S-TMSI in Msg3 is 40 bits

- EstablishmentCause 4 bits

- Reestablishment cause 3 bits

- Short-MAC-I 16 bits

- Reestablishment ID: CRNTI + PCI

Possible optimizations proposed by companies:

- Remove critical extensions in the message level to end up to 56/72 bits for Msg3

- Move 1 bit from 5G-S-TMSI from Msg3 to Msg5. Move 1 bit from I-RNTI from Msg3 to Msg5.

- Combine RRCReestablishmentRequest and RRCResumeRequest message

- Delete 1 bit from Short-MAC-I

From these solutions, the first one seems to be most acceptable.

Agree to remove critical extensions in the message level to end up to 56/72 bits for Msg3.

Agree signalling structure as presented in this TP

Additional changes 1) changing establishment cause from 4 to 3 bits and 2) move 1 bit in 5G-STMSI from Msg3 to Msg5 can be discussed later.

# Discussion

## UL-CCH structure

In the following, UL-CCH structure is proposed.

UL-CCCH-MessageType ::= CHOICE {

c1						CHOICE {

rrcSetupRequest					RRCSetupRequest,

rrcResumeRequest				RRCResumeRequest,

rrcReestablishmentRequest		RRCReestablishmentRequest,

rrcSystemInfoRequest			RRCSystemInfoRequest

},

messageClassExtension	SEQUENCE {}

}

## RRCSetupRequest

RRCSetupRequest is discussed below.

#### –	RRCSetupRequest

The RRCSetupRequest message is used to request the establishment of an RRC connection.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCSetupRequest message

-- ASN1START

-- TAG-RRCSETUPREQUEST-START

RRCSetupRequest ::=			SEQUENCE {

rrcSetupRequest							RRCSetupRequest-IEs

}

RRCSetupRequest-IEs ::=		SEQUENCE {

ue-Identity							InitialUE-Identity,

establishmentCause					EstablishmentCause

}

InitialUE-Identity ::=				CHOICE {

ng-5g-s-tmsi						NG-5G-S-TMSI,

randomValue							BIT STRING (SIZE (40))

}

-- FFS Which additional cause values are supported: delayTolerantAccess, MO videop, MO SMS, etc.

EstablishmentCause ::=				ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall,spare1, spare2, spare3,spare4,

spare5, spare6, spare7, spare8, spare9, spare10}

-- TAG-RRCSETUPREQUEST-STOP

-- ASN1STOP

With the coding presented above, the size of the message is:

| RRCConnectionRequest |  |
| --- | --- |
| IE | Size (bits) |
| Message Structure bits | 3 |
| ue-Identity | 41 |
| establishmentCause | 4 |
| MAC Header | 8 |
| Spare | 0 |
| Total | 56 |

## RRCResumeRequest

#### –	RRCResumeRequest

The RRCResumeRequest message is used to request the resumption of a suspended RRC connection or perform an RNA update.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCResumeRequest message

-- ASN1START

-- TAG-RRCRESUMEREQUEST-START

RRCResumeRequest ::=	SEQUENCE {

rrcResumeRequest			RRCResumeRequest-IEs

}

RRCResumeRequest-IEs ::=		SEQUENCE {

resumeIdentity							CHOICE {

i-RNTI-Value							I-RNTI-Value,

truncated-i-RNTI						BIT STRING (SIZE (24))

},

resumeMAC-I									BIT STRING (SIZE (16)),

resumeCause									ResumeCause

}

-- FFS Which additional resume causes are supported: delayTolerantAccess, RNA Update, periodic RNA Update, MO video, MO SMS, etc.

ResumeCause ::=				ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,										mo-Data, mo-VoiceCall, rna-Update, spare1, spare2, spare3, spare4,

spare5, spare6, spare7, spare8, spare9}

I-RNTI-Value ::=						BIT STRING (SIZE(40))

-- TAG-RRCRESUMEREQUEST-STOP

-- ASN1STOP

RRCResumeRequest field descriptions

resumeCause Provides the resume cause for the RRC connection resume request as provided by the upper layers.

resumeIdentity UE identity to facilitate UE context retrieval at gNB.

resumeMAC-I Authentication token to facilitate UE authentication at gNB.

With the coding presented above, the size of the Msg3 is:

| RRC connection resume request |  |
| --- | --- |
| IE | Size (bits) |
| Message Structure | 3 |
| ue-Identity | 41 |
| resumeCause | 4 |
| shortMac-I | 16 |
| Spare | 0 |
| MAC Header | 8 |
| Total | 72 |

| RRC connection resume request |  |
| --- | --- |
| IE | Size (bits) |
| Message Structure | 3 |
| ue-Identity | 25 |
| resumeCause | 4 |
| shortMac-I | 16 |
| Spare | 0 |
| MAC Header | 8 |
| Total | 56 |

## RRCReestablishmentRequest

In the following, coding for RRCReestablishmentRequest is presented below.

- –	RRCReestablishmentRequest

The RRCReestablishmentRequest message is used to request the reestablishment of an RRC connection.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCReestablishmentRequest message

-- ASN1START

-- TAG-RRCREESTABLISHMENTREQUEST-START

RRCReestablishmentRequest ::= SEQUENCE {

RRCReestablishmentRequest-IEs

}

RRCReestablishmentRequest-IEs ::= SEQUENCE {

ue-Identity						ReestabUE-Identity,

reestablishmentCause			ReestablishmentCause,

spare							BIT STRING (SIZE (1))

}

ReestabUE-Identity ::=				SEQUENCE {

c-RNTI								RNTI-Value,

physCellId							PhysCellId,

shortMAC-I							ShortMAC-I

}

ReestablishmentCause ::=			ENUMERATED {

reconfigurationFailure, handoverFailure,

otherFailure, spare1}

-- TAG-RRCREESTABLISHMENTREQUEST-STOP

-- ASN1STOP

With the coding presented above, the size of the message is:

| RRC connection re-establishment request |  |
| --- | --- |
| IE | Size (bits) |
| Message Structure | 3 |
| ue-Identity | 26 |
| reestablishmentCause | 2 |
| shortMac-I | 16 |
| Spare | 1 |
| MAC Header | 8 |
| Total | 56 |

# Conclusion