3GPP TSG-RAN WG2 Meeting #102	R2-1809126

Busan, Korea, 21-25 May 2018

Title:	LS on RNAU without context relocation

# Release:	Rel-15

Work Item:	NR_newRAT

Attachment: 	None

Source:	Qualcomm Incorporated (to be RAN2)

To:	RAN3

# Contact Person:

# Name:	Ozcan Ozturk

E-mail Address:	oozturk@qti.qualcomm.com

# 1. Overall Description:

RAN2 has discussed the RAN Area Update (RNAU) procedure in Inactive state and has agreed on the following steps:

- The UE in Connected state receives a ciphered and integrity protected RRC release message on SRB1 with suspend configuration, I-RNTI, and NCC on SRB1 to move the UE to Inactive state.

- When RNAU is triggered, the UE in Inactive state sends to the current serving gNB the resume message (i.e. msg3 with RNAU cause value) on SRB0, which also includes a resume MAC-I and I-RNTI .

- The current serving gNB sends the UE information in msg3 (I-RNTI and resume MAC-I) to the last serving gNB for verification.

- After UE verification and context re-location, the UE receives the ciphered and integrity protected RRC release message (i.e. msg4) with suspend configuration, I-RNTI, and NCC on SRB1 from the current serving gNB which sends the UE back to Inactive state.

After discussing and confirming the above procedure with SA3, RAN2 has concluded that the current serving gNB can send msg4 in step-4 only after the current serving gNB receives a new security key from AMF (at the end of path switch procedure). This was necessary to fulfill the 2-hop security requirement from SA3 during RNAU procedure.

Since waiting for the completion of path switch procedure introduces additional latency and UE power consumption during RNAU procedure, RAN2#102 has agreed on the following:

RAN2 preference for RAN3 to support RNAU without context relocation, under the assumption that there is no major RAN2 impact.

The above agreement implies that the RRC release message in step-4 will be generated by the last serving gNB and delivered by the current serving gNB.

RAN2 respectfully requests RAN3 to take the above into account in their work.

# 2. Actions:

RAN2 respectfully requests RAN3 to take the above into account in their work.

# 3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting 	AH-1807 	2 - 6 July 2018	Montreal, Canada

TSG RAN WG2 Meeting #103	20 – 24 August 2018	Gothenburg, Sweden