3GPP TSG-RAN WG2 Meeting #103 	R2-1811071

# Gothenberg, Sweden, 20th August 2018 – 24th August 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0298 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on bwp-InactivityTimer when PDCCH indicating UL grant is received |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2 #101bis meeting, it has agreed based on R2-1806442 : BWP inactivity timer is started/restarted for both scheduled and scheduling cell at cross carrier scheduling It’s observed that in the latest TS 38.321 (v15.2.0), the bwp-InactivityTimer is started/restarted when a PDCCH indicating uplink grant for the active UL BWP is received for both paired spectrum and unpaired spectrum. For unpaired (TDD) spectrum, since the UL BWP and DL BWP with the same index are always linked, it makes sense to start/restarts the bwp-InactivityTimer associated with the active DL BWP if a PDCCH indicating uplink grant for the active UL BWP is received, otherwise the active UL BWP may be switched due to the expiry of bwp-InactivityTimer and the linkage for TDD. However, for paired spectrum (FDD), it has not been discussed whether a PDCCH indicating uplink grant for the UL active BWP should start/restart the bwp-InactivityTimer. For paired (FDD) spectrum, the UL and DL BWPs are independent and no linked unless when CB-RACH is iniated. It’s not necessary to start/restart the bwp-InactivityTimer associated with the active DL BWP if a PDCCH indicating uplink grant for the active UL BWP is received. In some cases, this behaviour will increase power comsumption especially for a UL heavy traffic UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The bwp-InactivityTimer is not started or restarted when PDCCH indicating UL grant for the active UL BWP for paired spectrum. Four changes are made: - a PDCCH is received on the active DL BWP; - a PDCCH indicating downlink assignemtn is received for the active DL BWP; - a PDCCH indicating uplink grant is received for the active UL BWP for unpaired spectru; - a MAC PDU is transmitted in a configured uplink grant for unpaired spectrum; Impact analysis Impacted functionality: BWP operation. Inter-operability: - If the network is implemented according to the CR and the UE is not there are no inter-operability problems. - If the UE is implemented according to the CR and the network is not, there are no inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will unnecessarily starts/restarts the bwp-InactivityTimer when a PDCCH indicating uplink grant is received for the UL BWP or if a MAC PDU is transmitted in a configured uplink grant for paired spectrum. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

### 5.15	Bandwidth Part (BWP) operation

[Omit]

If the bwp-InactivityTimer is configured, the MAC entity shall for each activated Serving Cell:

1>	if the defaultDownlinkBWP is configured, and the active DL BWP is not the BWP indicated by the defaultDownlinkBWP; or

1>	if the defaultDownlinkBWP is not configured, and the active DL BWP is not the initialDownlinkBWP:

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active DL BWP; or

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment is received for the active DL BWP; or

2> if a PDCCH addressed to C-RNTI or CS-RNTI indicating uplink grant is received for the active UL BWP for unpaired spectrum; or

2>	if a MAC PDU is transmitted in a configured uplink grant for unpaired spectrum or received in a configured downlink assignment:

3>	if there is no ongoing random access procedure associated with this Serving Cell; or

3>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in subclauses 5.1.4 and 5.1.5):

4>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

[Omit]

End of change