3GPP TSG-RAN WG2 #103	Tdoc R2-1811144

Göteborg, Sweden, 20th – 24th August 2018	Resubmission of R2-1808159

Agenda Item:	9.22

Source:	Ericsson

Title:	Discarding configured grants and assignments when receiving RRC reconfiguration of SPS-Config

Document for:	Discussion, Decision

# Introduction

With the recent additions to SPS there is a need to revisit the RRC reconfiguration of SPS and to come up with a more secure and future proof solution.

# Discussion

Two meetings ago, there was some confusion on the concepts we presented. Here, we first give an overview of the intended outcomes with a precise terminology used in the specification TS 36.321.

## Background

In order to use SPS, a two-step approach by RRC and DCI is used. RRC configures SPS C-RNTI and SPS interval while the actual physical resources are indicated by DCI. A summary of what is feasible in the current specification is shown in the following figure with black arrows.

By TS 36.321 terminology, SPS is used in the following way,

- eNB “enables” SPS by sending an RRC IE sps-Config in which for example the SPS interval is configured;

- If enabled, eNB activates SPS by transmitting an UL grant or DL assignment, which is scrambled with SPS C-RNTI and with some fixed special fields in DCI to indicate this is an activation. These grants and assignments reoccur every SPS interval, and they are stored as a “configured uplink grant” or “configured downlink assignment”.

- To deactivate SPS, the “configured uplink grant” or “configured downlink assignment” can be “cleared” by transmitting an DCI SPS release scrambled with SPS C-RNTI.

- A “configured uplink grant” or “configured downlink assignment” can also be “disabled by RRC” by an RRC SPS release in the sps-Config message, which then results in the corresponding “configured uplink grant” or “configured downlink assignment” being “discarded”.

## Issues in legacy

The red part “Re-config by RRC” in the figure above is not feasible currently, since it is written in the TS 36.331 spec that E-UTRAN does not RRC reconfigure sps-Config while there is a configured uplink grant or configured downlink assignment. The reason why SPS is specified in this way can be found in [3], [4] and [5]. Thus, the text in 36.331 prohibits eNB from RRC reconfiguration of sps-Config while there is an active configured grant or assignment. This means the behaviour if a UE receives an RRC reconfiguration of sps-Config while there is a corresponding configured grant or assignment is unspecified.

UE behaviour is unspecified when receiving RRC reconfiguration of sps-Config while there is a corresponding configured grant or assignment.

This means that to reconfigure a different SPS interval, the eNB has to go through the two steps of first “clearing” the grants by DCI and then re-config by RRC. Clearing a grant by DCI introduces a small risk of eNB and UEs having a mismatch on the state of SPS in the UE. Increasing the reliability of deactivation of SPS is possible by using some extra time to listen if, at the next SPS occasion, the UE does not send anything. Alternatively, eNB can discard the grant by RRC and then configure a new sps-Config, but this also introduces extra latency.

RRC reconfiguration of SPS may be delayed when there is a configured grant or assignment.

Here, we discuss a scenario where we need to have this functionality, i.e., to “discard” the corresponding “configured uplink grant” or “configured downlink assignment” (if there is one) and then apply the new configuration when receiving an RRC reconfiguration of sps-Config.

After release 8 there have been only minor corrections to the SPS functionality until release 14 where new functionality was introduced in latency reduction WI [1] (UL SPS scheduling intervals down to 1 ms and possibilities for a MAC control element for SPS confirmation of activation/deactivation) and the V2V WI [2] (sidelink SPS). With the new functionalities added in release 14, it becomes more likely that eNB needs to RRC reconfigure the sps-Config to support different services to the users and to manage resources between users.

One example of a scenario where SPS interval needs to be reconfigured is a UE configured with 40 ms SPS service for a VoIP conversation (a typical VoIP coder produces packets every 20 ms, but QoS is usually good enough with 40 ms scheduling interval) and later the UE starts a low latency service that needs 1 ms SPS, the eNB then need to RRC reconfigure the SPS interval. Vice versa, a UE configured with 1 ms SPS service starts a VoIP conversation (which can be supported on the 1 ms SPS) and if the low latency service becomes inactive, the eNB might need to reconfigure the SPS interval to 40 ms to free up resources for other low latency users and to conserve the UE battery.

New SPS functionalities introduced in release 14 increases the need for eNB to RRC reconfigure SPS while there is a corresponding configured downlink assignment or configured uplink grant.

## Solution

To avoid the unspecified error case (Observation 1) and avoid delay of RRC reconfiguration for SPS (Observation 2) when RRC reconfiguration for SPS becomes more common (Observation 3) we propose:

At the reception of sps-Config, the MAC Entity shall discard any corresponding configured downlink assignment or configured uplink grant.

To “discard any corresponding configured downlink assignment or configured uplink grant” is the same action taken as when the MAC entity receives an sps-Config release. This means there will be no stored configured grant or assignment after an RRC reconfiguration of sps-Config. To start using the new SPS parameters, a new activation by scheduling the UE using SPS-RNTI is needed.

Proposed CRs for Rel 15 are available in [6] and [7].

# Conclusion

In section 2 we made the following observations:

Observation 1	UE behaviour is unspecified when receiving RRC reconfiguration of sps-Config while there is a corresponding configured grant or assignment.

Observation 2	RRC reconfiguration of SPS may be delayed when there is a configured grant or assignment.

Observation 3	New SPS functionalities introduced in release 14 increases the need for eNB to RRC reconfigure SPS while there is a corresponding configured downlink assignment or configured uplink grant.

Based on the discussion in section 2 we propose the following:

Proposal 1	At the reception of sps-Config, the MAC Entity shall discard any corresponding configured downlink assignment or configured uplink grant.

# References

RP-162026, WI: L2 latency reduction techniques for LTE, Ericsson, RAN#71, 2016-03-07

RP-161603, WI: Support for V2V services based on LTE sidelink, LG Electronics, Huawei, HiSilicon, CATT, RAN#73, 2016-09-19

R2-116388, Clarification on SPS occasions, Samsung, Ericsson, Huawei, HiSilicon, LG Electronics Inc., Nokia Siemens Networks, Qualcomm, ST-Ericsson, ZTE, RAN2#76, 2011-11-14

R2-115891, Details on SPS reconfiguration, Panasonic, RAN2#76, 2011-11-14

R2-116392, SPS reconfiguration, Huawei, HiSilicon, CATT, Nokia Siemens Networks, ZTE, MediaTek, Panasonic, RAN2#76, 2011-11-14

36321_CR1264r2_(Rel-15)_R2-1811145 - Discarding configured grants and assignments when receiving RRC reconfiguration of SPS-Config, Ericsson, RAN2#103, 2018-08

36331_CR3366r2_(Rel-15)_R2-1811146 - Discarding configured grants and assignments when receiving RRC reconfiguration of SPS-Config, Ericsson, RAN2#103, 2018-08