3GPP TSG-RAN WG2 Meeting #103	R2-18111436

# Gothenburg, Sweden, 20 - 24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0044 | rev | 2 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Signalling Aspects of Slicing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rapporteur (Nokia) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The signalling aspects of Slicing contains a few errors: - UE described being in active mode. - Tracking Area used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes were made: - RRC_CONNECTED used instead. - Registration Area used instead. Impact analysis Impacted functionality: Slicing. Inter-operability: None expected as the corresponding requirements are captured in Stage 3 specifications (3GPP TSs 25.502, 38.331, 38.413 and 38.423) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The signalling overview of Slicing contains errors and innacuracies. Stage 2 file size is larger than necessary. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 16.3.4	Signalling Aspects

#### 16.3.4.1	General

In this sub clause, signalling flows related to the realization of network slicing in the NG-RAN are given.

#### 16.3.4.2	CN Instance and NW Slice Selection

RAN selects the AMF based on a Temp ID or assistance information provided by the UE.

Figure 16.3.4.2-1: AMF instance selection

In case a Temp ID is not available, the NG-RAN uses the NSSAI provided by the UE at RRC connection establishment to select the appropriate AMF instance (the information is provided after MSG3 of the random access procedure). If such information is also not available, the NG-RAN routes the UE to one of the configured default AMF instances.

The NG-RAN uses the list of supported S-NSSAI(s) previously received in the NG Setup Response message when selecting the AMF with the assistance information. This list may be updated via the AMF Configuration Update message.

#### 16.3.4.3	UE Context Handling

Following the initial access, the establishment of the RRC connection and the selection of the correct AMF, the AMF establishes the complete UE context by sending the Initial Context Setup Request message to the NG-RAN over NG-C. The message contains the S-NSSAI(s) as part of the PDU session/s resource description. Upon successful establishment of the UE context and allocation of PDU resources to the relevant NW slice/s, the NG-RAN responds with the Initial Context Setup Response message.

Figure 16.3.4.3-1: Network Slice-aware Initial Context Setup

#### 16.3.4.4	PDU Session Handling

When new PDU sessions need to be established or existing ones modified or released, the 5GC requests the NG-RAN to allocate/release resources relative to the relevant PDU sessions by means of the PDU Session Setup/Modify/Release procedures over NG-C. In case of network slicing, S-NSSAI information is added per PDU session, so NG-RAN is enabled to apply policies at PDU session level according to the SLA represented by the network slice, while still being able to apply (for example) differentiated QoS within the slice.

NG-RAN confirms the establishment/modification/release of a PDU session associated to a certain NW slice by responding with the PDU Session Setup/Modify/Release Response message over the NG-C interface.

Figure 16.3.4.4-1: Network Slice-aware PDU Session Setup/Modify/Release

#### 16.3.4.5	Mobility

To make mobility slice-aware in case of Network Slicing, S-NSSAI is introduced as part of the PDU session information that is transferred during mobility signalling. This enables slice-aware admission and congestion control.

Both NG and Xn handovers are allowed regardless of the slice support of the target NG-RAN node i.e. even if the target NG-RAN node does not support the same slices as the source NG-RAN node. An example for the case of active mode mobility across different Registration Areas, is shown in Figure 16.3.4.5-1 for the case of 5GC involved handover and in Figure 16.3.4.5-2 for the case of Xn based handover.

Figure 16.3.4.5-1: CN based mobility across different Registration Areas

Figure 16.3.4.5-2: Xn based mobility across different Registration Areas