3GPP TSG-RAN WG2 Meeting #103                    R2-1811480

Gothenburg, Sweden, 20th - 24th August 2018

Source:	ZTE , Sanechips

Title:	Considering on the RA triggered BWP switch in case SUL is configured

Agenda Item:	10.3.1.4.3

Document for:	Discussion & Decision

Introduction

Based on the current specification, both the BWP switch and SUL/NUL selection may be performed before the selection of RA resources. However, in case SUL is configured, two UL BWPs can be activated simultaneously for SUL and NUL accordingly, and the UE’s behavior for RA triggered BWP switch is not clear in such case.

The target of this contribution is to clarify this issue and share our view on this aspect.

Discussion

According to the current spec, the description about RA triggered BWP switch and SUL selection can be found as follow:

---------------------------------------From 38.321f20----------------------------------------------------------------------

5.1.1 Random Access Procedure Initiation

/*omitted for short*/

1>	if the carrier to use for the Random Access procedure is explicitly signalled:

2>	select the signalled carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the signalled carrier.

1>	else if the carrier to use for the Random Access procedure is not explicitly signalled; and

1>	if the Serving Cell for the Random Access procedure is configured with supplementaryUplink; and

1>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

2>	select the SUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the SUL carrier.

1>	else:

2>	select the NUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the NUL carrier.

/*omitted for short*/

5.15 Bandwidth Part (BWP) operation

/*omitted for short*/

Upon initiation of the Random Access procedure on a Serving Cell, the MAC entity shall for this Serving Cell:

1>	if PRACH occasions are not configured for the active UL BWP:

2>	switch the active UL BWP to BWP indicated by initialUplinkBWP;

2>	if the Serving Cell is a SpCell:

3>	switch the active DL BWP to BWP indicated by initialDownlinkBWP.

1>	else:

2>	if the Serving Cell is a SpCell:

3>	if the active DL BWP does not have the same bwp-Id as the active UL BWP:

4>	switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP.

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

---------------------------------------From 38.321f20----------------------------------------------------------------------

Based on the description of BWP switch above, it seems the SUL case has not been taken into account in the RA triggered BWP switch operation.

ISSUE : The SUL case has not been taken into account in the RA triggered BWP switch operation.

For the RACH fallback due to the lack of RA resource, in case SUL is configured, the following three cases should be considered:

- Case1: There are RA resources configured on both the activated BWP of SUL and NUL.

- Case2: There is RA resources on the activated BWP of either SUL or NUL (i.e. only one carrier has RA resources)

- Case3: There is no RA resources on the activated BWP of neither the SUL nor NUL.

FOR CASE 1: There are RA resources configured on both the activated BWP of SUL and NUL

According to the current spec, for the SpCell, if the active DL BWP does not have the same bwp-Id as the active UL BWP, the UE should switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP. However, since the active BWP ID may be different in the SUL and UL, we think the SUL/NUL selection should be made before the BWP switch operation, otherwise, it is not clear which DL BWP should be switched to.

Observation 1: Since the active BWP ID may be different in the SUL and UL, the SUL/NUL selection should be made before the BWP switch operation. Otherwise, it is not clear which DL BWP should be switched to.

Based on the observation above, we give our proposal 1 as follow:

Proposal 1: The SUL/NUL selection should be made before the BWP switch operation, and the RA triggered BWP switch operation should be made for the selected carrier.

FOR CASE 2: There is RA resources on the activated BWP of either SUL or NUL (i.e. only one carrier has RA resources)

For the case 2, it should be clarified first that whether the RA resources is mandatory present on SUL active BWP. Considering the intention of SUL is to provide better UL coverage as a supplement of NUL, and the main use case is the RA procedure, in order to save the complexity, we propose to mandate the present of RA resources on all the BWP of SUL carrier, in case SUL is configured.

Proposal 2: RA resources for CBRA should be mandatory present on all the BWPs of SUL, in case SUL is configured. And the restriction should be captured in RRC specs.

With the proposal 2, the case 2 can be simplified to that: there is CBRA resources on SUL but no CBRA resources on UL. Since there is RA resource on SUL, it seems there is no need to trigger the BWP switch operation on the NUL.

Proposal 3: For CBRA, in case the SUL is configured but there is no CBRA resources on the active BWP of NUL, the SUL should be selected for the CBRA procedure.

For the CASE 3: There is no RA resources on the activated BWP of neither the SUL nor NUL

Based on the proposal 2, we think the case 3 should be eliminated.

Conclusion

Based on the analysis shared above, we give our observation and proposal as follow:

ISSUE : The SUL case has not been taken into account in the RA triggered BWP switch operation.

For the UE initiated RA procedure (e.g. SR triggered RA), the following three cases have been discussed in this paper:

- Case1: There are RA resources configured on both the activated BWP of SUL and NUL.

- Case2: There is RA resources on the activated BWP of either SUL or NUL (i.e. only one carrier has RA resources)

- Case3: There is no RA resources on the activated BWP of neither the SUL nor NUL.

For CASE 1:

Observation 1: Since the active BWP ID may be different in the SUL and NUL, the SUL/NUL selection should be made before the BWP switch operation, otherwise, it is not clear which DL BWP should be switched to.

Proposal 1: The SUL/NUL selection should be made before the BWP switch operation, and the RA triggered BWP switch operation should be made for the selected carrier.

For CASE 2&3

Proposal 2: RA resources for CBRA should be mandatory present on all the BWPs of SUL, in case SUL is configured. And the restriction should be captured in RRC specs.

Proposal 3: For CBRA, in case the SUL is configured but there is no CBRA resources on the active BWP of NUL, the SUL should be selected for the CBRA procedure.

The corresponding CR can be found in [2].

References

- RAN2-38.321f20

- R2-1811481	CR to the RA triggered BWP switch in case SUL is configured