3GPP TSG-RAN WG2 Meeting #103	R2-1811519

Gothenburg, Sweden, 20th - 24th August 2018

Agenda Item	: 10.4.5.2

Source		: CMCC

Title			: Further discussion on the immediate cell reselection issue

Document for	: Discussion and decision

- Introduction

The immediate or ping-pong cell reselection problem was confirmed in the last meeting [1][2][3]. However the solution has not been decided. In this contribution, we would discuss the possible solutions and further analysis the pros and cons of these solutions.

=> RAN2 confirm the immediate cell reselection issue and figure out solution next meeting.

- Discussion

The immediate cell reselection issue happens when a UE try to reselect to a target cell which fulfilling the S criteria according to the frequency specific Qrxlevmin/Qqualmin in camping cell’s SIB3/4/5 but not fulfilling the S criteria according to the target cell’s cell specific Qrxlevmin/Qqualmin in SIB1. The main reason is that the cell specific Q parameter is much higher than the frequency specific Q parameters.

To solve the immediate cell reselection issue, three solutions are raised from different angle::

- UE based solution

That is, it depends on UE implementation to decide whether cell specific or frequency specific Qrxlevmin/Qqualmin is used. As a result, some UEs with higher UE capability would use the cell specific Qrxlevmin/Qqualmin to evaluate neighbour cell’s quality by reading the neighbour cell’s SIB1, while some UEs may only use the frequency specific Qrxlevmin/Qqualmin that may not reflect the actual requirement of the target cell. The immediate cell reselection issue could not be solved at least for part of UEs since the UE does not need to read system information from neighbor cells for determining cell quality and ranking according to RAN2#101bis agreements [4].

- Network based solution A: introducing cell specific offset in camping cell’s SIB3/4/5

When evaluating neighbour cell’s RX level or quality, the neighbour cell specific offset Qrxlevminoffsetcell or Qqualminoffsetcell is considered.

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset + Qrxlevminoffsetcell)– Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset + Qqualminoffsetcell) - Qoffsettemp

In this case, neighbour cells’ Qrxlevminoffsetcell and Qqualminoffsetcell need to be broadcast in SIB3 (intra frequency cell reselection)/SIB4 (inter-frequency cell re-selection)/SIB5 (inter-RAT cell re-selection) of the camping cell. The specification impacts are small since only 96bits (3bits * 2*16) need to be introduced per SIB. The following is a revision example.

SIB3 information element

-- ASN1START

-- TAG-SIB3-START

SIB3 ::=		SEQUENCE {

intraFreqNeighCellList				IntraFreqNeighCellList		OPTIONAL,	-- Need N

intraFreqBlackCellList				IntraFreqBlackCellList		OPTIONAL,	-- Need N

...,

lateNonCriticalExtension				OCTET STRING				OPTIONAL

}

IntraFreqNeighCellList ::=		SEQUENCE (SIZE (1..maxCellIntra)) OF IntraFreqNeighCellInfo

IntraFreqNeighCellInfo ::=		SEQUENCE {

physCellId							PhysCellId,

q-OffsetCell						Q-OffsetRange,

ssb-ConfigMobility					SSB-ConfigMobility			OPTIONAL,

q-RxLevMinOffsetCell				INTEGER (1..8)				OPTIONAL	-- Need OP

q-QualMinOffsetCell					INTEGER (1..8)				OPTIONAL	-- Need OP

...

}

-- TAG-SIB3-STOP

-- ASN1STOP

- Network based solution B: Using a modified ranking process

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

Rs = Qmeas,s +Qhyst Rn = Qmeas,n -Qoffset- Qoffsetcell

For simplicity, assume Qhyst = 0dB, Qoffset = 0dB, frequency specific Qrxlevmin/Qqualmin = -116dBm.

When serving cell Qrxlevmin/Qqualmin = -116dBm, neighbour cell 1 Qrxlevmin/Qqualmin = -110dBm, neighbour cell 2 Qrxlevmin/Qqualmin = -116dBm, what Qoffsetcell will be for neighbour cell 1? 6dB? And what Qoffsetcell will be for neighbour cell 2? 0dB?

Taking the following examples with the assumption Qoffsetcell = 6dB for the neighbour cell 1 and Qoffsetcell = 0dB for the neighbour cell 2:

|  | Serving cell | Neighbour cell 1 | Neighbour cell 2 |
| --- | --- | --- | --- |
| Example 1 | Rs = Qmeas,s = -114dBm | Qmeas,n = -112dBm, then if Qoffsetcell = 6dB, Rn = Qmeas,n -Qoffset- Qoffsetcell = -118dBm | - |
|  | In this case, it seems that immediate cell reselection can be avoided |  |  |
| Example 2 | Rs = Qmeas,s = -114dBm | Qmeas,n = -108dBm, then if Qoffsetcell = 6dB, Rn = Qmeas,n -Qoffset- Qoffsetcell = -114dBm, | - |
|  | Better neighbour cell (i.e. Neighbour cell 1) cannot be reselected. |  |  |
| Example 3 | Rs = Qmeas,s = -100dBm | Qmeas,n = -96dBm, then if Qoffsetcell = 6dB, Rn = Qmeas,n -Qoffset- Qoffsetcell = -104dBm, | - |
|  | Better neighbour cell (i.e. Neighbour cell 1) cannot be reselected. |  |  |
| Example 4 | Rs = Qmeas,s = -114dBm | Qmeas,n = -96dBm, then if Qoffsetcell = 6dB, Rn = Qmeas,n -Qoffset- Qoffsetcell = -104dBm, | Qmeas,n = -100dBm, then if Qoffsetcell = 0dB, Rn = Qmeas,n -Qoffset- Qoffsetcell = -100dBm |
|  | Better neighbour cell (i.e. Neighbour cell 1) cannot be reselected. |  |  |

From these illustrations, it is obvious that the Qoffsetcell cannot resolve the problem without side effect.

In network based solution B, neighbour cells’ Qoffsetcell need to be broadcast in SIB3/4/5 of the camping cell. The specification impacts are small since 80bits (5bits *16) need to be introduced per SIB considering 32 Qoffsetcell values may be similar to the IE Q-OffsetRange.

Generally, the pros and cons of these possible solutions are as follows:

|  | UE based solution | Network based solution A: introducing cell specific offset in camping cell’s SIB3/4/5 | Network based solution B: Using a modified ranking process |
| --- | --- | --- | --- |
| Issue solved? | Partly NOT | YES | YES |
| Cell reselection performance | NO GAIN | BETTER | Partly WORSE May reselect a suboptimal neighbour cell. |
| Specification impact | NO | Small (96bits per SIB3/4/5) | Small (80bits per SIB3/4/5) |

Taking all these into consideration, we propose:

- Broadcast neighbour cell’s Qrxlevminoffsetcell and Qqualminoffsetcell in camping cell’s SIB3/4/5.

- Conclusion

In this contribution, we further analyzed the pros and cons of the solutions to the immediate cell reselection issue and have the following proposal.

- Broadcast neighbour cell’s Qrxlevminoffsetcell and Qqualminoffsetcell in camping cell’s SIB3/4/5.

In addition, we kindly ask RAN2 to agree the following TP in the Appendix.

4	References

- R2-1810760  Discussion on the immediate cell reselection issue, CMCC, RAN2#AH-1807

- R2-150546  Clarification for the UE behavior on cell reselection, CMCC, RAN2#89

- RAN2-NR-AH1807  Report from breakout session of idle/inactive mode procedures

- R2-1806202 Report from breakout session, 3GPP TSG-RAN WG2 #101bis, Sanya, China, 16th - 20th April 2018

Appendix A  Text Proposal for TS 38.304

#### 5.2.3.2	Cell Selection Criterion

The cell selection criterion S in normal coverage is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset + Qrxlevminoffsetcell)– Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset+ Qqualminoffsetcell) - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm). If the UE supports SUL frequency for this cell, Qrxlevmin is obtained from q-RxLevMin-sul, if present, in SIB1, else Qrxlevmin is obtained from q-RxLevMin in SIB1. |
| Qqualmin | Minimum required quality level in the cell (dB) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [9] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [9] |
| Qrxlevminoffsetcell | Cell specific offset to the signalled Qrxlevmin for the neighbour cell’s Srxlev evaluation |
| Qqualminoffsetcell | Cell specific offset to the signalled Qqualmin for the neighbour cell’s Squal evaluation |
| Pcompensation | If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB) |