3GPP TSG-RAN WG2 Meeting #103	Tdoc R2-1811615

Gothenburg, Sweden, 20th – 24th August 2018	Revision of R2-1810082

Agenda Item:	10.3.1.4.3

Source:	Ericsson

Title:	Solution to RA-RNTI ambiguity

Document for:	Discussion, Decision

# 1	Introduction

In the previous RAN2#102 meeting a possible ambiguity for RA-RNTI when two PRACH resource pools are configured were discussed. This is a relevant problem that needs to be solved in order for the MAC protocol to be functional. This problem is discussed further in this contribution along with proposals to resolve the ambiguity.

# 2	Discussion

In contribution [1], the problem of ambiguous RA-RNTI definition when two or more PRACH resource pools are configured was noted. This was further discussed in RAN2 AH 1807 which resulted in an offline discussion reported in [2]. Different PRACH pools may happen when one pool is configured for CBRA and another pool is configured for CFRA. In addition to CFRA and CBRA, a third resource pool may be defined for Msg1 SI request. The problem is illustrated in Figure 1 where two PRACH resource pools are configured and two different resources will obtain the same RA-RNTI.

Figure 1 – Two PRACH pools, one for CFRA and one for CBRA

The problem stems from the fact that RA-RNTI only indicates the relative position of the preamble transmission within the PRACH pool. In the case exemplified in Figure 1, the pools have different msg1-FrequencyStart resulting in different frequency positions even for PRACH resources with the same time index and same f_id. The resulting problem is that a UE cannot determine that a RAR scrambled with the RA-RNTI corresponds to the UE’s preamble transmission, since it could just as well correspond to a preamble transmission on the corresponding resource in the other PRACH pool.

The current RA-RNTI is not sufficient to unambiguously determine the PRACH resource used for the preamble transmission when more than one PRACH resource pool is configured.

## 2.1	Analysis

Several methods of how to solve this issue is given and discussed in [1] and [2]. One solution is to use one of the reserved bits in the RAR to distinguish CFRA and CBRA and thereby the PRACH resource pools. This would not be a viable way since the agreed RAR format only has one R bit. Using this R bit would only enable to distinguish 2 pools (e.g. CB and CF) and it would not be  use wise to use the reserved bit for a problem which could be solved by other methods.

Another suggested approach is to restrict the PDCCH search space to distinguish different PRACH pools. However, designing search space would impact RAN1 and should be avoided at this time.

A solution which would be straightforward to introduce is to distinguish the PRACH pools by introducing a frequency ID offset for the f_ids. In this way the f_id numbering would be different for the different pools. In the example shown in Figure 2, the PRACH pool for CBRA has a configured frequency ID offset so that the f_id starts at f_id=1, while the PRACH pool for CFRA has a frequency ID offset so that the f_ids start at f_id=5.

Figure 2. Example with PRACH pools using different frequency ID offsets.

Since it is possible to have up to 8 different f_id per PRACH resource pool, the offset should be able to indicate a start point of f_id=16, resulting in 24 possible f_ids in the RA-RNTI formula.

Introduce a frequency ID offset k for frequency offset calculation so that CFRA and CBRA have unique frequency IDs.

The frequency ID offset is signalled in RACH-ConfigGeneric together with frequencyStart.

It is up to the network to ensure that the offsets are configured to result in orthogonal resource allocation between CB and CFRA.

There is a CR for MAC in R2-1811616.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The current RA-RNTI is not sufficient to unambiguously determine the PRACH resource used for the preamble transmission when more than one PRACH resource pool is configured.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce a frequency ID offset k for frequency offset calculation so that CFRA and CBRA have unique frequency IDs.

Proposal 2	The frequency ID offset is signalled in RACH-ConfigGeneric together with frequencyStart.

Proposal 3	It is up to the network to ensure that the offsets are configured to result in orthogonal resource allocation between CB and CFRA.

# 4	References

R2-1809661, Discussion on the ambiguity in Msg2 reception, ZTE Corporation, 3GPP TSG-RAN WG2 NR Meeting 1807 AH, 2nd - 6th July 2018

R2-1810802, Offline Discussion#110 on Ambiguity in Msg.2 reception, ZTE Corporation, 3GPP TSG-RAN WG2 AH-1807, 2nd – 6th July 2018