3GPP TSG-RAN WG2 Meeting #103 	R2-1811689

# Gothenburg, Sweden 20 – 24 August 2018 									 Revision of R2-1809780

Agenda item:		11.3

Source:	Intel Corporation

Title:	Evaluation of NR control plane latency

Document for:		Discussion and Decision

# Introduction

Control plane latency is one of the metrics to be evaluated for ITU-R 5G submission [1]:

Proposal 2: For performance evaluation, the following allocation of responsibility is proposed. - … - User plane latency, control plane latency, mobility interruption time evaluation results (by analytical) are reviewed by RAN WG2 and reported by LS to RAN. - …

There were evaluations results for control plane latency submitted in previous RAN2 meetings (e.g. [10][11]). In RAN2#101 meeting, control plane latency reduction for LTE was discussed, and a LS [2] was sent to RAN1 regarding the reduction of processing delay for RAR.

In this contribution, we evaluate control plane latency based on latest RAN1 and RAN2 progress and the methodology used in LTE. The contribution is an update of previous contribution R2-1809780.

# Discussion

## Requirements

The requirements for control plane latency from ITU-R M.2140 [3] are as follows:

Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state). This requirement is defined for the purpose of evaluation in the eMBB and URLLC usage scenarios. The minimum requirement for control plane latency is 20 ms. Proponents are encouraged to consider lower control plane latency, e.g. 10 ms.

## Methodology

In LTE, control plane latency evaluation was captured in section 13.2 of TR 25.912 [4], and the evaluation results were further updated in section 16.2 and Annex B.1 of TR 36.912 [5].

In the study for CP latency reduction for LTE to meet the IMT-2020 requirement, evaluation is based on the procedure of resumption of a suspended RRC connection [2]. In RAN2#102 meeting, following was agreed regarding the components of LTE CP latency as below:

Table 1: Summary of control plane latency for FDD

| Component | Description | Latency [ms] |
| --- | --- | --- |
| 1 | Delay due to RACH scheduling period (1TTI) | 0 |
| 2 | Transmission of RACH Preamble | 1 |
| 3 | Preamble detection and processing in eNB | 2 |
| 4 | Transmission of RA response | 1 |
| 5 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Connection Resume Request) | 4 |
| 6 | Transmission of RRC Connection Resume Request | 1 |
| 7 | Processing delay in eNB (L2 and RRC) | 3 |
| 8 | Transmission of RRC Connection Resume | 1 |
| 9 | Processing delay in UE of RRC Connection Resume including grant reception | 7 |
| 10 | Transmission of RRC Connection Resume Complete and UP data | 0 |
|  | Total delay [ms] | 20 |
| Notes |  |  |
| 1 | For step 1, the procedure for "transition […] from a most “battery efficient” state" has yet not begun, hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. |  |
| 2 | For step 5, the latency of 4 ms has been agreed by RAN1, see LS in R2-1806411 |  |
| 3 | For step 7, the processing delay in eNB (L2 and RRC) has been reduced to 3 ms. |  |
| 4 | For step 9, the RRCConnectionResume message only includes MAC and PHY configuration. No DRX, SPS, CA, or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRC Connection Resume Complete and the data is transmitted over common search space with DCI format 0. |  |
| 5 | For step 10, the beginning of this subframe is considered to be "the start of continuous data transfer", hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. |  |

For the CP latency evaluation for NR, we consider a similar approach as in R2-1809185 [12]. The state transition from battery-efficient state RRC_INACTIVE to RRC_CONNECTED is considered. The control plane latency includes time to complete the RRC procedure in Figure 1 below. The same RRC message names as LTE are used in Figure 1.

# Figure 1: Transition from RRC_INACTIVE to RRC_CONNECTED

Assumptions:

- RACH scheduling period: short sequence preamble format B1/C0 is considered, which can have the highest density of PRACH configurations in time domain. The duration of Format B1/C0 is two OFDM symbols. According to RACH configurations in TS 38.211 [7], for 15 kHz and 30 kHz SCS numerologies, PRACH slots containing preamble B1 can be configured in every UL slot, and there can be 7 PRACH occasions configured in one PRACH slot for paired and spectrum. For unpaired spectrum, configuration index 187 and 188 are used for 15 and 30 kHz SCS, respectively (since alternating DL/UL pattern is assumed, as discussed later). Relevant configurations in TS 38.211 [7] are copied below.

Table 6.3.3.2-2: Random access configurations for FR1 and paired spectrum/supplementary uplink.

| PRACH Configuration Index | Preamble format |  |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | , number of time-domain PRACH occasions within a RACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |  |  |
| 196 | B1 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 7 | 2 |

Table 6.3.3.2-3: Random access configurations for FR1 and unpaired spectrum.

| PRACH Configuration Index | Preamble format |  |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | , number of time-domain PRACH occasions within a RACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |  |  |
| 187 | C0 | 1 | 0 | 1,3,5,7,9 | 2 | 1 | 6 | 2 |
| 188 | C0 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 8 | 1 | 3 | 2 |

- Preamble handling delay: this is specified in TS 38.213 [8] section 8.2 as “The window starts at the first symbol of the earliest control resource set the UE is configured for Type1-PDCCH common search space, as defined in Subclause 10.1, that is at least  symbols after the last symbol of the preamble sequence transmission, where  is defined in [10, TS 38.133].” So far, there is no agreement made by RAN4. As an assumption, 8 and 10 OFDM symbols (same as UE processing time N1 defined in Table 5.3-1 of TS 38.214 [9]) are assumed for 15 and 30 kHz SCS, respectively.

- 2, 4 and 7 OFDM symbols are used for PDSCH/PUSCH transmission. PDSCH Type B mapping is mandatory, according to TS 38.306 [6] below:

| Definitions for parameters | Per | M | FDDTDD DIFF | FR1 FR2 DIFF |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| pdsch-MappingTypeB Indicates whether the UE supports receiving PDSCH using PDSCH mapping type B. | UE | Yes | No | No |

For PDSCH Type B mapping, according to TS 38.214 [9] below, when 2 OFDM symbols are used, PDSCH can start in OFDM symbols with index from 0 to 12 in the DL slot; when 4 OFDM symbols are used, PDSCH can start in OFDM symbols with index from 0 to 10 in the DL slot, when 7 OFDM symbols are used, PDSCH can start in OFDM symbols with index from 0 to 7 in the DL slot,

Table 5.1.2.1-1: Valid S and L combinations

| PDSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | {0,1,2,3} | {3,…,14} | {3,…,14} | {0,1,2,3} | {3,…,12} | {3,…,12} |
| Type B | {0,…,12} | {2,4,7} | {2,…,14} | {0,…,10} | {2,4,6} | {2,…,12} |

For PUSCH mapping, below is specified in TS 38.214 [9]. PUSCH Type B mapping is assumed. When 2 OFDM symbols are used, PUSCH can start with index from 0 to 12 in the UL slot. When 4 OFDM symbols are used, PUSCH can start with index from 0 to 10 in the UL slot. When 7 OFDM symbols are used, PUSCH can start with index from 0 to 7 in the UL slot.

Table 6.1.2.1-1: Valid S and L combinations

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} | {0,…,12} | {1,…,12} | {1,…,12} |

- RAR processing time: In TS 38.213 [8] section 8.2, following is specified regarding the timing difference between RAR reception and Msg3: The UE may assume a minimum time between the last symbol of a PDSCH reception conveying a RAR and the first symbol of a corresponding Msg3 PUSCH transmission scheduled by the RAR in the PDSCH for a UE is equal to  msec.  is a time duration of  symbols corresponding to a PDSCH reception time for PDSCH processing capability 1 when additional PDSCH DM-RS is configured and  is a time duration of  symbols corresponding to a PUSCH preparation time for PUSCH processing capability 1 [6, TS 38.214]. Therefore when TA is not considered, the RAR processing time is shown in Table 1 below:

# Table 1: RAR processing time

| Numerology | (OFDM symbols) | (OFDM symbols) | Minimum RAR to Msg3 delay [OFDM symbols) |
| --- | --- | --- | --- |
| 15 kHz SCS | 13 | 10 | 30 |
| 30 kHz SCS | 13 | 12 | 39 |

- gNB and UE processing time: The same processing time as in R2-1809185 [12] is used for NR evaluation, i.e. gNB processing time in component 7 is 3 ms, and UE processing time in component 9 is 7 ms.

## Evaluation for paired spectrum

The CP latency depends on the relative timing offset when the RACH is triggered. The CP latency (component 1 to 8) for various cases are shown in Figure 2 and Figure 3 below for 15 and 30 kHz SCS, assuming 4 OFDM symbol duration for PDSCH/PUSCH. To avoid very wide diagram, only 1 ms is shown for component 7.

# Figure 2: Component 1 to 8 in paired spectrum, 15 kHz SCS

# Figure 3: Component 1 to 8 in paired spectrum (30 kHz SCS)

The evaluation for paired spectrum for the first row in Figure 2 (for 15 kHz SCS) and Figure 3 (for 30 kHz SCS) is shown in Table 2 below, assuming 4 OFDM symbol duration for PDSCH/PUSCH.

# Table 2: Transition time from RRC_INACTIVE to RRC_CONNECTED for paired spectrum

| Component | Description | Duration |  |  |
| --- | --- | --- | --- | --- |
|  |  | 15 kHz SCS | 30 kHz SCS |  |
| 1 | Average delay due to RACH scheduling period | 0 |  |  |
| 2 | Transmission of RACH Preamble | 2 symbols |  |  |
| 3 | Preamble detection and processing in gNB | 8 symbols | 10 symbols |  |
| 4a | Frame alignment | 0 | 2 symbols |  |
| 4 | Transmission of RA response | 4 symbols |  |  |
| 5 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Resume Request) | 30 symbols |  | 39 symbols |
| 6 | Transmission of RRC Resume Request | 4 symbols |  |  |
| 7 | Processing delay in gNB (L2 and RRC) | 3 ms |  |  |
| 8 | Transmission of RRC Resume (and UL grant) | 4 symbols |  |  |
| 9 | Processing delay in the UE of RRC Resume including the reception of UL grant | 7 ms |  |  |
| 10 | Transmission of RRC Resume Complete | 0 |  |  |
|  | Total delay | 52 symbols + 10 ms = 13.71 ms |  | 65 symbols + 10 ms = 12.32 ms |

CP latency evaluation results for paired spectrum is shown in Table 3 below by averaging all cases shown in Figure 2 and Figure 3.

# Table 3: CP latency evaluation results (ms) for paired spectrum

| Numerology | 15 kHz SCS | 30 kHz SCS |
| --- | --- | --- |
| 2 OFDM symbol PDSCH/PUSCH duration | 13.3 | 12.0 |
| 4 OFDM symbol PDSCH/PUSCH duration | 13.8 | 12.3 |
| 7 OFDM symbol PDSCH/PUSCH duration | 14.8 | 12.8 |

Observation 1: The analysis shows that the 20 ms control plane latency requirement can be satisfied for paired spectrum.

## Evaluation for unpaired spectrum

For unpaired spectrum, an alternating DL/UL pattern is assumed, i.e. the periodicity is 2 slots with one slot for DL and the other slot for UL.

For 15 kHz SCS, RACH configuration 187 is used, where RACH is configured in odd slots, and there are 6 RACH occasions in each RACH slot. For 30 kHz SCS, RACH configuration 188 is used, where RACH is configured in odd slots, and there are 3 RACH occasions in each RACH slot.

The CP latency depends on the relative timing offset when the RACH is triggered. The CP latency (component 1 to 8) for various cases are shown in Figure 4 and Figure 5 below for 15 and 30 kHz SCS, assuming 4 OFDM symbol duration for PDSCH/PUSCH. To avoid very wide diagram, only 1 ms is shown for component 7.

# Figure 4: Component 1 to 8 in unpaired spectrum, 15 kHz SCS

# Figure 5: Component 1 to 8 in unpaired spectrum, 30 kHz SCS

The evaluation for unpaired spectrum for the first row in Figure 4 (for 15 kHz SCS) and Figure 5 (for 30 kHz SCS) is shown in Table 4 below, assuming 4 OFDM symbol duration for PDSCH/PUSCH.

# Table 4: Transition time from RRC_INACTIVE to RRC_CONNECTED for unpaired spectrum

| Component | Description | Duration |  |  |
| --- | --- | --- | --- | --- |
|  |  | 15 kHz SCS |  | 30 kHz SCS |
| 1 | Average delay due to RACH scheduling period | 0 |  |  |
| 2 | Transmission of RACH Preamble | 2 symbols |  |  |
| 3 | Preamble detection and processing in gNB | 8 symbols | 10 symbols |  |
| 4a | Frame alignment | 2 symbols |  | 0 |
| 4 | Transmission of RA response | 4 symbols |  |  |
| 5 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Resume Request) | 30 symbols |  | 39 symbols |
| 6a | Frame alignment | 8 symbols |  | 0 |
| 6 | Transmission of RRC Resume Request | 4 symbols |  |  |
| 7 | Processing delay in gNB (L2 and RRC) | 3 ms |  |  |
| 8a | Frame alignment | 0 |  | 3 symbols |
| 8 | Transmission of RRC Resume (and UL grant) | 4 symbols |  |  |
| 9 | Processing delay in the UE (L2 and RRC) of RRC Resume including the reception of UL grant | 7 ms |  |  |
| 10 | Transmission of RRC Resume Complete | 0 |  |  |
|  | Total delay | 62 symbols + 10 ms = 14.42 ms |  | 66 symbols + 10 ms = 12.36 ms |

CP latency evaluation results for unpaired spectrum is shown in Table 5 below by averaging all cases shown in Figure 4 and Figure 5. Note that first row in Figure 4 and Figure 5 is allocated more weight since the corresponding PRACH occasion is the first occasions and have more probability to be utilized.

# Table 5: CP latency evaluation results (ms) for unpaired spectrum

| Numerology | 15 kHz SCS | 30 kHz SCS |
| --- | --- | --- |
| 2 OFDM symbol PDSCH/PUSCH duration | 14.0 | 12.3 |
| 4 OFDM symbol PDSCH/PUSCH duration | 14.3 | 12.4 |
| 7 OFDM symbol PDSCH/PUSCH duration | 15.0 | 13.4 |

Observation 2: The analysis shows that the 20 ms control plane latency requirement can be satisfied for unpaired spectrum.

# Conclusion

In this contribution, we evaluate control plane latency. We have the following observations:

Observation 1: The analysis shows that the 20 ms control plane latency requirement can be satisfied for paired spectrum.

Observation 2: The analysis shows that the 20 ms control plane latency requirement can be satisfied for unpaired spectrum.

We propose the following:

Proposal 1: The control plane latency results are captured in IMT-2020 self-evaluation.

# References

[1] RP-172532, Huawei, "Work plan for self evaluation towards 3GPP Update Submission to ITU-R"

[2] R2-1803985, "LS on CP latency reduction"

[3] Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”

[4] 3GPP TR 25.912, "Feasibility study for evolved Universal Terrestrial Radio Access (UTRA) and Universal Terrestrial Radio Access Network (UTRAN)"

[5] 3GPP TR 36.912, "Feasibility study for Further Advancements for E-UTRA (LTE-Advanced)"

[6] 3GPP TS 38.306, "NR; User Equipment (UE) radio access capabilities"

[7] 3GPP TS 38.211, "NR; Physical channels and modulation"

[8] 3GPP TS 38.213, "NR; Physical layer procedures for control"

[9] 3GPP TS 38.214, "NR; Physical Layer Procedures for data"

[10] R2-1805716, Huawei et al, “Consideration on self evaluation of IMT-2020 for CP latency”

[11] R2-1805653, Ericsson, “IMT-2020 self-evaluation: CP latency in NR”

[12] R2-1809185, RAN2, “Reply LS on CP latency reduction”