3GPP TSG-RAN2 Meeting #103	R2-1811710

# Gothenburg, Sweden, 20-24 Aug, 2018

Agenda item:	9.11.2

Source: 	Qualcomm Incorporated

Title: 	UE DL categories for 1024QAM

Document for:     Discussion/Decision

# Introduction

RAN2 received an LS from RAN1 in [1] on new UE categories for 1024QAM support. RAN1 endorsed new DL categories 22-26, plus enabling 1024QAM support for DL category 20. Therefore, RAN2 needs to capture the endorsed categories in RAN2 specifications.

Since there is a running 36.331 CR for 1024QAM WI, we provide a text proposal below to be merged to the running CR. We provide a separate CR for TS 36.306 in [2].

# TP for TS 36.331

Following changes are shown assuming running RRC CR [3] as baseline text (note the red text represents corrections needed in the existing text in current running CR [3]).

<<unchanged text omitted>>

#### –	UE-EUTRA-Capability

The IE UE-EUTRA-Capability is used to convey the E-UTRA UE Radio Access Capability Parameters, see TS 36.306 [5], and the Feature Group Indicators for mandatory features (defined in Annexes B.1 and C.1) to the network. The IE UE-EUTRA-Capability is transferred in E-UTRA or in another RAT.

NOTE 0:	For (UE capability specific) guidelines on the use of keyword OPTIONAL, see Annex A.3.5.

UE-EUTRA-Capability information element

-- ASN1START

UE-EUTRA-Capability ::=			SEQUENCE {

<<unchanged text omitted>>

UE-EUTRA-Capability-v1520-IEs ::= SEQUENCE {

measParameters-v1520				MeasParameters-v1520,

nonCriticalExtension				UE-EUTRA-Capability-v1530-IEs		OPTIONAL

}

UE-EUTRA-Capability-v1530-IEs ::= SEQUENCE {

rf-Parameters-v1530				RF-Parameters-v15x0						OPTIONAL,

ue-CategoryDL-v1530				INTEGER (22..26)						OPTIONAL,

nonCriticalExtension			SEQUENCE {}								OPTIONAL

}

<<unchanged text omitted>>

| UE-EUTRA-Capability field descriptions | FDD/ TDD diff |
| --- | --- |
| ue-CategoryDL UE DL category as defined in TS 36.306 [5]. Value n17 corresponds to UE category 17, value m1 corresponds to UE category M1, value oneBis corresponds to UE category 1bis, value m2 corresponds to UE category M2. For ASN.1 compatibility, a UE indicating DL category 0, m1 or m2 shall also indicate any of the categories (1..5) in ue-Category (without suffix), which is ignored by the eNB, a UE indicating UE category oneBis shall also indicate UE category 1 in ue-Category (without suffix), and a UE indicating UE category m2 shall also indicate UE category m1. The field ue-CategoryDL is set to values 0, m1, oneBis, m2, 4, 6, 7, 9 to 16, n17, 18, 19, 20, 21, 22, 23, 24, 25, 26 in this version of the specification. | - |

# Summary

Include TP in section 2 to RRC running CR for 1024QAM.

# References

- R2-1811008/R1-1807549, LS on details of new categories for 1024QAM

- R2-1811711, UE DL categories for 1024QAM, CR for TS 36.306

- R2-1807712, Support of 1024QAM in TS 36.331