3GPP TSG-RAN WG2 Meeting #103	R2-1811786

Gothenburg, Sweden, 20th - 24th August 2018	revision of R2-1809879

Source:	vivo

Title:	Discussion on the IDC solutions for EN-DC

Agenda Item:	10.4.2.4

Document for:	Discussion and Decision

# Introduction

According to the LS from RAN4 on the IDC issue for EN-DC [3], RAN4 confirms that “IDC problems not only for other systems already identified in LTE CA but also for mmWave ISM bands are expected with EN-DC”, and suggests to reuse the current LTE IDC solution frame work. In the Annex we listed the potential NR bands including both FR1 and FR2 which could have the IDC problem. In this contribution, we provide some analysis on how to reuse the current LTE IDC solution frame work for EN-DC.

# Discussion

## Potential IDC issue(s) and solution(s) for EN-DC

According to the LTE RRC specification 36.331 [1], the following IDC issues which also occur for EN-DC are addressed:

- Issue 1: Adjacent frequency interference from 3GPP UL frequency to non-3GPP frequency

- Issue 2: Adjacent frequency interference from non-3GPP frequency to 3GPP DL frequency

- Issue 3: harmonic wave interference from 3GPP UL band combination to non-3GPP frequency

- Issue 4: Hardware sharing problem

The specified IDC solutions are listed as follows:

- Solution 1: report the 3GPP frequency as the attacking frequency

- Solution 2: report the 3GPP frequency as the victim frequency

- Solution 3: report the 3GPP band combination as the attacking band combination

- Solution 4: report one hardware sharing indication

- Solution 5: report the DRX related assistance information per UE

- Solution 6: report the desired subframe reservation patterns related assistance information per UE

- Solution 7: configured UL subframes per UE in which the UE can deny UL transmission.

In the following table, we give the mapping table between the solutions and the issues in LTE IDC.

Table 1: IDC issues and corresponding solutions in LTE

|  | Solution 1 | Solution 2 | Solution 3 | Solution 4 | Solution 5 | Solution 6 | Solution 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Issue 1 | Yes |  |  |  |  | Yes | Yes |
| Issue 2 |  | Yes |  |  | Yes | Yes |  |
| Issue 3 |  |  | Yes |  |  | Yes | Yes |
| Issue 4 |  |  |  | Yes |  |  |  |

In EN-DC, the IDC issue could be more complicated than LTE, as both MCG and SCG can have independent IDC issues. And due to the flexible design of NR, some LTE configurations/fields (e.g. solution 6 or 7) cannot be simply re-used for the NR.

Regarding the Solution 1, 2 and 3 (i.e. the frequency domain IDC solution), we think the general concept of reporting the affected frequency(ies) or band combination(s) can be reused. The detailed specification impacts of Solution 1, 2 and 3 can be found in [9] [10] [11].

Proposal 1: The NR frequency of EN-DC can be reported as attacking or victim frequency.

Proposal 2: The EN-DC band combination can be reported as attacking band combination.

Regarding Solution 4, as the LTE and the NR are different RATs which could use different RF chains, then the UE could only have the LTE RF chain shared by other non-3GPP RAT (e.g. WiFi). The current one bit hardware sharing indication cannot differentiate whether the hardware sharing is caused by NR or LTE.

Proposal 3: The hardware sharing indication can be reported per RAT (i.e. LTE or NR).

Regarding Solution 5, as MCG and SCG could have interference from different non-3GPP RAT(s), the UE desired DRX pattern should be independent for MCG and SCG.

Proposal 4: The UE desired DRX pattern can be reported per CG (i.e. MCG or SCG).

Regarding Solution 6, according to the current NR RRC 38.331 [8] as quoted below, the NR does not have a specific UL-DL subframe pattern number as in LTE. Then the UE should be allowed to indicate any subframe/slot as attacking or victim subframe separately. Furthermore, the desired subframe pattern should be reported per CG (i.e. MCG or SCG).

38.331: TDD-UL-DL-Pattern ::= SEQUENCE { dl-UL-TransmissionPeriodicity ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10}, nrofDownlinkSlots INTEGER (0..maxNrofSlots), nrofDownlinkSymbols INTEGER (0..maxNrofSymbols-1), nrofUplinkSlots INTEGER (0..maxNrofSlots), nrofUplinkSymbols INTEGER (0..maxNrofSymbols-1), ... }

36.331: IDC-SubframePattern-r11 ::= CHOICE { subframePatternFDD-r11 BIT STRING (SIZE (4)), subframePatternTDD-r11 CHOICE { subframeConfig0-r11 BIT STRING (SIZE (70)), subframeConfig1-5-r11 BIT STRING (SIZE (10)), subframeConfig6-r11 BIT STRING (SIZE (60)) }, ... }

Proposal 5: The UE desired subframe pattern can be reported per CG (i.e. MCG or SCG).

Proposal 6: The UE desired subframe pattern of NR can indicate the attacking UL subframe(s)/slot(s) via a bit map.

Proposal 7: The UE desired subframe pattern of NR can indicate the victim DL subframe(s)/slot(s) via a bit map.

Regarding Solution 7, as NR is using the slot as the time unit for the uplink transmission, the NR gNB should be able to configure the number of UL slots which can be denied by the UE.

36.331: IDC-Config-r11 ::= SEQUENCE { idc-Indication-r11 ENUMERATED {setup} OPTIONAL, -- Need OR autonomousDenialParameters-r11 SEQUENCE { autonomousDenialSubframes-r11 ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1}, autonomousDenialValidity-r11 ENUMERATED { sf200, sf500, sf1000, sf2000, spare4, spare3, spare2, spare1} } OPTIONAL, -- Need OR ..., [[ idc-Indication-UL-CA-r11 ENUMERATED {setup} OPTIONAL -- Cond idc-Ind ]], [[ idc-HardwareSharingIndication-r13 ENUMERATED {setup} OPTIONAL -- Need OR ]] }

Proposal 8: The autonomous deny parameters can be configured per CG (i.e. MCG or SCG).

Proposal 9: The autonomous deny parameters for NR includes the number of UL slots which can be denied within a configured time period.

# Conclusion

According to the analysis given above, we have the following Proposals:

Proposal 1: The NR frequency of EN-DC can be reported as attacking or victim frequency.

Proposal 2: The EN-DC band combination can be reported as attacking band combination.

Proposal 3: The hardware sharing indication can be reported per RAT (i.e. LTE or NR).

Proposal 4: The UE desired DRX pattern can be reported per CG (i.e. MCG or SCG).

Proposal 5: The UE desired subframe pattern can be reported per CG (i.e. MCG or SCG).

Proposal 6: The UE desired subframe pattern of NR can indicate the attacking UL subframe(s)/slot(s) via a bit map.

Proposal 7: The UE desired subframe pattern of NR can indicate the victim DL subframe(s)/slot(s) via a bit map.

Proposal 8: The autonomous deny parameters can be configured per CG (i.e. MCG or SCG).

Proposal 9: The autonomous deny parameters for NR includes the number of UL slots which can be denied within a configured time period.

# Reference

- 3GPP TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

- 3GPP TS 38.101-2, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

- R4-1803254, LSout, From RAN4, To RAN2, “LS reply on In-Device Coexistence solution for EN-DC”.

- 3GPP TS 36.331, “Radio Resource Control (RRC); Protocol specification”

- 3GPP TS 36.300, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”

- R2-1802609, IDC indication for EN-DC, Qualcomm Incorporated	discussion

- R2-1802101, Discussion on the IDC report of NR frequency in EN-DC, vivo	discussion

- 3GPP TS 38.331, “NR; Radio Resource Control (RRC); Protocol specification”

- R2-1809876, Discussion on the IDC report of NR frequency in EN-DC, vivo

- R2-1809877, CR of 38.331 on the IDC report of NR frequency in EN-DC, vivo

- R2-1809878, CR of 36.331 on the IDC report of NR frequency in EN-DC, vivo

# Annex A

The following table gives the potential victim frequency range between NR band(s) and other systems:

| Victim Systems | Frequency range [MHz] |  |  | Regions | Side-lobe | NR second harmonic order | NR third harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  | n50(UL)/n74(UL) |  |  |
| Galileo | 1559 | - | 1591 |  | n50(UL)/n74(UL) |  |  |
| GLONASS | 1591 | - | 1610 |  | n50(UL)/n70(UL) |  |  |
| GPS | 1563 | - | 1587 |  | n50(UL)/n74(UL) |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | US/Europe | n7/n38/n41 |  | n5/n20/n28/n82 |
|  | 2400 | - | 2494 | Asia | n7/n38/n41 |  | n5/n20/n28/n82 |
| ISM band (5GHz) | 5150 | - | 5925 | US | n79 | n7/n38/n41 | n1/n2/n3/n66/n80/n84 |
|  | 5150 | - | 5350 | Europe | n79 | n7/n38/n41 | n1/n2/n3/n66/n80/n84 |
|  | 5470 | - | 5725 |  |  |  |  |
|  | 5150 | - | 5825 | Asia | n79 | n7/n38/n41 | n1/n2/n3/n66/n80/n84 |
| ISM band (45GHz) | 42300 | - | 47000 | China |  | n258 |  |
|  | 47200 | - | 48400 |  |  |  |  |
| ISM band (60GHz) | 57000 | - | 66000 | Europe |  | n257 |  |
|  | 57050 | - | 64000 | USA - Canada |  | n257 |  |
|  | 57000 | - | 64000 | South Korea |  | n257 |  |
|  | 59000 | - | 66000 | Japan |  | n257 |  |
|  | 59000 | - | 64000 | China |  | n257 |  |
|  | 59400 | - | 62900 | Australia |  | n257 |  |

# Annex B

The following table for FR1 is extracted from 38.101-1 [1]:

Table 5.2-1: NR operating bands in FR1

| NR Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | FDD |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n50 | 1432 MHz | – | 1517 MHz | 1432 MHz | – | 1517 MHz | TDD |
| n51 | 1427 MHz | – | 1432 MHz | 1427 MHz | – | 1432 MHz | TDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
| n70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| n74 | 1427 MHz | – | 1470 MHz | 1475 MHz | – | 1518 MHz | FDD |
| n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |
| n76 | N/A |  |  | 1427 MHz | – | 1432 MHz | SDL |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
| n80 | 1710 MHz | – | 1785 MHz | N/A |  |  | SUL |
| n81 | 880 MHz | – | 915 MHz | N/A |  |  | SUL |
| n82 | 832 MHz | – | 862 MHz | N/A |  |  | SUL |
| n83 | 703 MHz | – | 748 MHz | N/A |  |  | SUL |
| n84 | 1920 MHz | – | 1980 MHz | N/A |  |  | SUL |

# Annex C

The following table for FR2 is extracted from 38.101-2 [2]:

Table 5.2-1: NR operating bands in FR2

| NR Operating Band |  | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n257 | 26500 MHz |  | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
| n258 | 24250 MHz |  | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |
| n260 | 37000 MHz |  | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |