3GPP TSG-RAN WG2 #103	Tdoc R2-1811884

Gothenburg, Sweden, 20th – 24th August 2018

Agenda Item:	10.2.2.1

Source:	Ericsson

Title:	Duplication support in EN-DC

Document for:	Discussion, Decision

# Introduction

With the introduction of PDCP duplication in both LTE and NR, as well as EN-DC as such, a remaining issue is to clarify support of the combination of such features. In particular, we discuss the support of LTE CA duplication in EN-DC in this contribution.

The related agreements are:

Agreements RAN2 AH2 for NR, 2017

1:	CA packet duplication is not applied to LTE CA of EN-DC.

2: 	In the EN-DC and NG-EN-DC case, CA packet duplication can only be configured for SCG bearer. In the NE-DC case, CA packet duplication can only be configured for the MCG bearer.

3: 	In the NR-NR DC case, CA packet duplication can only be configured for non-split bearer.

Agreements RAN2#99 for NR

1	We will not support MAC CE activation/deactivation of duplication within LTE MAC.

2	We will not support the CA duplication in LTE

3	CA duplication is supported for all non-split UM DRBs if the bearer uses NR-PDCP, for all architecture options (apart from cases excluded by 1 and 2)

FFS: for AM DRBs and SRBs

4	DC duplication is supported for all split DRB and SRBs if the bearer uses NR-PDCP, for all architecture options

# Discussion

According to current agreements, PDCP duplication is supported in the following architecture options:

|  | MCG CA duplication | Split, DC duplication | SCG CA duplication |
| --- | --- | --- | --- |
| LTE single | Supported with HRLLC | - | - |
| LTE DC | Supported with HRLLC | Supported with HRLLC | Supported with HRLLC |
| NR single | Supported | - | - |
| NR-NR DC | Supported | Supported | Supported |
| EN-DC | Not supported | Supported (NR PDCP) | Supported |
| NE-DC | Supported | Supported (NR PDCP) | Not supported |

It is noted that the agreements in RAN2 AH2, 2017 and RAN2#99 were made before the LTE CA duplication was defined as part of the HRLLC work item. Therefore, e.g. [1] revisited these agreements. We analyse the feasibility of LTE CA duplication in EN-DC in the following. Thereby, the following two MCG bearer termination options should be considered:

Figure 1: LTE CA duplication protocol architectures in EN-DC.

Given the agreements so far, it is first of all noted, that it is unclear whether E-UTRA PDCP or NR PDCP are to be supported. While LTE HRLLC work assumed E-UTRA PDCP for LTE CA duplication, NR work assumed always NR PDCP for duplication and split bearer support.

For MCG LTE CA duplication in EN-DC (and likewise SCG LTE CA duplication in NE-DC), which where discussed in [1], it becomes clear from Figure 1 that one further needs to distinguish between MN and SN terminated bearers. If the PDCP entity is terminated in a separate node than the RLC entities, inter-node signalling must be considered. For NR-NR DC and MN terminated SCG bearer RAN3 made already the required signalling additions (SgNB addition/modification request) to e.g. enable CA based duplication in the SgNB, i.e. the setup of two GTP-U tunnels. Furthermore, the primary RLC entity/logical channel must be synchronized between the nodes, i.e. must be known to PDCP [38.331 RIL issues E510 and E511].

If also LTE CA duplication should be supported in EN-DC, similar inter-node signalling must be specified for this case. Alternatively, the support of LTE CA duplication in EN-DC could be limited to MN terminated MCG bearers.

It appears however, that a scenario, in which LTE must provide high-reliability with CA duplication via the LTE carriers, while the UE is also connected to NR with EN-DC, but the NR link should not be used for high-reliability provisioning, is somewhat questionable. Considering the limited use-case relevance, as well as the further standardization complexity, we propose:

Confirm that in EN-DC, LTE CA duplication is not supported.

For DRBs, as an additional alternative to RRC reconfiguration of PDCP duplication, MAC CEs have been defined to control the duplication, both for NR and LTE. Currently, these MAC CEs are based on different formats, however discussion to align them is ongoing in LTE HRLLC. Nevertheless, for discussed case of LTE CA duplication in EN-DC, in case it should be supported, we do not see a need to support the MAC CE’s cross-technology, which would come with a further increased complexity. Instead, one could rely on RRC-reconfiguration, or the MAC CE within the own cell-group/technology.

Confirm that in EN-DC, LTE MAC CE for duplication control is not considered.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Confirm that in EN-DC, LTE CA duplication is not supported.

Proposal 2	Confirm that in EN-DC, LTE MAC CE for duplication control is not considered.

# References

- R2-1809459, OPPO, 3GPP TSG-RAN WG2 AH-1807, Montreal, Canada, July 2 – July 6, 2018