3GPP TSG-RAN WG2 Meeting #103		R2-1811905

Gothenburg, Sweden, 20th - 24th August 2018

Source:	vivo

Title:	Discussion on the rationality of using MCS-C-RNTI in C-RNTI MAC CE

Agenda Item:	10.3.1.2

Document for:	Discussion and Decision

- Introduction

With the introduction of a new RNTI, some agreements on the new RNTI were achieved in RAN2#AH1807 meeting:

- RAN2 assumes that the “new RNTI” is equivalent to C-RNTI w.r.t. MAC functionality.

- The details of when the new RNTI is monitored is not specified in MAC, except that RAN2 assumes that DRX active time applies also to the new RNTI as for C-RNTI. No need to make exception in MAC.

- From MAC point of view the new RNTI can be used for MSG2 reception for CF BFR. No need to make exception.

- FFS if the new RNTI can be used in C-RNTI MAC CE.

- The new RNTI is applicable to Contention Resolution for CBRA, when configured. No need to make exception.

From the agreements, the new RNTI is equivalent to C-RNTI. But whether it can be used in C-RNTI MAC CE is FFS. In this contribution, we discuss the rationality of using the new RNTI in C-RNTI MAC CE.

- Discussion

First, from the view of functionality, the C-RNTI MAC CE is included in Msg3 to identify a UE. The MCS-C-RNTI is configured per UE. And it can realize the same function as C-RNTI. Then if the MCS-C-RNTI could be used in C-RNTI MAC CE, it can achieve the same functionality with C-RNTI in C-RNTI MAC CE.

Observation 1: If the MCS-C-RNTI could be used in C-RNTI MAC CE, it can achieve the same functionality with C-RNTI in C-RNTI MAC CE.

Second, from the view of procedure, it seems to maximise the similarity of the MCS-C-RNTI and C-RNTI according to the agreements for Msg2 reception for CF BFR and Msg4 reception for CBRA. During the CF BFR, C-RNTI and MCS-C-RNTI can be used for MSG2 reception .During the CBRA, C-RNTI and MCS-C-RNTI can be applied to Contention Resolution.  At the same timer, C-RNTI MAC CE can be included in Msg3. To align the procedure, it is better to use MCS-C-RNTI in C-RNTI MAC CE.

Observation 2: If the MCS-C-RNTI could be used in C-RNTI MAC CE, the procedure is aligned with C-RNTI in C-RNTI MAC CE.

Proposal: MCS-C-RNTI if configured can be used in C-RNTI MAC CE and it is up to UE implementation which RNTI is used in C-RNTI MAC CE.

We have provided a more detailed CR in [2].

- Conclusion

In this contribution, we discuss discuss the rationality of using the new RNTI in C-RNTI MAC CE with the following observations and proposal:

Observation 1: If the MCS-C-RNTI could be used in C-RNTI MAC CE, it can achieve the same functionality with C-RNTI in C-RNTI MAC CE.

Observation 2: If the MCS-C-RNTI could be used in C-RNTI MAC CE, the procedure is aligned with C-RNTI in C-RNTI MAC CE.

Proposal: MCS-C-RNTI if configured can be used in C-RNTI MAC CE and it is up to UE implementation which RNTI is used in C-RNTI MAC CE.

References

- RAN2 #AH1807 Chairman’s Notes.

- R2-18xxxxx,  CR of 38.321 on MCS-C-RNTI, vivo