3GPP TSG-RAN WG2 Meeting #103	R2-1812002

Gothenburg, Sweden, 20 - 24 August 2018	R2-1809912

Agenda item:	10.4.1.6.2

Source:	Nokia, Nokia Shanghai Bell

Title:	UE behaviour if SIB1 is not broadcast (or not received)

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In this paper we discuss what should be UE behaviour if MIB/SIB1 is not received (or broadcasted)

# 2	Essential SIB1 missing

In NR we have two different cases of SIB1 reception will not succeed in the cell:

- SIB1 is not transmitted in the cell as indicated by ssb-SubcarrierOffset (NSA)

- SIB1 is transmitted in the cell as indicated by ssb-SubcarrierOffset, but UE is unable to acquire it

Currently there is no behaviour captured whether UE is allowed to consider cell on same carrier for reselection. As we have introduced intraFreqReselection parameter in MIB UE should follow this parameter in our view

Proposal: If UE is not able to receive SIB1 or SIB1 is not broadcast then UE should follow intraFreqReselection parameter from MIB

# 5	Conclusion

In this paper we proposed how to capture details of SIB1 not transmitted or not being able to receive it:

Proposal: If UE is not able to receive SIB1 or SIB1 is not broadcast then UE should follow intraFreqReselection parameter from MIB

Proposal: Agree on the changes as provided in the Annex Text Proposal

# Annex TP

#### 5.2.2.5	Essential system information missing

The UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	if the UE is unable to acquire the MIB:

3>	consider the cell as barred in accordance with TS 38.304 [20]; and

3>	perform barring as if intraFreqReselection is set to allowed;

2>	else if the UE is unable to acquire the SIB1 or ssb-SubcarrierOffset indicates SIB1 not scheduled in the cell:

3>	if intraFreqReselection in MIB1 is set to notAllowed:

4> consider cell re-selection to other cells on the same frequency as the barred cell as not allowed, as specified in TS 38.304 [X].

3>	else:

4> consider cell re-selection to other cells on the same frequency as the barred cell as allowed, as specified in TS 38.304 [X].

##### --------------------------------------------NEXT CHANGES--------------------------------------------