3GPP TSG-RAN WG2 #103	R2-1812018

Gothenburg, Sweden, 20th – 24th August 2018	(Revision of R2-1809950)

Agenda Item:	10.5.1

Source:	Ericsson

Title:	SCG failure handling in case of NE-DC and NN-DC (TP to 37.340)

Document for:	Discussion, Decision

# Introduction

In RAN2#97bis, the following agreements were made regarding EN-DC [1]:

- If radio link failure is detected for MCG, UE initiates RRC connection re-establishment procedure with the PCell.

- If radio link failure is detected for SCG, UE suspends all SCG radio bearers (including SCG split bearers) and SCG transmissions for split radio bearers, and reports SCG failure information

In this contribution, we will outline the SCG RLF handling for NR E-UTRAN Dual Connectivity (NE-DC) and NR-NR Dual Connectivity (NN-DC) cases along with the discussion of what principles we can reuse and what changes we can propose regarding SCG RLF handling. However, MCG failure discussion is separately given in [2].

# Discussion

In LTE, when a Radio Link Failure (RLF) is detected on the MCG, RRC connection reestablishment procedure is triggered, suspending the ongoing communication until the UE re-synchronizes with the network. On the other hand, if the UE experiences a failure on the SCG, then the UE does not have to initiate an RRC connection reestablishment procedure but instead it sends SCGFailureInformation via MCG SRB. Upon receiving SCGFailureInformation, MN can then decide to initiate an SN release (and optionally, change the SCG to an SN with a better radio conditions). In this case, as the MCG radio link is available, and the traffic can be rerouted via MCG, UE does not experience a notable service interruption as in MCG RLF.

In EN-DC, the actions triggered upon an RLF are mainly based on the existing LTE procedures. Accordingly, in RAN2#97bis meeting, it was agreed that if an RLF is detected on the MCG (i.e., MCG RLF), the UE initiates the RRC connection re-establishment procedure. It was also agreed that when an RLF is detected on the SCG (i.e., SCG RLF), UE suspends all SCG SRBs/DRBs as well as SCG transmission for MCG split SRBs/DRBs / SCG split DRBs. In this case, instead of directly triggering the RRC re-establishment procedure, UE indicates to the MN an SCG RLF has occurred with an SCGFailureInformation via the MCG path of MCG Split SRB or MCG SRB.

Figure 1: Illustration of MCG Split SRB where SCG failure occurs on the SCG path (SRB1S) in case of NE-DC scenario.

In order to utilize the mobility robustness benefits of Split SRB – that is also proposed for NE-DC and NN-DC as discussed in [3] – the EN-DC decisions regarding SCG RLF can be taken as a baseline for SCG failure handling for NE-DC and NN-DC. Accordingly, NE-DC and NN-DC should support at least the following SCG failure cases:

- SCG RLF;

- SN change failure;

However, since direct SRB is proposed not to be supported for NE-DC as discussed in [3], but it is proposed to be supported for NN.DC [4], the following SCG failure cases are only supported for EN-DC and NG-EN-DC (as earlier agreed for both) as well as NN-DC (as proposed herein).

- SCG configuration failure (only for messages on SRB3);

- SCG RRC integrity check failure (on SRB3).

Accordingly, the necessary changes in Stage 2 is provided in the appended TP in Section 3.

NE-DC should support at least the following SCG failure cases: SCG RLF and SN change failure.

NN-DC should support the following SCG failure cases: SCG RLF, SN change failure, SCG configuration failure (only for messages on SRB3) and SCG RRC integrity check failure (on SRB3).

The appended Stage 2 text is taken as a baseline for the supported SCG failure cases in NE-DC and NN-DC.

# Text Proposal to 37.340

START OF CHANGES

## 7.7	SCG/MCG failure handling

RLF is declared separately for the MCG and for the SCG.

If radio link failure is detected for MCG, the UE initiates the RRC connection re-establishment procedure.

The following SCG failure cases are supported for EN-DC, NGEN-DC, NE-DC and NN-DC:

-	SCG RLF;

-	SN change failure;

The following SCG failure cases are only supported for EN-DC, NGEN-DC and NN-DC:

-	SCG configuration failure (only for messages on SRB3);

-	SCG RRC integrity check failure (on SRB3).

In EN-DC, NGEN-DC, NE-DC and NN-DC, upon SCG failure the UE suspends SCG transmissions for all radio bearers and reports the SCG Failure Information to the MN, instead of triggering re-establishment.

In all SCG failure cases, the UE maintains the current measurement configurations from both the MN and the SN and the UE continues measurements based on configuration from the MN and the SN if possible. The SN measurements configured to be routed via the MN will continue to be reported after the SCG failure.

NOTE:	UE may not continue measurements based on configuration from the SN after SCG failure in certain cases (e.g. UE cannot maintain the timing of PSCell).

The UE includes in the SCG Failure Information message the measurement results available according to current measurement configuration of both the MN and the SN.	The MN handles the SCG Failure Information message and may decide to keep, change, or release the SN/SCG. In all the cases, the measurement results according to the SN configuration and the SCG failure type may be forwarded to the old SN and/or to the new SN.

END OF CHANGES

# Conclusion

Based on the discussion in Section 2, we proposed the following:

- NE-DC should support at least the following SCG failure cases: SCG RLF and SN change failure.

NN-DC should support the following SCG failure cases: SCG RLF, SN change failure, SCG configuration failure (only for messages on SRB3) and SCG RRC integrity check failure (on SRB3).

The appended Stage 2 text is taken as a baseline for the supported SCG failure cases in NE-DC and NN-DC.

# References

- 3GPP TSG-RAN WG2 #97bis, Chairman Notes, Spokane, USA, 3th – 7th February 2017.

- R2-1812017, MCG failure handling in case of NE-DC and NN-DC, Ericsson, RAN2#103, Gothenburg, Sweden, 20th – 24th August 2018.

- R2-1812014, Support of SRBs for NE-DC and NG-EN-DC, Ericsson, RAN2#103, Gothenburg, Sweden, 20th – 24th August 2018.

- R2-1812059, Support of SRB3 for NN-DC, Ericsson, RAN2#103, Gothenburg, Sweden, 20th – 24th August 2018