3GPP TSG-RAN WG2 #103	R2-1812020

Gothenburg, Sweden, 20th – 24th August 2018

Agenda Item:	10.5.4

Source:	Ericsson

Title:	NR RRC design principles for NN-DC

Document: 	Discussion, Decision

- Introduction

RAN2 has not so far agreed on the RRC design details regarding NR-NR Dual Connectivity (NN-DC). In order to ensure the progress, we will be discussing the RRC design principles for NN-DC in this contribution, both from the RRC architecture and ASN.1 design perspectives.

- Discussion

  - RRC baseline for NN-DC

In EN-DC, as the primarily specified DC scenario, we agreed that the both MN and SN can generate ASN.1 [1]. In addition, it was agreed that ASN.1 generated by the secondary node (SN) can be transported by the master node (MN) at least in some cases, e.g., for the first configuration or the configurations requiring capability coordination between MN and SN. Furthermore, while LTE RRC Connection Reconfiguration message is re-used, NR lower layer configurations generated by NR RRC are encapsulated into the LTE RRC message as an NR RRC message. Further, it was decided that separate containers for NR PDCP configurations are used allowing flexible mapping between NR PDCP entities and lower layers (LTE or NR).

As a next step, we need to specify the RRC design principles for the MR-DC cases. Considering that we have many other DC cases in addition to EN-DC, i.e., NE-DC (Option 4), NG-EN-DC (Option 7) and NN-DC, ideally it would be optimal to have a common RRC baseline for all. This way, the specified RRC architecture and ASN.1 structure could be re-used as much as possible. Further, the efforts needed for both the network and the UE implementations could be significantly reduced. In this context, the ASN.1 design of EN-DC can be considered as the baseline in NN-DC where the baseline can be complemented with the necessary changes. Furthermore, the supported bearer types are harmonized as illustrated in Figure 1.

It would simplify both the network and the UE implementations if ASN.1 design structure follows the same baseline for the MR-DC and NN-DC cases where the baseline can be complemented with the necessary changes.

The ASN.1 design of EN-DC is taken as the baseline for NN-DC bearer configuration.

Figure 1. DRBs to be suported in NN-DC.

  - Current ASN.1 structure for EN-DC

RAN2 has agreed that NR configuration embedded in the LTE RRCConnectionReconfiguration is RRC PDU. Later on, it was agreed that this is the same as NR RRCReconfiguration message. In addition, it was agreed that bearer configuration is given in another transparent container, one used to configure bearers using KeNB and constructed by the MN and another used to configure bearers using S-KgNB and constructed by the SN.

In the latest merged CR for NR RRC specifications, NR RRC Reconfiguration structure is defined as follows:

RRCReconfiguration message (38.331)

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::= 				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcReconfiguration					RRCReconfiguration-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCReconfiguration-IEs ::= 			SEQUENCE {

radioBearerConfig						RadioBearerConfig 														OPTIONAL, -- Need M

secondaryCellGroup						OCTET STRING (CONTAINING CellGroupConfig)								OPTIONAL, -- Need M

measConfig								MeasConfig																OPTIONAL, -- Need M

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					RRCReconfiguration-vxx-IEs												OPTIONAL

}

RRCReconfiguration-vxx-IEs ::= 			SEQUENCE {

masterCellGroup							OCTET STRING (CONTAINING CellGroupConfig)								OPTIONAL, -- Need M

fullConfig								ENUMERATED {true} 														OPTIONAL, -- Need N

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF DedicatedInfoNAS						OPTIONAL, -- Cond nonHO

keyRefresh								KeyRefresh															OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery				OCTET STRING (CONTAINING SIB1)										OPTIONAL,

dedicatedSystemInformationDelivery	OCTET STRING (CONTAINING SystemInformation)							OPTIONAL,

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

KeyRefresh ::=					SEQUENCE {

keySetChangeIndicator			BOOLEAN															OPTIONAL,	-- Cond MasterKeyChange

nextHopChainingCount			NextHopChainingCount											OPTIONAL,	-- Cond MasterKeyChangeNCC

n2ModeNAS-Container				OCTET STRING													OPTIONAL,	-- Cond InterSystemHO

...

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

In the embedded NR RRC PDU in the LTE RRCConnectionReconfiguration, the fields that can be present are

- secondaryCellGroup to configure SCG

- measConfig to configure NR measurements

It was also agreed in RAN2#100 that, for EN-DC for the Dec-17 drop, the UE behaviour will only be specified for the case that the embedded NR RRC PDU in the LTE RRRCConnectionReconfiguration includes secondaryCellGroup and/or measConfig but nothing else.

Furthermore, RadioBearerConfig structure below can be used to reconfigure PDCP via SRB1 at least for the cases when the relevant parameters are needed to be coordinated with MN. NR PDCP configuration can be sent via SRB3 when the coordination with MN is not needed.

RadioBearerConfig information element (38.331)

-- ASN1START

-- TAG-RADIO-BEARER-CONFIG-START

RadioBearerConfig ::=					SEQUENCE {

srb-ToAddModList						SRB-ToAddModList										OPTIONAL,	-- Cond HO-Conn

srb3-ToRelease							ENUMERATED{true}										OPTIONAL,	-- Need N

drb-ToAddModList						DRB-ToAddModList										OPTIONAL,	-- Cond HO-toNR

drb-ToReleaseList						DRB-ToReleaseList										OPTIONAL,	-- Need N

securityConfig 							SecurityConfig											OPTIONAL,	-- Need M

...

}

SRB-ToAddModList ::=					SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod

SRB-ToAddMod ::=						SEQUENCE {

srb-Identity							SRB-Identity,

reestablishPDCP							ENUMERATED{true}										OPTIONAL, 	-- Need N

discardOnPDCP                           ENUMERATED{true}										OPTIONAL,	-- Need N

pdcp-Config								PDCP-Config												OPTIONAL,	-- Cond PDCP

...

}

DRB-ToAddModList ::=					SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddMod ::=						SEQUENCE {

cnAssociation							CHOICE {

eps-BearerIdentity						INTEGER (0..15),									-- EPS-DRB-Setup

sdap-Config								SDAP-Config											-- 5GC

} 										OPTIONAL, -- Cond DRBSetup

drb-Identity							DRB-Identity,

reestablishPDCP							ENUMERATED{true}										OPTIONAL, 	-- Need N

recoverPDCP								ENUMERATED{true}										OPTIONAL, 	-- Need N

pdcp-Config								PDCP-Config												OPTIONAL,	-- Cond PDCP

...

}

DRB-ToReleaseList ::=					SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity

SecurityConfig ::=						SEQUENCE {

securityAlgorithmConfig					SecurityAlgorithmConfig									OPTIONAL,	-- Cond RBTermChange

keyToUse								ENUMERATED{master, secondary}								OPTIONAL,	-- Cond RBTermChange

...}

-- TAG-RADIO-BEARER-CONFIG-STOP

-- ASN1STOP

These parameters are for the bearers configured with NR PDCP. Rest of the parameters the UE gets via LTE RRC (SecurityModeCommand and RRCConnectionReconfiguration with MobilityControlInfo).

RRCConnectionReconfiguration message - Rel-15 IEs (36.331)

RRCConnectionReconfiguration-v1510-IEs ::= SEQUENCE {

nr-Config-r15					CHOICE {

release							NULL,

setup							SEQUENCE {

endc-ReleaseAndAdd-r15	BOOLEAN,

nr-SecondaryCellGroupConfig-r15	OCTET STRING				OPTIONAL,	-- Need ON

p-MaxEUTRA-r15					P-Max						OPTIONAL	-- Need ON

}

}																	OPTIONAL,	-- Need ON

sk-Counter-r15					INTEGER (0.. 65535)					OPTIONAL,	-- Need ON

nr-RadioBearerConfig1-r15		OCTET STRING						OPTIONAL,	-- Need ON

nr-RadioBearerConfig2-r15		OCTET STRING						OPTIONAL,	-- Need ON

tdm-PatternConfig-r15			CHOICE {

release							NULL,

setup							SEQUENCE {

subframeAssignment-r15			SubframeAssignment-r15,

harq-Offset-r15					INTEGER (0.. 9)

}

}																	OPTIONAL,	-- Cond FDD-PCell

nonCriticalExtension			RRCConnectionReconfiguration-v15x0-IEs		OPTIONAL

}

RRCConnectionReconfiguration-v15x0-IEs ::= SEQUENCE {

dedicatedInfoNASListExt-r15		SEQUENCE (SIZE(maxDRB-Plus1..maxDRB-r15)) OF

DedicatedInfoNAS			OPTIONAL,	-- Cond nonHO

sCellToAddModListExt-v15xy		SCellToAddModListExt-v15xy			OPTIONAL,	-- Need ON

sCellGroupToReleaseList-r15		SCellGroupToReleaseList-r15			OPTIONAL,	-- Need ON

sCellGroupToAddModList-r15		SCellGroupToAddModList-r15			OPTIONAL,	-- Need ON

securityConfigHO-r15				SecurityConfigHO-r15					OPTIONAL,	-- Cond HO-to5GC

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

  - Introduction of NN-DC

When NN-DC is introduced, there are fundamental questions need to be answered first regarding ASN.1. For instance, one of the main questions is how SN configuration is constructed for lower layers and higher layers.  Another is whether SCG configuration is a complete RRC message or IE(s).

We consider that it is beneficial to reuse the structure where PDCP configuration and SCG configuration are separated also in NN-DC. This makes it possible to use harmonised bearer concept also in the NN-DC scenario as well as for the split architecture option. Moreover, within the new field proposed (see radioBearerConfig2-r15 below), RadioBearerConfig IE can be embedded as an octet string so as to enable the scenario where MN and SN support different specification releases. Note that the same radioBearerConfig2-r15 can also be used for NE-DC, as shown in [2].

In NN-DC, separate bearer configuration fields (similar to nr-RadioBearerConfigX in LTE RRC) are introduced per termination point in NR RRC.

Different from multi-RAT DC scenarios, both MN and SN follow NR RRC protocol. Therefore, in NN-DC, SN lower layer configuration generated by SN can be encapsulated into the MN RRC message as NR RRC IE (i.e., CellGroupConfig). On the other hand, as discussed in [3], to be able to allow both MN and SN RRC entities configure measurements, a new field for measurement configuration has to be introduced.

In NN-DC, SN lower layer configuration generated by SN is encapsulated into the MN RRC message as NR RRC IE (i.e., CellGroupConfig).

In NN-DC, to be able to allow both MN and SN configure measurement configurations, a new filed for measurement configuration is introduced.

The ASN.1 example for the NN-DC extension in NR RRC (TS 38.331) is given below. Here, the lines highlighted in yellow indicate the new lines for the radio bearer and measurement configurations, and the line highlighted in green indicates the line already exists for SCG configuration.

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::= 				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcReconfiguration					RRCReconfiguration-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCReconfiguration-IEs ::= 			SEQUENCE {

radioBearerConfig						RadioBearerConfig 														OPTIONAL, -- Need M

secondaryCellGroup						OCTET STRING (CONTAINING CellGroupConfig)								OPTIONAL, -- Need M

measConfig								MeasConfig																OPTIONAL, -- Need M

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					RRCReconfiguration-vxx-IEs												OPTIONAL

}

RRCReconfiguration-vxx-IEs ::= 			SEQUENCE {

masterCellGroup							OCTET STRING (CONTAINING CellGroupConfig)								OPTIONAL, -- Need M

fullConfig								ENUMERATED {true} 														OPTIONAL, -- Need N

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF DedicatedInfoNAS						OPTIONAL, -- Cond nonHO

keyRefresh								KeyRefresh															OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery				OCTET STRING (CONTAINING SIB1)												OPTIONAL,

dedicatedSystemInformationDelivery	OCTET STRING (CONTAINING SystemInformation)									OPTIONAL,

radioBearerConfig2-r15				OCTET STRING (CONTAINING RadioBearerConfig)									OPTIONAL,	-- Need M

measConfig2-r15             		OCTET STRING (CONTAINING MeasConfig)										OPTIONAL,	-- Need M

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

KeyRefresh ::=					SEQUENCE {

keySetChangeIndicator			BOOLEAN															OPTIONAL,	-- Cond MasterKeyChange

nextHopChainingCount			NextHopChainingCount											OPTIONAL,	-- Cond MasterKeyChangeNCC

n2ModeNAS-Container				OCTET STRING													OPTIONAL,	-- Cond InterSystemHO

...

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

- Conclusion

Based on the discussion in section 2, we have the following observations:

- It would simplify both the network and the UE implementations if ASN.1 design structure follows the same baseline for the MR-DC and NN-DC cases where the baseline can be complemented with the necessary changes.

Further, we propose the following:

- The ASN.1 design of EN-DC is taken as the baseline for NN-DC bearer configuration.

- In NN-DC, separate bearer configuration fields (similar to nr-RadioBearerConfigX in LTE RRC) are introduced per termination point in NR RRC.

- In NN-DC, SN lower layer configuration generated by SN is encapsulated into the MN RRC message as NR RRC IE (i.e., CellGroupConfig).

- In NN-DC, to be able to allow both MN and SN configure measurement configurations, a new filed for measurement configuration is introduced.

- References

- 3GPP TS 37.340, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2 (Release 15).

- R2-1812016, NR RRC design principles for NE-DC, Ericsson, RAN2#103, Gothenburg, Sweden, 20th – 24th August 2018.

- R2-1811580, MR-DC and NR-NR DC measurement principles, Ericsson, RAN2#103, Gothenburg, Sweden, 20th – 24th August 2018.