3GPP TSG-RAN2 Meeting #103	R2-1812053

# Gothenburg, Sweden, 20th Aug 2018 - 24th Aug 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1335 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction and improvement for SR with HARQ-ACK in FeNB-IoT_(Option 1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2018-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In section 5.4.4 in MAC specification, the NB-IoT UE will choose one way to send SR among SR with HARQ-ACK, dedicated NPRACH resources for SR and normal NPRACH resources, etc. However, we think there exists ambiguity about the first condition of ”MAC entity has no valid dedicated resource for SR”. One understanding is this condition means neither SR with HARQ-ACK nor SR without HARQ-ACK is configured. However, the case that SR with HARQ-ACK is configured but no acknowledgement of the data in this TTI hasn’t be covered by the “else” branch, then even normal Random Access Procedure will not be triggered for SR in this case. In order to avoid such incomplete processes, a clarification is needed. The simplest way is that the condition of ”MAC entity has no valid dedicated resource for SR” would also include the case that SR with HARQ-ACK is configured but no acknowledgement of the data in this TTI. Furthermore, from service or application perspective, downlink and uplink transmission doesn’t always happen at the same time. That means, when the UE has SR to be sent, there may not always has HARQ-ACK for DL transmission to be sent. Since the UE cannot predict the upcoming DL scheduling, if there has no HARQ-ACK in current TTI, the random access procedure would always be triggered for SR no matter whether there will have DL data transmission in a short period later. That will not only cause more UE power consumption for initiating random access, but also make piggybacked SR with HARQ-ACK rarely used and the benefits of introducing this function greatly reduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the clarification (no change to specification) that the condition of ”MAC entity has no valid dedicated resource for SR” would also include the case that SR with HARQ-ACK is configured but no acknowledgement of the data in this TTI, it’s suggest to introduce a sr-DelayTimer, which would be activated if SR with HARQ-ACK is configured but no acknowledgement of the data and no configured grant for BSR. This delay timer is used for pending SR transmission over Random Access Procedure and waiting for the possible HARQ-ACK for DL transmission. Impact Analysis Impacted functionality None. Inter-operability - If the network is implemented according to the CR and the UE is not, there is no inter-operability problem. - If the UE is implemented according to the CR and the network is not, there is no inter-operability problem. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not approval of the change will not only cause more UE power consumption for initiating random access, but also make piggybacked SR with HARQ-ACK rarely used and the benefits of introducing this function greatly reduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4, 5.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.331 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of the modified section>

5.4.4	Scheduling Request

The Scheduling Request (SR) is used for requesting UL-SCH resources for new transmission.

When an SR is triggered, it shall be considered as pending until it is cancelled. All pending SR(s) shall be cancelled and sr-ProhibitTimer and ssr-ProhibitTimer shall be stopped when a MAC PDU is assembled and this PDU includes a BSR which contains buffer status up to (and including) the last event that triggered a BSR (see subclause 5.4.5), or, if all pending SR(s) are triggered by Sidelink BSR, when a MAC PDU is assembled and this PDU includes a Sidelink BSR which contains buffer status up to (and including) the last event that triggered a Sidelink BSR (see subclause 5.14.1.4), or, if all pending SR(s) are triggered by Sidelink BSR, when upper layers configure autonomous resource selection, or when the UL grant(s) can accommodate all pending data available for transmission.

If the MAC entity has resources for SR configured on only one of SPUCCH and PUCCH, that SR resource is valid for all logical channels. If the MAC entity has resources for SR configured on both PUCCH and SPUCCH, MAC entity shall consider all logical channels that have triggered an SR (and at retxBSR-Timer expiry, MAC entity shall consider all logical channels, belonging to a LCG, with data available for transmission):

-	PUCCH resources for SR are valid if logicalChannelSr-Restriction is not configured, or if logicalChannelSr-Restriction allows SR on PUCCH, for any of the logical channels;

-	SPUCCH resources for SR are valid if logicalChannelSr-Restriction is not configured, or if logicalChannelSr-Restriction allows SR on SPUCCH, for any of the logical channels.

If an SR is triggered and there is no other SR pending, the MAC entity shall set the SR_COUNTER and the SSR_COUNTER to 0.

As long as one SR is pending, the MAC entity shall for each TTI:

-	if no UL-SCH resources are available for a transmission in this TTI:

-	Except for NB-IoT:

-	if the MAC entity has no valid PUCCH nor valid SPUCCH resource for SR configured in any TTI:

-	if the MAC entity is a MCG MAC entity and rach-Skip is not configured; or

-	if the MAC entity is a SCG MAC entity and rach-SkipSCG is not configured:

-	initiate a Random Access procedure (see subclause 5.1) on the corresponding SpCell and cancel all pending SRs;

-	else if this TTI is not part of a measurement gap or Sidelink Discovery Gap for Transmission, and if transmission of V2X sidelink communication is not prioritized in this TTI as described in subclause 5.14.1.2.2:

-	if the MAC entity has at least one valid SPUCCH resource for SR configured for this TTI and if ssr-ProhibitTimer is not running:

-	if SSR_COUNTER < dssr-TransMax:

-	increment SSR_COUNTER by 1;

-	instruct the physical layer to signal the SR on one valid SPUCCH resource for SR;

-	start the ssr-ProhibitTimer.

-	else:

-	notify RRC to release SPUCCH for all serving cells;

-	if the MAC entity has no valid PUCCH resource for SR configured in any TTI:

-	notify RRC to release PUCCH for all serving cells;

-	notify RRC to release SRS for all serving cells;

-	clear any configured downlink assignments and uplink grants;

-	initiate a Random Access procedure (see subclause 5.1) on the SpCell and cancel all pending SRs.

-	if the MAC entity has at least one valid PUCCH resource for SR configured for this TTI and if sr-ProhibitTimer is not running:

-	if SR_COUNTER < dsr-TransMax:

-	increment SR_COUNTER by 1;

-	instruct the physical layer to signal the SR on one valid PUCCH resource for SR;

-	start the sr-ProhibitTimer.

-	else:

-	notify RRC to release PUCCH and SPUCCH for all serving cells;

-	notify RRC to release SRS for all serving cells;

-	clear any configured downlink assignments and uplink grants;

-	initiate a Random Access procedure (see subclause 5.1) on the SpCell and cancel all pending SRs.

-	For NB-IoT:

-	if the MAC entity has no valid dedicated resource for SR or configured grant for BSR in any TTI and sr-DelayTimer is not running:

-	initiate a Random Access Procedure (see subclause 5.1) and cancel all pending SRs.

-	else:

-	if the MAC entity has valid resource for SR together with acknowledgement of the data in this TTI:

-	instruct the physical layer to signal the SR together with acknowledgement of the data.

-	stop sr-DelayTimer, if running.

-	else:

-	if the MAC entity has valid PRACH resource for SR configured in this TTI and sr-ProhibitTimer is not running:

-	instruct the physical layer to signal the SR on one valid PRACH resource for SR.

-	start the sr-ProhibitTimer in the subframe containing the last repetition of the corresponding SR transmission.

NOTE 1:	The selection of which valid PUCCH/SPUCCH resource for SR to signal SR on when the MAC entity has more than one valid PUCCH/SPUCCH resource for SR in one TTI or overlapping TTIs is left to UE implementation.

NOTE 2:	SR_COUNTER is incremented for each SR bundle. sr-ProhibitTimer is started in the first TTI of an SR bundle.

5.4.5	Buffer Status Reporting

The Buffer Status reporting procedure is used to provide the serving eNB with information about the amount of data available for transmission in the UL buffers associated with the MAC entity. RRC controls BSR reporting by configuring the three timers periodicBSR-Timer, retxBSR-Timer and logicalChannelSR-ProhibitTimer and by, for each logical channel, optionally signalling logicalChannelGroup which allocates the logical channel to an LCG [8].

For the Buffer Status reporting procedure, the MAC entity shall consider all radio bearers which are not suspended and may consider radio bearers which are suspended.

For NB-IoT the Long BSR is not supported and all logical channels belong to one LCG.

A Buffer Status Report (BSR) shall be triggered if any of the following events occur:

-	UL data, for a logical channel which belongs to a LCG, becomes available for transmission in the RLC entity or in the PDCP entity (the definition of what data shall be considered as available for transmission is specified in [3] and [4] or [17] respectively) and either the data belongs to a logical channel with higher priority than the priorities of the logical channels which belong to any LCG and for which data is already available for transmission, or there is no data available for transmission for any of the logical channels which belong to a LCG, in which case the BSR is referred below to as "Regular BSR";

-	UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC control element plus its subheader, in which case the BSR is referred below to as "Padding BSR";

-	retxBSR-Timer expires and the MAC entity has data available for transmission for any of the logical channels which belong to a LCG, in which case the BSR is referred below to as "Regular BSR";

-	periodicBSR-Timer expires, in which case the BSR is referred below to as "Periodic BSR".

For Regular BSR:

-	if the BSR is triggered due to data becoming available for transmission for a logical channel for which logicalChannelSR-Prohibit is configured by upper layers:

-	start or restart the logicalChannelSR-ProhibitTimer;

-	else:

-	if running, stop the logicalChannelSR-ProhibitTimer.

For Regular and Periodic BSR:

-	if more than one LCG has data available for transmission in the TTI where the BSR is transmitted: report Long BSR;

-	else report Short BSR.

For Padding BSR:

-	if the number of padding bits is equal to or larger than the size of the Short BSR plus its subheader but smaller than the size of the Long BSR plus its subheader:

-	if more than one LCG has data available for transmission in the TTI where the BSR is transmitted: report Truncated BSR of the LCG with the highest priority logical channel with data available for transmission;

-	else report Short BSR.

-	else if the number of padding bits is equal to or larger than the size of the Long BSR plus its subheader, report Long BSR.

For NB-IoT or BL UEs:

-	if rai-Activation is configured, and a buffer size of zero bytes has been triggered for the BSR, and the UE may have more data to send or receive in the near future:

-	cancel any pending BSR.

If the Buffer Status reporting procedure determines that at least one BSR has been triggered and not cancelled:

-	if the MAC entity has UL resources allocated for new transmission for this TTI:

-	instruct the Multiplexing and Assembly procedure to generate the BSR MAC control element(s);

-	start or restart periodicBSR-Timer except when all the generated BSRs are Truncated BSRs;

-	start or restart retxBSR-Timer.

-	else if a Regular BSR has been triggered and logicalChannelSR-ProhibitTimer is not running:

-	if an uplink grant is not configured or the Regular BSR was not triggered due to data becoming available for transmission for a logical channel for which logical channel SR masking (logicalChannelSR-Mask) is setup by upper layers; or

-	if sr-WithHARQ-ACK-Config is configured and there is valid resource for SR together with acknowledgement of the data in this TTI:

-	a Scheduling Request shall be triggered.

-	if sr-DelayTimer is configured and there is no valid dedicated NPRACH resource for SR or configured grant for BSR in any TTI:

-	start or restart sr-DelayTimer.

A MAC PDU shall contain at most one MAC BSR control element, even when multiple events trigger a BSR by the time a BSR can be transmitted in which case the Regular BSR and the Periodic BSR shall have precedence over the padding BSR.

For EDT, the MAC entity shall not generate a BSR MAC control element if new transmission is for Msg3.

The MAC entity shall restart retxBSR-Timer upon indication of a grant for transmission of new data on any UL-SCH.

All triggered BSRs shall be cancelled in case the UL grant(s) in this TTI can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC control element plus its subheader. All triggered BSRs shall be cancelled when a BSR is included in a MAC PDU for transmission.

The MAC entity shall transmit at most one Regular/Periodic BSR in a TTI. If the MAC entity is requested to transmit multiple MAC PDUs in a TTI, it may include a padding BSR in any of the MAC PDUs which do not contain a Regular/Periodic BSR.

All BSRs transmitted in a TTI always reflect the buffer status after all MAC PDUs have been built for this TTI. Each LCG shall report at the most one buffer status value per TTI and this value shall be reported in all BSRs reporting buffer status for this LCG.

NOTE 1:	A Padding BSR is not allowed to cancel a triggered Regular/Periodic BSR, except for NB-IoT. A Padding BSR is triggered for a specific MAC PDU only and the trigger is cancelled when this MAC PDU has been built.

NOTE 2:	If UL HARQ operation is autonomous for the HARQ entity and if the BSR is already included in a MAC PDU for transmission by this HARQ entity, but not yet transmitted by lower layers, it is up to UE implementation how to handle the BSR content.

<End of the modified section>