3GPP TSG-RAN2 Meeting #103	R2-1812101

# Gothenburg, Sweden, 20th Aug 2018 - 24th Aug 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR |  | rev |  | Current version: | 15.2.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MIB-TDD-NB for FeNB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh-Core |  |  |  |  |  | Date: |  |  | 2018-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In the IE of operationModeInfo-r15, the choice of Guardband-NB-r15 is used for indicating the case that anchor carrier in guard-band operationMode. Furthermore, there are four choices in sub-IE sib-GuardbandInfo-r15 in Guardband-NB-r15, one of which is sib-GuardbandAnchor and used to indicate that SIB1 and other SIs are on anchor carrier. Since there also have other IEs of sib1-CarrierInfo and si-CarrierInfo which can be both set to “anchor” to indicate the case that SIB1 and other SIs are on anchor carrier, we think it’s no need to waste a code point in sib-GuardbandInfo-r15 to redundantly indicate such case. - In current specification, there has no field description for sib-GuardbandInfo-r15 and only has field description for the sub-IE sib-GuardbandGuardband-r15. We think the RAN1 agreements about how to determine the non-anchor carrier location for the cases of in-band non-anchor carrier + guard-band anchor carrier haven’t been captured. So the description for sib-GuardbandInfo-r15 including description for each choice in it would be needed. Moreover, as all the sub-IEs are already under the IE of sib-GuardbandInfo-r15 which explicitly indicates guard-band mode for anchor carrier, we slightly suggest to delete the first “Guardband” for each sub-IE name in sib-GuardbandInfo-r15 in order to shorten the IE name. - Based on the RAN1 agreements, only the following two cases need the information of LTE system bandwidth, one is that non-anchor carrier is in the opposite guard-band side (symmetric guard-band), the other is that non-anchor carrier is in in-band samePCI mode. However, in the current Guardband-NB-r15 structure, lteBandwitdh-r15 is outside the sib-GuardbandInfo-r15, that means it can be applied to any choice in sib-GuardbandInfo-r15 even it’s not useful for some of choices. So we think it’s better to move lteBandwitdh-r15 into the related choices in sib-GuardbandInfo-r15 which really need it. In order to include the lteBandwitdh-r15 into sib-GuardbandInfo-r15 without increasing the length of the sib-GuardbandInfo-r15, we can make use of the 1 bit saved by deleting ”sib-GuardbandAnchor-r15” from sib-GuardbandInfo-r15. Then it’s no need to increase the length of sib-GuardbandInfo-r15. - Based on field description, guardbandEdge-r15 is used to indicate the side of the guard-band anchor carrier relative to the LTE carrier. However, we cannot find the straight RAN1 agreement related to this IE. And we understand UE can determine the location of guard-band anchor carrier with the raster offset. So the guardbandEdge-r15 IE can be deleted. - With deleting lteBandwitdh-r15, guardbandEdge-r15 from Guardband-NB-r15 and no change to length of sib-GuardbandInfo-r15, the total length of Guardband-NB-r15 can be reduced to 6 bits. It’s feasible to also reduce 1 spare bit for Inband-SamePCI-NB-r15, Inband-DifferentPCI-NB-r15 and Standalone-NB-r15. Then the length of operationModeInfo-r15 would be reduced by 1 bit. This bit can be saved as spare bit for MasterInformationBlock-TDD-NB-r15. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To delete the first code point “sib-GuardbandAnchor-r15” from sib-GuardbandInfo-r15. - To provide field description for “sib-GuardbandInfo-r15”, with description for each choice in it. And to delete the field description of sib-GuardbandGuardband-r15. Also to delete the first “Guardband” for each sub-IE name in sib-GuardbandInfo-r15 in order to shorten the IE name. - To move the IE lteBandwitdh-r15 into sib-GuardbandInfo-r15. The IE lteBandwitdh-r15 in Guardband-NB-r15 can be deleted, including: - To add a new code point “sib-SameGuardband-r15” into the sib-GuardbandInfo-r15 by making use of the 1 bit saved by deleting ”sib-GuardbandAnchor-r15” from sib-GuardbandInfo-r15. - To change the name “sib-GuardbandGuardband-r15” to “sib-OppositeGuardband-r15” and in which the existing bit is applied for indicating lteBandwitdh-r15. - To shorten the name “sib-GuardbandInbandSamePCI-r15” to “sib-InbandSamePCI-r15” and in which the spare bit is applied for indicating lteBandwitdh-r15. - To delete the IE guardbandEdge-r15 from Guardband-NB-r15. - To reduce 1 bit for operationModeInfo-r15 and save this bit as spare bit for MasterInformationBlock-TDD-NB-r15. Impact Analysis Impacted functionality None. Inter-operability - If the network is implemented according to the CR and the UE is not, there is no inter-operability problem. - If the UE is implemented according to the CR and the network is not, there is no inter-operability problem. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The field description is not clear and one more spare bit in MIB-TDD-NB will be occupied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of the modified section>

MasterInformationBlock-TDD-NB

The MasterInformationBlock-TDD-NB includes the system information transmitted on BCH in TDD.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: E-UTRAN to UE

MasterInformationBlock-TDD-NB

-- ASN1START

MasterInformationBlock-TDD-NB-r15 ::=	SEQUENCE {

systemFrameNumber-MSB-r15				BIT STRING (SIZE (4)),

hyperSFN-LSB-r15						BIT STRING (SIZE (2)),

schedulingInfoSIB1-r15					INTEGER (0..15),

systemInfoValueTag-r15					INTEGER (0..31),

ab-Enabled-r15							BOOLEAN,

operationModeInfo-r15				CHOICE {

inband-SamePCI-r15					Inband-SamePCI-NB-r15,

inband-DifferentPCI-r15				Inband-DifferentPCI-NB-r15,

guardband-r15						Guardband-NB-r15,

standalone-r15						Standalone-NB-r15

},

sib1-CarrierInfo-r15					ENUMERATED {anchor, non-anchor},

si-CarrierInfo-r15						ENUMERATED {anchor, non-anchor},

spare									BIT STRING (SIZE (8))

}

Guardband-NB-r15 ::=				SEQUENCE {

rasterOffset-r15					ChannelRasterOffset-NB-r13,

sib-GuardbandInfo-r15				CHOICE {

sib-SameGuardband-r15			SIB-SameGuardbandInfo-NB-r15,

sib-OppositeGuardband-r15	SIB-OppositeGuardbandInfo-NB-r15,

sib-InbandSamePCI-r15		SIB-InbandSamePCIInfo-NB-r15,

sib-InbandDiffPCI-r15		SIB-InbandDiffPCIInfo-NB-r15

},

}

Inband-SamePCI-NB-r15 ::=			SEQUENCE {

eutra-CRS-SequenceInfo-r15			INTEGER (0..31),

sib-InbandLocation-r15				ENUMERATED {lower, higher},

}

Inband-DifferentPCI-NB-r15 ::=			SEQUENCE {

eutra-NumCRS-Ports-r15					ENUMERATED {same, four},

rasterOffset-r15						ChannelRasterOffset-NB-r13,

sib-InbandLocation-r15					ENUMERATED {lower, higher},

spare									BIT STRING (SIZE (2))

}

Standalone-NB-r15 ::=					SEQUENCE {

sib-StandaloneLocation-r15				ENUMERATED {lower, higher},

spare									BIT STRING (SIZE (5))

}

SIB-SameGuardbandInfo-NB-r15 ::= 	SEQUENCE {

spare									BIT STRING (SIZE (1))

}

SIB-OppositeGuardbandInfo-NB-r15 ::= 	SEQUENCE {

eutra-Bandwitdh-r15					ENUMERATED {bw5or15, bw10or20},

}

SIB-InbandSamePCIInfo-NB-r15 ::= SEQUENCE {

eutra-Bandwitdh-r15					ENUMERATED {bw5or15, bw10or20},

}

SIB-InbandDiffPCIInfo-NB-r15 ::= SEQUENCE {

sib-EUTRA-NumCRS-Ports-r15				ENUMERATED {same, four}

}

-- ASN1STOP

MasterInformationBlock-TDD-NB field descriptions

ab-Enabled Value TRUE indicates that access barring is enabled and that the UE shall acquire SystemInformationBlockType14-NB before initiating RRC connection establishment or resume.

eutra-Bandwitdth EUTRA system bandwidth. Value bw5or15 corresponds to bandwidth 5 or 15 MHz, value bw10or20 corresponds to bandwidth 10 or 20 MHz,

eutra-CRS-SequenceInfo Information of the carrier containing NPSS/NSSS/NPBCH. Each value is associated with an E-UTRA PRB index as an offset from the middle of the LTE system sorted out by channel raster offset. See TS 36.211[21] and TS 36.213 [23].

eutra-NumCRS-Ports, sib-eutra-NumCRS-Ports Number of E-UTRA CRS antenna ports, either the same number of ports as NRS or 4 antenna ports. See TS 36.211 [21], TS 36.212 [22], and TS 36.213 [23].

hyperSFN-LSB Indicates the 2 least significant bits of hyper SFN. The remaining bits are present in SystemInformationBlockType1-NB.

operationModeInfo Deployment scenario (in-band/guard-band/standalone) and related information. See TS 36.211 [21] and TS 36.213 [23]. Inband-SamePCI indicates an in-band deployment and that the NB-IoT and LTE cell share the same physical cell id and have the same number of NRS and CRS ports. Inband-DifferentPCI indicates an in-band deployment and that the NB-IoT and LTE cell have different physical cell id. guardband indicates a guard-band deployment. standalone indicates a standalone deployment.

rasterOffset NB-IoT offset from LTE channel raster. Unit in kHz. See TS 36.211[21] and TS 36.213 [23].

schedulingInfoSIB1 This field contains an index to a table specified in TS 36.213 [23, Table 16.4.1.3-3] that defines SystemInformationBlockType1-NB scheduling information. If sib1-CarrierInfo is set to non-anchor, E-UTRAN configures a value between 0 and 7.

si-CarrierInfo Carrier used for SI message transmission. Value anchor cooresponds to anchor carrier, value non-anchor corresponds to non-anchor carrier. See TS 36.213 [23].

sib-GuardbandInfo Location of the non-anchor carrier used for SIB1 and/or SI transmission when operationmodeInfo is set to guardband. See TS 36.213 [23]. sib-SameGuardband corresponds to the carrier adjacent to the anchor carrier on the outer side of the guardband. sib-OppositeGuardband corresponds to the carrier closest to the edge of the LTE carrier in the opposite guardband. sib-InbandSamePCI corresponds to the carrier at the edge of the LTE carrier and on the same side as the anchor carrier. sib-InbandDiffPCI corresponds to the carrier at the edge of the LTE carrier and on the same side as the anchor carrier. If both sib1-CarrierInfo and si-CarrierInfo values are set to anchor, the UE ignores sib-GuardbandInfo.

sib-InbandLocation Location of the non-anchor carrier used for SIB1 and/or SI transmission when operationmodeInfo is set to inband-SamePCI or inband-DifferentPCI. See TS 36.213 [23]. If sib1-CarrierInfo and/or si-CarrierInfo values is set to non-anchor, value lower corresponds to the lower adjacent carrier relative to the anchor carrier and value higher corresponds to the higher adjacent carrier relative to the anchor carrier. If both sib1-CarrierInfo and si-CarrierInfo values are set to anchor, the UE ignores sib-InbandLocation.

sib-StandaloneLocation Location of the non-anchor carrier used for SIB1 and/or SI transmission when operationmodeInfo is set to standalone. See TS 36.213 [23]. If sib1-CarrierInfo and/or si-CarrierInfo values is set to non-anchor, value lower corresponds to the lower adjacent carrier relative to the anchor carrier and value higher corresponds to the higher adjacent carrier relative to the anchor carrier. If both sib1-CarrierInfo and si-CarrierInfo values are set to anchor, the UE ignores sib-StandaloneLocation.

sib1-CarrierInfo Carrier used for SIB1 transmission. See TS 36.213 [23, 16.4.1.3]. Value anchor corresponds to anchor carrier, value non-anchor cooresponds to non-anchor carrier.

systemFrameNumber-MSB Defines the 4 most significant bits of the SFN. As indicated in TS 36.211 [21], the 6 least significant bits of the SFN are acquired implicitly by decoding the NPBCH.

systemInfoValueTag Common for all SIBs other than MIB-NB, SIB14-NB and SIB16-NB.

<End of the modified section>