3GPP TSG-RAN2 Meeting #103	R2-1812289

# Gothenburg, Sweden, 20th – 24th August 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | Stage 2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to support frequency-multiplexed paging occasions. |
| --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-07 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Frequency-multiplexing of POs enables 4 POs to be configured per PF, even in scenarios where less than 4 POs can fit time-multiplexed in PF. This ensures that a maximum paging capacity on par with that in LTE can be achieved without increasing the UE energy consumption through increased frequency of “false” page detections, i.e. cases where a page monitoring UE detects a page indication on the PDCCH and goes on to decode the Paging message on the PDSCH, only to find that it is not being paged (i.e. its UE ID is not included in the Paging message). Relying only on the increased maximum number of paging records in a Paging message (32 in NR instead of 16 in LTE) increases the paging capacity at the expense of an increased rate of “false” page detections, since the PO density is lower and hence more UEs have to be allocated to each PO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of the new optional parameter secondControlResourceSetForPaging in the PDCCH-ConfigCommon IE to support frequency-multiplexing of POs. This CR is applicable for SA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The maximum paging capacity of NR will not be on par with that of LTE in all scenarios or the rate of “false” page detections will increase, thereby increasing the energy consumption of UEs in RRC_IDLE and RRC_INACTIVE state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS/TR 38.304 CR0043 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### –	PDCCH-ConfigCommon

The IE PDCCH-ConfigCommon is used to configure cell specific PDCCH parameters provided in SIB as well as during handover and PSCell/SCell addition.

PDCCH-ConfigCommon information element

-- ASN1START

-- TAG-PDCCH-CONFIGCOMMON-START

PDCCH-ConfigCommon ::=				SEQUENCE {

controlResourceSetZero				ControlResourceSetZero												OPTIONAL, 	-- Cond InitialBWP-Only

commonControlResourceSet			ControlResourceSet										OPTIONAL, 	-- Need R

secondControlResourceSetForPaging	ControlResourceSet													OPTIONAL,	-- Need R

searchSpaceZero						SearchSpaceZero											OPTIONAL, 	-- Cond InitialBWP-Only

commonSearchSpaceList				SEQUENCE (SIZE(1..4)) OF SearchSpace								OPTIONAL,	-- Need R

searchSpaceSIB1						SearchSpaceId														OPTIONAL,	-- Need S

searchSpaceOtherSystemInformation	SearchSpaceId														OPTIONAL,	-- Need S

pagingSearchSpace					SearchSpaceId														OPTIONAL, 	-- Need S

ra-SearchSpace						SearchSpaceId														OPTIONAL, 	-- Need S

...

}

-- TAG-PDCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PDCCH-ConfigCommon field descriptions

commonControlResourceSet An additional common control resource setwhich may be configured and used for RAR (see ra-SearchSpace). If the network configures this field, it uses a ControlResourceSetId other than 0 for this ControlResourceSet.

commonSearchSpaceList A list of additional common search spaces. If the network configures this field, it uses the SearchSpaceIds other than 0.

controlResourceSetZero Parameters of the common CORESET#0. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0) the UE operating in a different BWP acquires the CORESET#0 if the conditions defined in 38.213, section 10 are satisfied.

pagingSearchSpace ID of the Search space for paging. Corresponds to L1 parameter 'paging-SearchSpace' (see 38.213, section 10). If the field is absent, the monitoring occasions are derived as described in 38.213, section 10.1 and section 13.

ra-SearchSpace ID of the Search space for random access procedure. Corresponds to L1 parameter 'ra-SearchSpace' (see 38.214?, section FFS_Section) If the field is absent, the monitoring occasions are derived as described in 38.213, section 10.1 and section 13.

searchSpaceOtherSystemInformation ID of the Search space for other system information, i.e., SIB2 and beyond. Corresponds to L1 parameter 'osi-SearchSpace' (see 38.213, section 10) If the field is absent, the monitoring occasions are derived as described in 38.213, section 10.1 and section 13.

searchSpaceSIB1 ID of the search space for SIB1 message. If the field is absent, the monitoring occasions are derived as described in 38.213, section 10.1 and section 13. Corresponds to L1 parameter 'rmsi-SearchSpace' (see 38.213, section 10)

searchSpaceZero Parameters of the common SearchSpace#0. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0) the UE acquires the SearchSpace#0 irrespective of the currently active BWP as described in FFS_Spec, section FFS_Section).

secondControlResourceSetForPaging An additional common control resource set which may be configured and used for paging. Its purpose is to enable frequency-multiplexing of paging occasions, when 2 or 4 paging occasions are configured per paging frame. If it is configured when 2 paging occasions are configured per paging frame, then these two paging occasions are frequency-multiplexed – one associated with controlResourceSetZero and one associated with secondControlResourceSetForPaging. If it is configured when 4 paging occasions are configured per paging frame, then these 4 paging occasions are configured as two time-multiplexed pairs of frequency-multiplexed paging occasions. Within each time-multiplexed pair of paging occasions, one of the frequency-multiplexed paging occasions is associated with controlResourceSetZero and one is associated with secondControlResourceSetForPaging. See further chapter 7.1 in TS 38.304. If the network configures this field, it uses a ControlResourceSetId other than 0 for this ControlResourceSet.

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWP-Only | The field is mandatory present in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in dedicated signalling. It is absent in other BWPs and when sent in system information. |

END OF CHANGES