3GPP TSG-RAN WG2 Meeting #103	R2-1812392

Gothenburg, Sweden, 20 - 24 August 2018													 R2-1810104

Agenda item:	10.4.1.3.10

Source:	Nokia, Nokia Shanghai Bell

Title:	Mobility history information in NR

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In LTE mobility history information is supported, but it not concluded yet whether it is supported in NR.

In this contribution we discuss further mobility history information for NR.

# 2	Discussion

In LTE mobility history information procedure specifies how the mobility history information is stored and provided by the UE, covering RRC_CONNECTED and RRC_IDLE. The UE stores the history information of serving cells as specified in TS 36.331[1]. The UE stores cell identity and time spent on the cells the UE visits in RRC_CONNECTED in RRC_IDLE. The UE stores the information about E-UTRAN cells and inter-RAT cells and information about out of service.

If the UE has stored mobility history information available the IE indicates the mobility history information availability in MSG5 i.e. RRC Connection Setup Complete message or in RRC Connection Resume Complete. Based on the information in MSG5 the network may ask the UE to provide the stored mobility history information.

Based on the provided information the network is aware the of the UEs recent mobility information which can be used e.g. for configuring the connected mode configurations, adjust the connection release timer etc. We think that the mobility history information is useful for the network and therefore we propose the following:

Proposal 1: Mobility history information procedure is supported in NR

Proposal 2: LTE mobility history information procedure specified in 36.331 is used as a baseline

Proposal 2.1: The UE stores time spent on NR cells, Inter-RAT cells and out of service

Proposal 2.2: The UE provides the stored mobility history information after the request from the network

# 3	Conclusion

Based on the discussion the following is proposed:

Proposal 1: Mobility history information procedure is supported in NR

Proposal 2: LTE mobility history information procedure specified in 36.331 is used as a baseline

Proposal 2.1: The UE stores time spent on NR cells, Inter-RAT cells and out of service

Proposal 2.2: The UE provides the stored mobility history information after the request from the network

# References

- 3GPP TS 36.331: "E-UTRA; Radio Resource Control (RRC) - Protocol Specification".