3GPP TSG-RAN WG2#103	R2-1812413

# Gothenburg, Sweden, 20 – 24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [E571, C007, G302] Draft CR to 38.331 Corrections for T304 handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In agreed draft CR for inter-RAT handover, T304 is started in the subclause 5.4.2.3 specific for inter-RAT HO. However, as this timer is configured and started also when Reconfiguration with Sync is processed, there is no need for separate handling of the timer. Similarly, the generic procedure for T304 expiracy can be reused (subclause 5.3.5.8.3 instead of 5.4.2). This is different as compared to LTE where there is separate subclause for T304 expiracy in case of HO to LTE. In addition, as pointed by RIL C007 and G302, T304 handling for MCG is missing. This issue is addressed also in this contribution for compleness. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5.3.5.8.3 T304 expiracy - inter-RAT case is captured - MCG case is captured |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR inter-RAT handover would not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 5.3.5.8.3	T304 expiry (Reconfiguration with sync Failure)

The UE shall:

1>	if T304 of the MCG expires:

2>	revert back to the UE configuration used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in subclause 5.3.7.

NOTE 1:	In the context above, "the UE configuration" includes state variables and parameters of each radio bearer.

1>	else if T304 of a secondary cell group expires:

2>  release dedicated preambles provided in rach-ConfigDedicated;

2>	initiate the SCG failure information procedure as specified in subclause 5.7.3 to report SCG reconfiguration with sync failure, upon which the RRC reconfiguration procedure ends;

1> else if T304 expires when RRCReconfiguration is received via other RAT (HO to NR failure):

2> reset MAC;

2> perform the actions defined for this failure case as defined in the specifications applicable for the other RAT;

## 5.4	Inter-RAT mobility

#### 5.4.1	Introduction

NR support network controlled inter-RAT mobility between NR and E-UTRA which can be connected to either EPC or 5GC.

#### 5.4.2	Handover to NR

##### 5.4.2.1	General

Figure 5.4.2.1-1: Handover to NR, successful

The purpose of this procedure is to, under the control of the network, transfer a connection between the UE and another Radio Access Network (e.g. E-UTRAN) to NR.

The handover to NR procedure applies when SRBs, possibly in combination with DRBs, are established in another RAT. Handover from E-UTRA to NR applies only after integrity has been activated in E-UTRA.

##### 5.4.2.2	Initiation

The RAN using another RAT initiates the handover to NR procedure, in accordance with the specifications applicable for the other RAT, by sending the RRCReconfiguration message via the radio access technology from which the inter-RAT handover is performed.

The network applies the procedure as follows:

-	to activate ciphering, possibly using NULL algorithm, if not yet activated in the other RAT;

-	to re-establish SRBs and one or more DRBs;

##### 5.4.2.3	Reception of the RRCReconfiguration by the UE

The UE shall:

1>	start timer T304 with the timer value set to t304, as included in the reconfigurationWithSync;

- perform RRC reconfiguration procedure as specified in 5.3.5;

NOTE: 	In the case that UE is connected to 5GC in the source E-UTRA cell, the delta configuration can be carried in RRCReconfiguration, e.g. PDCP-config IE and SDAP-config IE can be absent, which means the PDCP entity and SDAP entity established in source cell of the DRBs admitted by target cell could be maintained in the target cell, in order to perform lossless intra-system inter-RAT handover.

##### 5.4.2.4	Reconfiguration failure

The UE shall:

1>	if the UE is unable to comply with any part of the configuration included in the RRCReconfiguration message:

2>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT;

NOTE 1:	The UE may apply above failure handling also in case the RRCReconfiguration message causes a protocol error for which the generic error handling as defined in clause 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/ failure.