3GPP TSG-RAN WG2 Meeting #103	R2-1812421

Gothenburg, Sweden, 20th - 24th August 2018	Revision of R2-1810123

Agenda item:	10.4.1.6.1

Source: 	Huawei, HiSilicon

Title: 	Updated SIB1 size estimate

Document for:	Discussion and Decision

# Introduction

Comparing with the previous version, the following changes are made

- update the estimation according to the latest SA CR

In the RAN2 #101 meeting, we evaluated RMSI size based on the TS 38.331 version of the time and 1700 bits were agreed by most of companies. In the RAN1 #92bis meeting, RAN1 has confirmed that NR can support RMSI size of approximately 1700bits in one TB for both FR1 and FR2 with an appropriate RMSI configuration, and max TBS of 2976 bits is supported for PDSCH by SI-RNTI in [3]. However, more contents have been included in RMSI (e.g., SI-SchedulingInfoList, cellAccessRelatedInfor for multi-PLMN) with the process of specification. Hence, it is necessary to re-estimate the RMSI size.

# Discussion

In this contribution, we re-estimate RMSI size based on [1] with the following assumptions for the values of some parameters.

- For the IE MultiFrequencyBandListNR in FrequencyInfoDownlink and FrequencyInfoUplink, it is defined by

MultiFrequencyBandListNR ::= 		SEQUENCE (SIZE (1..maxNrofMultiBands)) OF FreqBandIndicatorNR

The size of FreqBandIndicatorNR is 10 bits. The value of maxNrofMultiBands is 8.

We assume the number of FreqBandIndicatorNR is 3 instead of 8.

- For the IE pusch-AllocationList in PUSCH-ConfigCommon, it is defined by

pusch-AllocationList					SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation	OPTIONAL, 	-- Need R,

The size of PUSCH-TimeDomainResourceAllocation is 11. The value of maxNrofUL-Allocations is 16 too. Here, we assume there is no PUSCH-TimeDomainResourceAllocation configuration in RMSI based on RAN1 agreement that UE can use default PUSCH-TimeDomainResourceAllocation configuration if no related configuration in RMSI and RRC signalling.

- For the IE pdsch-AllocationList in PDSCH-ConfigCommon, it is defined by

pdsch-AllocationList			SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation			OPTIONAL,	-- Need R

The size of PDSCH-TimeDomainResourceAllocation is 11, and the value of maxNrofDL-Allocations is 16.

Similarly, we assume there is no PDSCH-TimeDomainResourceAllocation in RMSI, and UE uses default configuration.

- Assume there are 3 PLMN totally and they share common TA, Cell ID, SIAID configuration.

- Assume maximum 4 SI message with 8 specified SIBs.

- Assume 200 bits access control barring parameters

The estimation based above assumptions can be seen as a basic estimation. The following table shows the estimation results. (The calculation details see Appendix in section 5)

Table 1 RMSI size estimation

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| RMSI | 2603 | 5 | Total: 2608 |
| cellSelectionInfo SEQUENCE { q-RxLevMin Q-RxLevMin, q-RxLevMinSUL Q-RxLevMin OPTIONAL, -- Need N q-QualMin Q-QualMin OPTIONAL -- Need N } OPTIONAL | 19 | 1 |  |
| cellAccessRelatedInfo CellAccessRelatedInfo, | 180 |  |  |
| si-SchedulingInfo SI-SchedulingInfo OPTIONAL, | 226 |  |  |
| servingCellConfigCommon ServingCellConfigCommonSIB OPTIONAL, | 1958 | 1 |  |
| ims-EmergencySupport ENUMERATED {true} OPTIONAL, | 1 | 1 |  |
| eCallOverIMS-Support ENUMERATED {true} OPTIONAL | 1 | 1 |  |
| ue-TimersAndConstants UE-TimersAndConstants OPTIONAL, | 18 | 1 |  |
| AC Barring Info | 200 |  |  |

The variation of the values of those parameters has a big influence on the final RMSI size. In the following table, we provide RMSI size with different number of FreqBandIndicatorNR, PDSCH-TimeDomainResourceAllocation, PUSCH-TimeDomainResourceAllocation, PLMN, SI, and SIBs.

|  | Basic estimation | Worse case |  | Worst case |  |
| --- | --- | --- | --- | --- | --- |
|  |  | value | Additional bits consumed | value | Additional bits consumed |
| The number of FreqBandIndicatorNR | 3 | 6 | 90 bits | 8 | 150 bits |
| The number of PDSCH-TimeDomainResourceAllocation | 0 | 8 | 88 bits | 16 | 176 bits |
| The number of PUSCH-TimeDomainResourceAllocation | 0 | 8 | 176 bits | 16 | 352 bits |
| The number of PLMNs | 3 | 6 | 161 bits | 12 | 483 bits |
| The number of SI and SIBs | 3 SI for 8 SIBs | 8 SI for 8 SIBs | 56 bits | 10 SI for 16 SIBs | 164 bits |
| RMSI size [bits] | 2608 | 3219 |  | 3618 |  |

Note 1: Additional bits consumed are based on basic estimation.

Note 2: We assume 3 PLMNs are one group and they share common TA, Cell ID, SIAID configuration (i.e., 4 groups of related configuration to 12 PLMNs).

According to the above table, it can be observed that the RMSI size for most of configuration scenarios are under the maximum TB size 2976 bits provided by RAN1, but it will exceed this value in some configuration scenarios.

Observation: RMSI size can satisfy the maximum TB size limitation in most of configuration scenarios.

# Reference

- R2-1808190	[101bis#13][NR] Text Proposal on the ASN.1 for SIB content.	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core

- R2-1803356	The maximum size of RMSI	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core

# Conclusion

In this contribution, we re-estimate RMSI size and we get the following observation:

Observation: RMSI size can satisfy the maximum TB size limitation in most of configuration scenarios.

# Appendix

### –	RMSI

Table 2 RMSI size analysis

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| RMSI | 2603 | 5 | Total: 2608 |
| cellSelectionInfo SEQUENCE { q-RxLevMin Q-RxLevMin, q-RxLevMinSUL Q-RxLevMin OPTIONAL, -- Need N q-QualMin Q-QualMin OPTIONAL -- Need N } OPTIONAL | 19 | 1 |  |
| cellAccessRelatedInfo CellAccessRelatedInfo, | 180 |  |  |
| si-SchedulingInfo SI-SchedulingInfo OPTIONAL, | 226 |  |  |
| servingCellConfigCommon ServingCellConfigCommonSIB OPTIONAL, | 1958 | 1 |  |
| ims-EmergencySupport ENUMERATED {true} OPTIONAL, | 1 | 1 |  |
| eCallOverIMS-Support ENUMERATED {true} OPTIONAL | 1 | 1 |  |
| ue-TimersAndConstants UE-TimersAndConstants OPTIONAL, | 18 | 1 |  |
| AC Barring Info | 200 |  |  |

#### - ServingCellConfigCommonSIB

| Parameters | Size [bits] | Additional size | Notes |
| --- | --- | --- | --- |
| ServingCellConfigCommonSIB | 1950 | 8 | Total: 1958 |
| dlConfigCommon DownlinkConfigCommonSIB, | 723 |  |  |
| uplinkConfigCommon UplinkConfigCommonSIB OPTIONAL, | 381 | 1 |  |
| supplementaryUplink UplinkConfigCommonSIB OPTIONAL | 381 | 1 |  |
| n-TimingAdvanceOffset ENUMERATED { n25560, n39936 } OPTIONAL, -- Need S | 1 | 1 |  |
| ssb-PositionsInBurst SEQUENCE { inOneGroup BITSTRING (SIZE (8)), groupPresence BITSTRING (SIZE (8)) OPTIONAL -- Cond Above6GHzOnly }, | 8 | 1 |  |
| lte-CRS-ToMatchAround RateMatchPatternLTE-CRS OPTIONAL, -- Need R | 23 | 1 |  |
| rateMatchPatternList SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern OPTIONAL, | 360 | 1 |  |
| ssb-periodicityServingCell ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 } OPTIONAL | 3 | 1 |  |
| dmrs-TypeA-Position ENUMERATED {pos2, pos3}, | 1 |  |  |
| tdd-UL-DL-ConfigurationCommon TDD-UL-DL-ConfigCommon OPTIONAL, | 62 | 1 |  |
| ss-PBCH-BlockPower INTEGER (-60..50) | 7 |  |  |

##### - DownlinkConfigCommonSIB

| Parameters | Size [bits] | Additional bits | Notes |
| --- | --- | --- | --- |
| DownlinkConfigCommonSIB | 721 | 2 | Total: 723 |
| frequencyInfoDLSIB FrequencyInfoDLSIB, | 52 |  |  |
| pdcch-ConfigCommon PDCCH-ConfigCommon OPTIONAL, | 661 | 1 |  |
| pdsch-ConfigCommon PDSCH-ConfigCommon OPTIONAL, | 1 | 1 |  |
| bcch-Config BCCH-Config, | 2 |  |  |
| pcch-Config PCCH-Config, | 5 |  |  |

FrequencyInfoDLSIB ::= 				SEQUENCE {

frequencyBandList					MultiFrequencyBandListNR, maximum 3*10

absoluteFrequencyPointA					ARFCN-ValueNR   22 bits

}

###### - PDCCH-ConfigCommon

Table 3 PDCCH-ConfigCommon

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| PDCCH-ConfigCommon | 659 | 2 | Total:661 |
| CORESET for RAR | 470 | 1 |  |
| commonSearchSpaces | 3*57 | 1 |  |
| Search space for OSI | 6 |  |  |
| Search space for paging | 6 |  |  |
| Search space for RAR | 6 |  |  |

###### -PDSCH-ConfigCommon

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| PDSCH-ConfigCommon | 0 | 1 | Total:1 |
| pdsch-AllocationList SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation | 0 | 1 | Use default PDSCH-TimeDomainResourceAllocation |

maxNrofDL-Allocations=16

-PDSCH-TimeDomainResourceAllocation

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| PDSCH-TimeDomainResourceAllocation | 10 | 1 | Total:11 |
| k0 INTEGER (1..3) OPTIONAL, -- | 2 | 1 |  |
| commonSearchSpaces mappingType ENUMERATED {typeA, typeB}, | 1 |  |  |
| startSymbolAndLength BIT STRING (SIZE (7)) | 7 |  |  |

#### -UplinkConfigCommon

Table 4 UplinkConfigCommon

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| UplinkConfigCommon | 379 | 2 | Total: 381 |
| frequencyInfoUL FrequencyInfoUL OPTIONAL | 197 | 1 |  |
| initialUplinkBandwidthPart UplinkBandwidthPart OPTIONAL | 182 | 1 |  |

##### - FrequencyInfoUL

Table 5 FrequencyInfoUL

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| FrequencyInfoUL | 192 | 5 | Total: 197 |
| absoluteFrequencyPointA ARFCN-ValueNR OPTIONAL, | 22 | 1 |  |
| frequencyBandList MultiFrequencyBandListNR OPTIONAL | 3*10 | 1 | Assume there are 3 FreqBandIndicatorNR |
| scs-SpecificCarriers SEQUENCE (SIZE (1..ffsValue)) OF SCS-SpecificVirtualCarrie OPTIONAL, | 5*26 |  |  |
| additionalSpectrumEmission AdditionalSpectrumEmission OPTIONAL, | 3 | 1 |  |
| p-Max P-Max OPTIONAL, | 6 | 1 |  |
| frequencyShift7p5khz ENUMERATED {true} OPTIONAL, | 1 | 1 |  |

###### -SCS-SpecificVirtualCarrie

Table 6 SCS-SpecificVirtualCarrier

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| SCS-SpecificVirtualCarrier | 26 |  | Total: 26 |
| offsetToVirtualCarrier INTEGER (0..2199) | 12 |  |  |
| subcarrierSpacing SubcarrierSpacing | 3 |  |  |
| k0 ENUMERATED {n-6, n0, n6}, | 2 |  |  |
| carrierBandwidth INTEGER (1..maxNrofPhysicalResourceBlocks) , | 9 |  |  |

Notes:

SubcarrierSpacing ::=  ENUMERATED {kHz15, kHz30, kHz60, kHz120}

maxNrofPhysicalResourceBlocks INTEGER ::= 275

##### -UplinkBandwidthPart

Table 7 UplinkBandwidthPart

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| UplinkBandwidthPart | 176 | 6 | Total: 182 |
| genericParameters BWP | 21 |  |  |
| rach-ConfigCommon SetupRelease { RACH-ConfigCommon } OPTIONAL, , | 113 | 2 |  |
| pusch-ConfigCommon SetupRelease { PUSCH-ConfigCommon } OPTIONAL, | 16 | 2 |  |
| pucch-ConfigCommon SetupRelease { PUCCH-ConfigCommon } OPTIONAL, | 26 | 2 |  |

###### - BWP

Table 8 BandwidthPart

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| BWP | 20 | 1 | Total: 21 |
| locationAndBandwidth INTEGER (1.. 37949) , | 16 |  |  |
| subcarrierSpacing SubcarrierSpacing | 3 |  |  |
| cyclicPrefix ENUMERATED { extended } OPTIONAL | 1 | 1 |  |

Notes:

BandwidthPartId	::=  INTEGER (0..maxNrofBandwidthParts-1)

maxNrofBandwidthParts  INTEGER ::= 4

###### -RACH-ConfigCommon (Agreed in the RAN2 AH(1801))

Table 9 RACH-ConfigCommon

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| RACH-ConfigCommon | 103 | 10 | Total: 113 |
| prach-ConfigCommonGeneric PRACH-ConfigGeneric, | 39 |  |  |
| totalNumberOfRA-Preambles INTEGER (1..63) OPTIONAL | 6 | 1 |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { oneEighth ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, oneFourth ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, oneHalf ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, one ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, two ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32}, four INTEGER (1..16), eight INTEGER (1..8), sixteen INTEGER (1..4) } | 4 | 3 |  |
| groupBconfigured SEQUENCE { ra-Msg3SizeGroupA ENUMERATED {b56, b144, b208, b256, b282, b480, b640, b800, b1000, spare7, spare6, spare5, spare4, spare3, spare2, spare1}, messagePowerOffsetGroupB ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18}, numberOfRA-PreamblesGroupA INTEGER (1..64) } OPTIONAL | 13 | 1 |  |
| ra-ContentionResolutionTimer ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64} | 3 |  |  |
| ssb-Threshold RSRP-Range OPTIONAL | 7 | 1 |  |
| sul-RSRP-Threshold RSRP-Range OPTIONAL, | 7 | 1 |  |
| prach-ConfigurationIndex INTEGER (0..255) OPTIONAL, | 8 | 1 |  |
| prach-RootSequenceIndex CHOICE { l839 INTEGER (0..837), l139 INTEGER (0..137) } | 10 | 1 |  |
| msg1-SubcarrierSpacing SubcarrierSpacing | 3 |  |  |
| restrictedSetConfig ENUMERATED {unrestricted, restrictedToTypeA, restrictedToTypeB}, | 2 |  |  |
| msg3-transformPrecoding ENUMERATED {true} OPTIONAL | 1 | 1 |  |

Notes:

RSRP-Range ::=					ENUMERATED {ffsTypeAndValue}

RSRQ-Range ::=					ENUMERATED {ffsTypeAndValue}

ControlResourceSetId ::=					INTEGER (0..maxNrofControlResourceSets-1)

maxNrofControlResourceSets 				INTEGER ::= ffsValue 	-- Max number of CoReSets configurable on a serving cell

maxNrofControlResourceSets-1			INTEGER ::= 12

- SearchSpace

Table 10 SearchSpace

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| SearchSpace | 41 | 6 | Total: 47 |
| searchSpaceId SearchSpaceId | 3 |  |  |
| controlResourceSetId ControlResourceSetId, | 4 |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { sl1 NULL, sl2 INTEGER (0..1), sl5 INTEGER (0..4), sl10 INTEGER (0..9), sl20 INTEGER (0..19) } OPTIONAL, | 5 | 2 |  |
| monitoringSymbolsWithinSlot BIT STRING (SIZE (14)) OPTIONAL, | 14 | 1 |  |
| nrofCandidates SEQUENCE { aggregationLevel1 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, aggregationLevel2 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, aggregationLevel4 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, aggregationLevel8 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, aggregationLevel16 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8} }, | 15 | 1 |  |
| searchSpaceType CHOICE { common SEQUENCE { -- FFS: Parameters that are applicable for CSS? }, ue-Specific SEQUENCE { -- FFS: Parameters that are applicable only for USS? } } OPTIONAL - | 1 | 2 |  |

Notes:

maxNrofSearchSpaces 					INTEGER ::= ffsValue

###### - PUSCH-ConfigCommon

Table 11 PUSCH-ConfigCommon

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| PUSCH-ConfigCommon | 12 | 4 | Total: 16 |
| groupHoppingEnabledTransformPrecoding ENUMERATED {enabled} OPTIONAL, | 1 | 1 |  |
| pusch-AllocationList SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation OPTIONAL, -- Need R | 0*11 | 1 | Use default PUSCH-TimeDomainResourceAllocation |
| msg3-DeltaPreamble INTEGER (-1..6) OPTIONAL, | 3 | 1 |  |
| p0-NominalWithGrant INTEGER (-202..24) OPTIONAL, | 8 | 1 |  |

maxNrofUL-Allocations=16

###### - PUCCH-ConfigCommon

Table 12 PUCCH-ConfigCommon

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| PUCCH-ConfigCommon | 23 | 3 | Total: 26 |
| pucch-ResourceCommon BIT STRING (SIZE (4)) OPTIONAL, | 4 | 1 |  |
| pucch-GroupHopping ENUMERATED { neither, enable, disable }, | 2 |  |  |
| hoppingId BIT STRING (SIZE (10)) OPTIONAL | 10 | 1 |  |
| p0-nominal INTEGER (-202..24) OPTIONAL, | 7 | 1 |  |

#### - TDD-UL-DL-ConfigCommon

Table 13 TDD-UL-DL-ConfigCommon

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| TDD-UL-DL-ConfigCommon | 57 | 5 | Total: 62 |
| refereceSubcarrierSpacing SubcarrierSpacing OPTIONAL | 3 |  |  |
| pattern1 TDD-UL-DL-Pattern, | 27 |  |  |
| pattern2 TDD-UL-DL-Pattern OPTIONAL | 27 | 1 |  |

Notes:

maxSymbolIndex							INTEGER ::= 13

#### - CellAccessRelatedInfo

| Parameters | Size [bits] | Additional bits | Notes |
| --- | --- | --- | --- |
| plmn-IdentityList | 1*161 |  | Consider 3 PLMN share common TA, Cell ID, SIAID configuration |
| ranac | 6 | 1 |  |
| reservedForFutureUse | 1 | 1 |  |
| Total: | 180 |  |  |

PLMN-IdentityInfoList ::=				SEQUENCE (SIZE (1..maxPLMNInfo)) OF PLMN-IdentityInfo

| Parameters | Size [bits] | Additional bits | Notes |
| --- | --- | --- | --- |
| plmn-IdentityList | 3*25 |  | Consider 3 PLMN |
| trackingAreaCode | 24 |  |  |
| cellIdentity | 36 |  |  |
| systemInformationAreaID | 24 | 1 |  |
| cellReservedForOperatorUse | 1 |  |  |
| Total: | 161 |  |  |

#### -CellSelectionInfo

| Parameter | Size [bits] | Comment |
| --- | --- | --- |
| q-RxLevMin | 6 |  |
| q-RxLeyMin for SUL | 6+1=7 |  |
| q-RxLevMinOffset | 5+1=6 |  |
| total | 19 |  |

#### -SI-schedulingInfoList

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| SI-SchedulingInfo | 222 | 4 | Total: 226 |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo | 4*4+8*4 |  | Assume 4 SI message |
| si-WindowLength ENUMERATED {ms1, ms2, ms5, ms10, ms15, ms20, ms40}, | 3 |  |  |
| si-Request-Config SI-Request-Config OPTIONAL | 83 | 1 |  |
| systemInformationAreaID BIT STRING (SIZE (24)) OPTIONAL, | 24 | 1 |  |
| sibValueTagList SEQUENCE (SIZE (1..maxSIB-1)) OF SIBValueTag, | 8*5 | 1 | Assume 8 SIBs |
| systemInfoAreaScope SystemInfoAreaScope OPTIONAL, | 24 | 1 |  |

| SchedulingInfo ::= SEQUENCE { si-BroadcastStatus ENUMERATED {broadcast, onDemand}, si-Periodicity ENUMERATED { rf8, rf16, rf32, rf64, rf128, rf256, rf512}, sib-MappingInfo SIB-Mapping } | The Maximum can be up to 36 bits |
| --- | --- |
| SIB-Mapping ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type | 8*4=32bits with assuming 8 SIBs |
| SIB-Type ::= ENUMERATED { sibType2, sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ... } | 4 bits |

SIBValueTag ::=		INTEGER (0..31)

##### - SI-Request-Config

| Parameters | Size | Additional bits | Comments |
| --- | --- | --- | --- |
| SI-Request-Config | 82 | 1 | Total: 83 |
| rach-OccasionsSI SEQUENCE { rach-ConfigSI RACH-ConfigGeneric, ssb-perRACH-Occasion ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen} } OPTIONAL, | 42 | 1 |  |
| si-Request-Resources ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SI-Request-Resources | 4*10 |  | Assume 4 SI message |

SI-Request-Resources ::=            SEQUENCE {

ra-PreambleStartIndex         INTEGER (0..63),

ra-ssb-OccasionMaskIndex      INTEGER (0..15)

}

#### -AC Barring Info

In [R2-1806449], Nokia provided the following table which presents the ACB parameter size calculation with different number access categories and set of barring parameters. (The "worst case" presents the maximum size (with 12 PLMNs), while the example column presents the case of a single PLMN.) Finally, option 1 is agreed in the last RAN2 meeting.

|  | 63 ACs 8 Sets of Barring |  | 63 ACs 4 Sets of Barring |  | 15 ACs 8 Sets of Barring |  | 15 ACs 4 Sets of Barring |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Worst Case | Example | Worst Case | Example | Worst Case | Example | Worst Case | Example |
| Number of Access Categories | 63 | 63 | 63 | 63 | 15 | 15 | 15 | 15 |
| Number of Set of Barring parameters | 8 | 8 | 4 | 4 | 8 | 8 | 4 | 4 |
| Number of PLMNs | 12 | 1 | 12 | 1 | 12 | 1 | 12 | 1 |
| Option 1: Index option | 2268 | 189 | 1512 | 126 | 540 | 45 | 360 | 30 |
| Option 2: Bitmap option | 6048 | 504 | 3024 | 252 | 1440 | 120 | 720 | 60 |

200 bits may be enough for AC barring parameters in real networks.