3GPP TSG-RAN WG2 Meeting #103			 R2-1812519

Gothenburg, Sweden, August 20th – 24th 2018

Agenda Item:	11.1.2 (FS_NR_IAB)

Source: 	LG Electronics Inc.

Title:         	TP for IAB Flow Control and Congestion Handling

Document for: 	Discussion and Decision

Introduction

This document proposes a text proposal for TR38.374 based on the summary of the following email discussion.

[AH1807#19][IAB] IAB Flow Control and Congestion Handling (LG)

Intended outcome: Report, a) describe problem b) identify solutions (on a high level), to be put on the table for next meeting.

Deadline:  Thursday 2018-08-02

Text proposal for TR38.374

Start of Change

### 8.2.5	Flow control and congestion handling

For the uplink, an intermediate IAB node acts as a gNB to child IAB nodes and can control the amount of uplink data from child IAB nodes and UEs by adjusting the UL grant. The uplink data may be congested but flow control mechanism is not considered for the uplink data congestion.

For the downlink, the downlink backhaul link may have higher capacity than the access link. In an intermediate IAB node, the capacity of a link with a child IAB node may be different to the capacity of a link with a parent IAB node. The DU side of the parent IAB node may not know exact downlink buffer status of the child IAB node. The downlink data may be congested and discarded at the intermediate IAB node. Flow control mechanism is considered as a solution for the downlink data congestion.

The parent IAB node or the IAB donor may handle downlink data congestion with feedback information. The feedback information may be forwarded from the congested IAB node to the parent IAB node or the IAB donor.

Content included in the feedback information

The study will identify all information to be included in the feedback. This may include:

- Buffer status

- IAB node id

- Potentially other information

The granularity of the feedback information is FFS, e.g. per UE radio bearer, per RLC-channel, per backhaul link.

The further study includes hop-by-hop and end-to-end flow control for downlink data congestion.

### 8.2.6	Specification Impact

The study item will identify the specification impact for the following two options for IAB:

- Option 1: Adaptation layer is placed above the RLC.

- Option 2: Adaptation layer is placed above the MAC

The specification impact in this context is defined relative to Rel 15 NR.

End of Change