3GPP TSG-RAN WG2 Meeting #103 	R2-1812669

# Gothenberg, Sweden, 20th August 2018 – 24th August 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0380 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on DRX inactivity timer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 defines a BWP switching delay, during which UE is not required to transmit UL signals or receive DL signals. For drx-InactivityTimer, it starts or restarts in the first symbol after the end of the PDCCH reception. However, if the PDCCH indicates BWP switching, it’s not clear the wether the first symbol starting/restarting drx-InactivityTimer refer to the symbol in the old BWP or new BWP. Besides, it’s not clear whether UE needs to monitor PDCCH during the BWP switching delay. In order to make the timer behaviour clear, it’s beneficial to make it start/restart in the first symble after BWP switching delay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Start/restart drx-InactivityTimer in the first symble after BWP switching delay. Impact analysis Impacted functionality: DRX operation Inter-operability: - If the network is implemented according to the CR and the UE is not there are no inter-operability problems. - If the UE is implemented according to the CR and the network is not, there are no inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It’s not clear whether UE needs to monitor PDCCH during the BWP switching delay when drx-InactivityTimer is running. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

## 5.7	Discontinuous Reception (DRX)

[Omit]

1>	if the MAC entity is in Active Time:

2>	monitor the PDCCH;

2>	if the PDCCH indicates a DL transmission or if a DL assignment has been configured:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH transmission;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL) without BWP switching indication:

3>	start or restart drx-InactivityTimer in the first symbol after the end of the PDCCH reception.

2>	if the PDCCH indicates a new transmission (DL or UL) with BWP switching indication:

3>	start or restart drx-InactivityTimer in the first symbol after BWP switching delay defined in TS 38.133.

1>	else (i.e. not part of the Active Time):

2>	not transmit type-0-triggered SRS defined in TS 38.214 [7].

1>	if CQI masking (cqi-Mask) is setup by upper layers:

2>	if drx-onDurationTimer is not running:

3>	not report CSI on PUCCH.

1>	else:

2>	if the MAC entity is not in Active Time:

3>	not report CSI on PUCCH.

End of change