3GPP TSG-RAN WG2 #103			    R2-1812674

Gothenburg, Sweden, 20th – 24th August, 2018

Agenda item:	   10.3.4.4 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 	Asymmetric mapping for UL and DL QoS flows

Document for:	Discussion and Decision

1.	Introduction

At the RAN2 AH#4 meeting, RAN2 discussed on asymmetric mapping for UL and DL QoS flows, and made the following agreements:

The L2 currently support mapping of UL and DL on different DRBs

Postpone Stage-2 and RRC change to next meeting (allow time to think).

In this document, we are discussing if asymmetric mapping is required.

2.	Discussion

gNB controls a mapping of QoS flow to DRB in two different ways: Explicit configuration by RRC and Reflective mapping. These ways can also support asymmetric mapping as well as symmetric mapping, as follows:

|  | Explicit Configuration | Reflective Mapping |
| --- | --- | --- |
| Symmetric Mapping | By including UL QoS flow in mappedQoS-FlowsToAdd field of SDAP-Config for the DRB to which DL QoS flow has been mapped | By setting RDI of DL SDAP PDU to ‘1’ when DL QoS flow is (re)mapped |
| Asymmetric Mapping | By including UL QoS flow in mappedQoS-FlowsToAdd field of SDAP-Config for different DRB | By setting RDI of DL SDAP PDU to ‘0’ when DL QoS flow is (re)mapped |

UL and DL for a QoS flow can be mapped to different DRBs in the current specifications. Nevertheless, we think RAN2 need to check if there is a use case for the asymmetric mapping because the asymmetric mapping has some impacts on RAN3. The mapping information decided by a gNB is transmitted to another gNB in the specific cases (e.g., handover and dual connectivity). RAN3 have designed the mapping information of the following messages based on the symmetric mapping, not the asymmetric mapping:

- Handover Request;

- Retrieve UE Context Response;

- UE Context Modification Request;

- S-Node Addition Request;

- S-Node Modification Request;

- S-Node Addition Request Acknowledge;

- S-Node Modification Request Acknowledge.

As mentioned in [1], some QoS parameters - UE Aggregate Maximum Bit Rate, Maximum Flow Bit Rate, Guaranteed Flow Bit Rate and Maximum Packet Loss Rate - can be configured differently for DL and UL. But, we don’t think the asymmetric mapping is necessary by the parameters. Such similar QoS parameters have already existed in LTE, as the following Figure. UL/DL QoS parameters for an EPS bearer can have different values, nevertheless UL and DL traffic are mapped to one DRB associated with the EPS bearer. Like LTE, a QoS flow, whose UL/DL QoS parameters have different values, would be mapped to the same DRB.

So, we couldn’t find any reasons why the asymmetric mapping is necessary. If most network venders think that the asymmetric mapping should be allowed, we will not oppose it. However, regardless of whether to allow the asymmetric mapping, there is no need to include any descriptions in the stage-2 because the asymmetric mapping can already be supported in the stage-3. Therefore, we propose not to add any descriptions related to the asymmetric mapping.

Proposal 1: There is no need to capture any descriptions related to the asymmetric mapping in the stage-2.

If RAN2 make a decision that the asymmetric mapping should be allowed, RAN2 have to ask RAN3 to redesign mapping information, which is transmitted in the specific cases (e.g., handover and dual connectivity), in consideration of the asymmetric mapping.

Proposal 2: A LS on the asymmetric mapping should be sent to RAN3 if RAN2 make a decision that the asymmetric mapping for UL/DL QoS flows can be allowed.

3.	Proposal

In this document, we present our view on the asymmetric mapping. We have following proposals:

Proposal 1: There is no need to capture any descriptions related to the asymmetric mapping in the stage-2.

Proposal 2: A LS on the asymmetric mapping should be sent to RAN3 if RAN2 make a decision that the asymmetric mapping for UL/DL QoS flows can be allowed.

References

[1] R2-1809974	DL and UL QoS Flow to DRB mapping	Huawei, HiSilicon