3GPP TSG-RAN2 Meeting RAN2#103	R2-1812780

# Gothenburg, Sweden, 20th - 24th Aug 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.324 | CR | 0006 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections for SDAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-07-19 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - Since the PDCP entity sevices the out-of-order delivery to the SDAP entity only when out-of-order delivery is configured, the text is changed from ‘in-order delivery except when reordering functionality is disabled by RRC [3]’ to ‘in-order delivery except when out of order delivery is configured by RRC [3]’. - Change of the ‘SDAP PDU’ to ‘SDAP data PDU’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In section 4.3.2, change the text to ‘in-order delivery except when out of order delivery is configured by RRC’. - In section 4.2.2, 5.2.2 and 5.3.2, change ‘SDAP PDU’ to ‘SDAP data PDU’. Impact analysis: Impacted functionality: There is no impaceted functionality. Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect information will be included in section 4.3.2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 4.3.2, 5.2.2, 5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of changes

4.2.2	SDAP entities

The SDAP entities are located in the SDAP sublayer. Several SDAP entities may be defined for a UE. There is an SDAP entity configured for each individual PDU session.

An SDAP entity receives/delivers SDAP SDUs from/to upper layers and submits/receives SDAP data PDUs to/from its peer SDAP entity via lower layers.

-	At the transmitting side, when an SDAP entity receives an SDAP SDU from upper layers, it constructs the corresponding SDAP data PDU and submits it to lower layers;

-	At the receiving side, when an SDAP entity receives an SDAP data PDU from lower layers, it retrieves the corresponding SDAP SDU and delivers it to upper layers.

Start of next changes

4.3.2	Services expected from lower layers

An SDAP entity expects the following services from lower layers:

-	user plane data transfer service;

-	in-order delivery except when out of order delivery is configured by RRC [3].

Start of next changes

5.2.2	Downlink

At the reception of an SDAP data PDU from lower layers for a QoS flow, the receiving SDAP entity shall:

-	if the DRB from which this SDAP data PDU is received is configured by RRC [3] with the presence of SDAP header:

-	perform reflective QoS flow to DRB mapping as specified in the subclause 5.3.2;

-	perform RQI handling as specified in the subclause 5.4;

-	retrieves the SDAP SDU from the DL SDAP data PDU as specified in the subclause 6.2.2.2;

-	else:

-	retrieve the SDAP SDU from the DL SDAP data PDU as specified in the subclause 6.2.2.1;

-	deliver the retrieved SDAP SDU to the upper layer.

Start of next changes

5.3.2	Reflective mapping

For each received DL SDAP data PDU with RDI set to 1, the SDAP entity shall:

-	process the QFI field in the SDAP header and determine the QoS flow;

-	if there is no stored QoS flow to DRB mapping rule for the QoS flow and a default DRB is configured:

-	construct an end-marker control PDU, as specified in subclause 6.2.3, for the QoS flow;

-	map the end-marker control PDU to the default DRB;

-	submit the end-marker control PDU to the lower layers;

-	if the stored QoS flow to DRB mapping rule for the QoS flow is different from the QoS flow to DRB mapping of the DL SDAP data PDU and the DRB according to the stored QoS flow to DRB mapping rule is configured by RRC [3] with the presence of SDAP header:

-	construct an end-marker control PDU, as specified in subclause 6.2.3, for the QoS flow;

-	map the end-marker control PDU to the DRB according to the stored QoS flow to DRB mapping rule;

-	submit the end-marker control PDU to the lower layers;

-	store the QoS flow to DRB mapping of the DL SDAP data PDU as the QoS flow to DRB mapping rule for the UL.

End of changes