3GPP TSG-RAN WG2 NR Ad Hoc 1807	R2-1812783

Gothenburg, Sweden, 20th - 24th August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0391 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on SR triggering condition regarding SR mask – Option 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the current specification, even when there is a dynamic grant available, an SR is triggered as long as the MAC entity is configured with configured grant(s) and the Regular BSR was not triggered by an LCH for which SR masking is setup. If this operation does not cause any problem and it is rather suitable to consider the dynamic grant same as the configured grant for SR masking operation, the SR triggering condition regarding SR masking does not need to be described only for configured grant(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made: - In subclause 5.4.5: In the SR triggering condition regarding SR masking, the first phrase is revised to include dynamic grant(s) as well as configured grant(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The normative text of SR triggering condition regarding SR masking may cause misunderstanding about the resulted UE behaviour. Impact analysis Impacted functionality The changes only impact SR triggering procedure. Inter-operability No inter-operability issue is foreseen. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

### 5.4.5	Buffer Status Reporting

The Buffer Status reporting (BSR) procedure is used to provide the serving gNB with information about UL data volume in the MAC entity.

RRC configures the following parameters to control the BSR:

-	periodicBSR-Timer;

-	retxBSR-Timer;

-	logicalChannelSR-DelayTimerApplied;

-	logicalChannelSR-DelayTimer;

-	logicalChannelSR-Mask;

-	logicalChannelGroup.

Each logical channel may be allocated to an LCG using the logicalChannelGroup. The maximum number of LCGs is eight.

The MAC entity determines the amount of UL data available for a logical channel according to the data volume calculation procedure in TSs 38.322 and 38.323 [3] [4].

A BSR shall be triggered if any of the following events occur:

-	the MAC entity has new UL data available for a logical channel which belongs to an LCG; and either

-	the new UL data belongs to a logical channel with higher priority than the priority of any logical channel containing available UL data which belong to any LCG; or

-	none of the logical channels which belong to an LCG contains any available UL data.

in which case the BSR is referred below to as 'Regular BSR';

-	UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC CE plus its subheader, in which case the BSR is referred below to as 'Padding BSR';

-	retxBSR-Timer expires, and at least one of the logical channels which belong to an LCG contains UL data, in which case the BSR is referred below to as 'Regular BSR';

-	periodicBSR-Timer expires, in which case the BSR is referred below to as 'Periodic BSR'.

For Regular BSR, the MAC entity shall:

1>	if the BSR is triggered for a logical channel for which logicalChannelSR-DelayTimerApplied is configured by upper layers:

2>	start or restart the logicalChannelSR-DelayTimer.

1>	else:

2>	if running, stop the logicalChannelSR-DelayTimer.

For Regular and Periodic BSR, the MAC entity shall:

1>	if more than one LCG has data available for transmission when the MAC PDU containing the BSR is to be built:

2>	report Long BSR for all LCGs which have data available for transmission.

1>	else:

2>	report Short BSR.

For Padding BSR:

1>	if the number of padding bits is equal to or larger than the size of the Short BSR plus its subheader but smaller than the size of the Long BSR plus its subheader:

2>	if more than one LCG has data available for transmission when the BSR is to be built:

3>	if the number of padding bits is equal to the size of the Short BSR plus its subheader:

4>	report Short Truncated BSR of the LCG with the highest priority logical channel with data available for transmission.

3>	else:

4>	report Long Truncated BSR of the LCG(s) with the logical channels having data available for transmission following a decreasing order of priority, and in case of equal priority, in increasing order of LCGID.

2>	else:

3>	report Short BSR.

1>	else if the number of padding bits is equal to or larger than the size of the Long BSR plus its subheader:

2>	report Long BSR for all LCGs which have data available for transmission.

For BSR triggered by retxBSR-Timer expiry, the MAC entity considers that the logical channel that triggered the BSR is the highest priority logical channel that has data available for transmission at the time the BSR is triggered.

The MAC entity shall:

1>	if the Buffer Status reporting procedure determines that at least one BSR has been triggered and not cancelled:

2>	if UL-SCH resources are available for a new transmission:

3>	instruct the Multiplexing and Assembly procedure to generate the BSR MAC CE(s);

3>	start or restart periodicBSR-Timer except when all the generated BSRs are long or short Truncated BSRs;

3>	start or restart retxBSR-Timer.

2>	if a Regular BSR has been triggered and logicalChannelSR-DelayTimer is not running:

3>	if there is no UL-SCH resource available for a new transmission; or

3>	if the MAC entity has UL-SCH resources available for a new transmission and the Regular BSR was not triggered for a logical channel for which logical channel SR masking (logicalChannelSR-Mask) is setup by upper layers; or

3>	if the UL-SCH resources available for a new transmission do not meet the LCP mapping restrictions (see subclause 5.4.3.1) configured for the logical channel(s) that triggered the BSR(s):

4>	trigger a Scheduling Request.

A MAC PDU shall contain at most one BSR MAC CE, even when multiple events have triggered a BSR. The Regular BSR and the Periodic BSR shall have precedence over the padding BSR.

The MAC entity shall restart retxBSR-Timer upon reception of a grant for transmission of new data on any UL-SCH.

All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader. All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU which includes a BSR MAC CE is transmitted.

<End of modification>