3GPP TSG-RAN WG2 Meeting #103	R2-1812813

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda item:	7.1.1.2

Source:		ITL

Title:		Discussion on RACH carrier selection for NR-U

Document for:	Discussion and Decision

Introduction

The RACH for NR-U has been discussed together with other technical issues in last RAN2 AH meeting and made the following agreements related to the RACH, focusing on the feasibility of Rel-14 LAA agreements [1]:

From Rel-14 LAA Agreements: Both CBRA and CFRA are supported on NR-U SpCell and CFRA is supported on NR-U SCells. At the first stage, RAR can be transmitted via SpCell Assume we Use a predefined HARQ process ID for RAR

In this contribution, we seek to further discuss RACH carrier selection issue for NR-U and present our view.

RACH carrier selection

The RACH procedure of Rel-15 NR has been defined as following order in Figure 1. Once a RACH event (e.g. SCell addition) is triggered, the initialization step for preamble transmission will be followed. In RACH initialization step, the carrier selection step should be performed when a UE is configured with SUL carrier for RACH and is not explicitly configured to use the carrier. In this case, whether or not to select NUL/SUL is up to the RSRP value of DL pathloss reference measured by the UE. That is, if the DL pathloss reference RSRP value is less than rsrp-ThresholdSSB-SUL, the UE selects the SUL carrier for performing RACH procedure and set the PCMAX of SUL carrier otherwise, NUL is selected with same manner.

Figure 1. Random Access procedure for Rel-15 NR

For NR-U, the RACH carrier selection method can be modified due to LBT failure. If the RACH carrier selection is simply dependent on only the DL pathloss reference RSRP in NR-U, it may result in unnecessary preamble transmission failures because the RACH carrier selection method does not correctly reflect whether the NR-U carrier is now busy or highly loaded or not. This would be first important turning point on whether following RACH procedure is successfully performed without failures especially in the case where UE is configured with SUL on licensed carrier but NUL is on unlicensed carrier. Even in the other case where both NUL and SUL is on unlicensed carriers, if RACH UE can properly select the RACH carrier firstly then it would increase the rate of successful RACH procedure.

One possible way for the proper RACH carrier selection in NR-U RACH is to consider the carrier loading status, on top of DL pathloss reference RSRP. In addition, there can be further relationship between the RACH carrier selection and LBT operation. For example, depending on LBT failures is over the threshold value, whether the RACH carrier selection should be re-performed or not is decided at a UE side and the threshold value can be configured by gNB.

Proposal 1:	It is proposed to enhance the RACH carrier selection method by additionally considering carrier loading status so that unnecessary LBT failures are avoided in following RACH procedures (e.g. preamble transmission), if UE is configured with SUL for RACH serving cell.

Conclusion

This contribution discusses the case of RACH carrier selection for NR-U if SUL is configured for a serving cell on unlicensed carrier and makes the following proposal:

Proposal 1:	It is proposed to enhance the RACH carrier selection method by additionally considering carrier loading status so that unnecessary LBT failures are avoided in following RACH procedures (e.g. preamble transmission), if UE is configured with SUL for RACH serving cell.

- References

| [1] | "RAN2 NR AH1807, Chairman Notes". |
| --- | --- |