3GPP TSG-RAN2 Meeting RAN2#103 	R2-1812820

Gothenburg, Sweden, 20th - 24th Aug 2018                  Revised of R2-1810529

Agenda Item	: 11.1.2 (FS_NR_IAB)

Source	: LG Electronics Inc.

Title	: Handling of the RLF on wireless backhaul link

Document for	: Discussion and Decision

Introduction

The RLF may occur on wireless backhaul link. However, RAN2 has not discussed how to handle the RLF on the wireless backhaul link. Thus, in this contribution, we show our views on the handling of the RLF on the wireless backhaul link. In addition, we propose a solution to prevent the packet loss in case of RLF on the wireless backhaul link.

Discussion

Since the IAB node is connected based on wireless backhaul link, the RLF can occur from time to time even though wireless backhaul connection would be stable. In general, the quick recovery from the RLF on wireless backhaul link should be guaranteed because the loss of wireless backhaul link may cause the service interruption to all UEs connected to the IAB node. Therefore, RAN2 should discuss how to handle the RLF in the IAB node.

Figure 1 RLF between IAB nodes

In our understanding, since the protocol stack would not be different with the UE and the IAB node, the legacy RLF procedure, i.e., RRC Connection Re-establishment, can be used for handling the RLF on the wireless backhaul link. Moreover, if new procedure for handling the RLF is defined for wireless backhaul link, it would cause a huge impact on the specification. Therefore, we propose that the legacy RLF procedure is used for handling the RLF on the wireless backhaul link.

Proposal 1. The legacy RLF procedure is used for handling of the RLF on wireless backhaul link.

If the IAB node performs the legacy RLF procedure when the RLF on wireless backhaul occurs, the packet loss may occur. This is because legacy RLF procedure performs the release and setup procedure to all configured RBs. In addition, a UE may consider the packets which are lost in the IAB node are successfully transmitted to the donor IAB node. Therefore, the UE cannot retransmit the lost packets by itself since the UE cannot identify the lost packets.

To prevent the packet loss, the straightforward solution is that the PDCP data recovery procedure is used. However, the PDCP data recovery procedure must be instructed by the donor IAB node.

But the donor IAB node may not know whether the RLF occurs in the IAB node. It means that the donor IAB node cannot instruct the PDCP data recovery to UEs associated with packet loss in the IAB node. Therefore, the IAB node that the RLF occurs should report the RLF occurrence to the donor IAB node.

Proposal 2. To prevent the packet loss due to the RLF on wireless backhaul link, the PDCP data recovery procedure is used.

Proposal 3. The IAB node reports the RLF occurrence to the donor IAB node.

Conclusion

In this contribution, we show our view on the handling of the RLF on the wireless backhaul link and discuss how to prevent the packet loss. Based on the above discussion, we propose followings.

Proposal 1. The legacy RLF procedure is used for handling of the RLF on wireless backhaul link.

Proposal 2. To prevent the packet loss due to the RLF on wireless backhaul link, the PDCP data recovery procedure is used.

Proposal 3. The IAB node informs the RLF occurrence to the donor IAB node.