k3GPP TSG-RAN2 Meeting #103	R2-1812938

# Gothenburg, Sweden, 20th Aug 2018 - 24th Aug 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | 0248 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on PDCP operation eV2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eV2X-Core |  |  |  |  |  | Date: |  |  | 2018-08-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - SN ambiguity cannot be solved by the half-SN-limitation as in cellular, since there is no feedback from RX UE in V2X communication. - PUSH window is not feasible for V2X communication, since the half-SN-limitation cannot be secured by V2X communication without feedback from RX UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 5.1.3 and 5.1.4, remove the maintaining of TX_HFN and RX_HFN, and clarify in 5.1.3 that, the same SN should not be used twice for a same source Layer-2 ID, to avoid SN wrap around ambiguity. - Add a section of 5.1.2.1.4.1a, to make use of the PULL window for duplication reception of SLRB, instead of the PUSH window operation of original section 5.1.2.1.4.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Since V2X communication cannot secure half-SN-limitation, the SN wrap around ambiguity cannot be solved. - Since V2X communication cannot secure half-SN-limitation, the packet that is out of the reordering window may be discarded, which cause packet loss. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3, 5.1.4, 5.1.2.1.4.1, 5.1.2.1.4.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5.1.3	SL Data Transmission Procedures

For Sidelink transmission, the UE shall follow the procedures in subclause 5.1.1 with following modifications:

-	the requirements for maintaining Next_PDCP_TX_SN and TX_HFN are not applicable;

-	determine a PDCP SN ensuring that a PDCP SN value is not reused with the same key;

-	perform ciphering (if configured) as specified in subclause 5.6.1 and 5.6.2;

-	perform the header compression (if configured) if SDU Type is set to 000, i.e. IP SDUs.

For sidelink duplication transmission capable UE, when PDCP duplication transmission is configured, for the SLRB with packets which have PPPR no lower than the configured PPPR threshold, the Sidelink transmission of UE shall follow the procedures in subclause 5.1.1 with following modifications compared to above Sidelink transmission procedure:

-	the requirements for maintaining Next_PDCP_TX_SN are applicable;

-	the PDCP entity duplicates the PDCP PDUs, and delivers the PDCP PDUs to both RLC entities for transmission.

-    UE shall not associate a same PDCP SN to two PDCP SDUs using the same source Layer-2 ID.

Next change

5.1.4	SL Data Reception Procedures

For Sidelink reception, the UE shall follow the procedures in subclause 5.1.2.1.3 with following modifications:

-	the requirements for maintaining Next_PDCP_RX_SN and RX_HFN are not applicable;

-	perform the deciphering (if configured) as specified in subclause 5.6.1 and 5.6.2;

-	perform the header decompression (if configured) if SDU Type is set to 000, i.e. IP SDUs.

For sidelink duplication reception capable UE, if it detects PDCP duplication reception in duplication logical channel, or if it receives a PDCP SN which is not "0" from the non-duplication logical channel, the Sidelink reception of the UE shall follow the procedures in subclause 5.1.2.1.4.1a with following modifications compared to above Sidelink reception procedure:

-	the requirements for maintaining Next_PDCP_RX_SN are applicable;

-	perform the re-ordering procedure as specified in subclause 5.1.2.1.4.1.

Next change

5.1.2.1.4.1	Procedures when a PDCP PDU is received from the lower layers

For DRBs mapped on RLC AM or RLC UM and for LWA bearers, or for DRBs and SRBs when PDCP duplication is used, when the reordering function is used, at reception of a PDCP Data PDU from lower layers, the UE shall:

<Text Removed>

Next change

5.1.2.1.4.1a	Procedures when a PDCP PDU is received from the lower layers

For DRBs mapped on SLRB for duplicated transmission, at reception of a PDCP Data PDU from lower layers, the UE shall:

-	if Next_PDCP_RX_SN – received PDCP SN > Reordering_Window, or if received PDCP SN >= Next_PDCP_RX_SN:

-	set Next_PDCP_RX_SN to the received PDCP SN + 1;

-	if Next_PDCP_RX_SN is larger than Maximum_PDCP_SN:

-	set Next_PDCP_RX_SN to 0;

-	if a PDCP SDU with the same PDCP SN is stored:

-	discard the PDCP PDU;

-	if the PDCP PDU has not been discarded in the above:

-	if received PDCP SN = Last_Submitted_PDCP_RX_SN + 1 or received PDCP SN = Last_Submitted_PDCP_RX_SN – Maximum_PDCP_SN:

-	deliver to upper layers in ascending order of the associated COUNT value:

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from the COUNT value associated with the received PDCP PDU;

-	set Last_Submitted_PDCP_RX_SN to the PDCP SN of the last PDCP SDU delivered to upper layers;

-	if t-Reordering is running:

-	if the PDCP SDU with Reordering_PDCP_RX_COUNT – 1 has been delivered to upper layers:

-	stop and reset t-Reordering;

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above):

-	if there is at least one stored PDCP SDU:

-	start t-Reordering;

-	set Reordering_PDCP_RX_COUNT to the COUNT value associated to RX_HFN and Next_PDCP_RX_SN.

End of changes