3GPP TSG-RAN WG2 #103	R2-1812971

# Gothenburg, Sweden, 20th – 24th August 2018

Agenda Item:	9.13.8

Source:	Ericsson

Title:	On correction of ltebandwidth in MIB-TDD-NB

Document for:	Discussion, Decision

# Introduction

Based upon the channel raster information, UE can narrow down the candidate LTE Bandwidth. The current lteBandwidth definition as proposed in the running CR (R2-1809276) is not correct. NB-IoTdevice cannot drive LTE bandwidth from this information.

# Discussion

In NB-IoT TDD, when SIB1-NB is transmitted on a non-anchor carrier and the anchor-carrier is guard-band mode, NB-IoT devices might need to derive the LTE carrier bandwidth from MIB-TDD-NB.

From the channel raster offset indicated in MIB-TDD-NB, NB-IoT devices can understand the LTE bandwidth is either 10/20 MHz (+/-2.5 kHz raster offset), or 5/15 MHz (+/-7.5 kHz raster offset).

However, to support IoT devices to further derive the exact LTE bandwidth, MIB-TDD-NB indicates the field ltebandwidth-r15, which should help IoT devices to distinguish LTE bandwidth between 10 or 20 MHz (in case of +/-2.5 kHz raster offset), or 5 or 15 MHz (in case of +/-2.5 kHz raster offset).

The current enumerated value for lteBandwidth-r15 (bw5or15, bw10or20) is wrong. It should changed to (bw5or10, bw15or20), so that the field can jointly indicate LTE bandwidth together with rasterOffset-r15 in MIB-TDD-NB.MIB-TDD-NB wastes 1 bit on SI-CarrierInfo, which should have been reserved for future usages

Correct lteBandwidth-r15 in MIB-TDD-NB

A text proposal based upon agreed CR R2-1809276 is provided below.

# 3.	Conclusions

In this paper, based on the discussion in the previous sections we propose the following:

Proposal 1	Correct lteBandwidth-r15 in MIB-TDD-NB

# References

R2-1809276	, “Introduction of further NB-IoT enhancements other than EDT in TS 36.331”, RAN2#102

# Text Proposal

#### MasterInformationBlock-TDD-NB

The MasterInformationBlock-TDD-NB includes the system information transmitted on BCH in TDD.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: E-UTRAN to UE

MasterInformationBlock-TDD-NB

-- ASN1START

MasterInformationBlock-TDD-NB-r15 ::=	SEQUENCE {

systemFrameNumber-MSB-r15				BIT STRING (SIZE (4)),

hyperSFN-LSB-r15						BIT STRING (SIZE (2)),

schedulingInfoSIB1-r15					INTEGER (0..15),

systemInfoValueTag-r15					INTEGER (0..31),

ab-Enabled-r15							BOOLEAN,

operationModeInfo-r15				CHOICE {

inband-SamePCI-r15					Inband-SamePCI-NB-r15,

inband-DifferentPCI-r15				Inband-DifferentPCI-NB-r15,

guardband-r15						Guardband-NB-r15,

standalone-r15						Standalone-NB-r15

},

sib1-CarrierInfo-r15					ENUMERATED {anchor, non-anchor},

si-CarrierInfo-r15						ENUMERATED {anchor, non-anchor},

spare									BIT STRING (SIZE (7))

}

Guardband-NB-r15 ::=				SEQUENCE {

rasterOffset-r15					ChannelRasterOffset-NB-r13,

sib-GuardbandInfo-r15				CHOICE {

sib-GuardbandAnchor-r15				SIB-GuardbandAnchorInfo-NB-r15,

sib-GuardbandGuardband-r15			SIB-GuardbandGuardbandInfo-NB-r15,

sib-GuardbandInbandSamePCI-r15		SIB-GuardbandInbandSamePCIInfo-NB-r15,

sib-GuardbandinbandDiffPCI-r15		SIB-GuardbandInbandDiffPCIInfo-NB-r15

},

eutra-Bandwidth-r15					ENUMERATED { bw5or10, bw15or20},

guardbandEdge-r15					ENUMERATED {lower, higher}

}

Inband-SamePCI-NB-r15 ::=			SEQUENCE {

eutra-CRS-SequenceInfo-r15			INTEGER (0..31),

sib-InbandLocation-r15				ENUMERATED {lower, higher},

spare								BIT STRING (SIZE (1))

}

Inband-DifferentPCI-NB-r15 ::=			SEQUENCE {

eutra-NumCRS-Ports-r15					ENUMERATED {same, four},

rasterOffset-r15						ChannelRasterOffset-NB-r13,

sib-InbandLocation-r15					ENUMERATED {lower, higher},

spare									BIT STRING (SIZE (3))

}

Standalone-NB-r15 ::=					SEQUENCE {

sib-StandaloneLocation-r15				ENUMERATED {lower, higher},

spare									BIT STRING (SIZE (6))

}

SIB-GuardbandAnchorInfo-NB-r15 ::= 		SEQUENCE {

spare									BIT STRING (SIZE (1))

}

SIB-GuardbandGuardbandInfo-NB-r15 ::= 	SEQUENCE {

sib-GuardbandGuardbandLocation-r15		ENUMERATED {same, opposite}

}

SIB-GuardbandInbandSamePCIInfo-NB-r15 ::= SEQUENCE {

spare									BIT STRING (SIZE (1))

}

SIB-GuardbandInbandDiffPCIInfo-NB-r15 ::= SEQUENCE {

sib-EUTRA-NumCRS-Ports-r15				ENUMERATED {same, four}

}

-- ASN1STOP

MasterInformationBlock-TDD-NB field descriptions

ab-Enabled Value TRUE indicates that access barring is enabled and that the UE shall acquire SystemInformationBlockType14-NB before initiating RRC connection establishment or resume.

guardbandEdge Indicates the side of the guardband anchor carrier relative to the LTE carrier. Value lower corresponds to the lower edge of the LTE carrier, value higher corresponds to the higher edge of the LTE carrier.

eutra-Bandwitdth EUTRA system bandwidth. Value bw5or10 corresponds to bandwidth 5 or 10 MHz, value bw15or20 corresponds to bandwidth 15 or 20 MHz,

eutra-CRS-SequenceInfo Information of the carrier containing NPSS/NSSS/NPBCH. Each value is associated with an E-UTRA PRB index as an offset from the middle of the LTE system sorted out by channel raster offset. See TS 36.211[21] and TS 36.213 [23].

eutra-NumCRS-Ports, sib-eutra-NumCRS-Ports Number of E-UTRA CRS antenna ports, either the same number of ports as NRS or 4 antenna ports. See TS 36.211 [21], TS 36.212 [22], and TS 36.213 [23].